3GPP TSG-SA WG1 Meeting #103 	S1-232228

Goteborg, Sweden, 21 – 25 August 2023

Source:	KPN

pCR Title:	Overview for Ambient IoT TS

Draft Spec:	3GPP TS 22.XXX

Agenda item:	7.2.2

Document for:	Approval

Contact:	Toon . Norp @tno .nl

Abstract: This document provides an alternative Overview section, based on what was proposed by Oppo and Apple in S1-232136. The main difference is that specific sections are proposed on AIoT device characteristics and typical AIoT use cases. The idea is that this way the Ambient IoT TS can provide some more information without immediately going into normative requirement statements. This should lead to better understanding by the readers.

Changes on changes indicate the difference with S1-232136

****************Start of Change *************

# 4	Overview

# 4.1	Introduction

In the 5G era, various LPWA technologies such as MTC, NB-IoT, and RedCap have been developed to fulfil the increasing demand from verticals. These LPWA technologies have achieved low cost, low power and massive connections and can meet requirements of many applications. However, there are still many use cases and applications that cannot be addressed. For example, a conventional battery-powered device cannot be deployed in extreme environmental conditions (e.g. high pressure, extremely high/low temperature, humid environment). Also, conventional battery devices cannot be used where maintenance-free devices are required (e.g. where the devices are inaccessible and it is not possible to replace the device battery). Finally, ultra-low complexity, very small device size/form factor (e.g. thickness of mm), longer life cycle, etc. are required for mass market use cases. Ambient power-enabled IoT is a promising technology to fulfil these unmet market requirements.

This technical specification describes the ambient IoT technology service requirements as part of the 5G system to enable new services and use cases. Ambient IoT has the potential to benefit a large number of verticals, e.g. smart manufacturing, logistics and warehousing, smart grid, agriculture, and smart home by providing functionalities that fulfil the needs of vertical use cases. Therefore, a new kind of IoT service for the verticals will be enabled by combining ambient power-enabled IoT with cellular networks, vastly benefitting the 3GPP ecosystem.

## 4.2	Characteristics of Ambient IoT

Not all Ambient IoT devices are the same. Nevertheless, Ambient IoT devices have the following characteristics.

-	Energy harvesting. An ambient power-enabled IoT device is an IoT device powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g. using a capacitor). Energy is provided through harvesting of radio waves, light, motion, heat, or any other suitable power source. Energy harvesting can be continuous or incidental (e.g. based on the vibration that a vibration sensor has to report). It is also possible that the network or a third party controls when / where power is provided. It cannot be assumed that Ambient IoT devices always have enough power to initiate or receive communication.

-	Low complexity. Ambient IoT devices are expected to have lower complexity, smaller size, reduced capabilities and lower power consumption than previously defined 3GPP IoT devices (e.g. NB-IoT/eMTC devices). Low complexity of Ambient IoT devices is also expected to be reflected in efficient use of network resources. In general, Ambient IoT applications will deploy very large numbers of Ambient IoT devices.

-	Low data rates. Generally, Ambient IoT data transmissions contain only a few hundred bits of data. Typical 5G services such as streaming are not supported.

-	Life span. Ambient IoT devices can be maintenance free and can have long life span (e.g. more than 10 years). However, the life span of an Ambient IoT device can also be relatively short, e.g. when tracking a package through a logistics chain.

## 4.3	Typical Ambient IoT use cases

Ambient IoT can support many different use cases. Nevertheless, in general the Ambient IoT use cases can be characterised in four different use case categories:

-	Inventory taking. With inventory taking, the main purpose is to discover what goods (e.g. boxes, containers, packages, tools) are present in a specific area. Upon request sent by the network within the specific area, Ambient IoT devices attached to these goods report an identifier associated with the good, possibly supplemented with other information such as status, measurement results and/or location.

-	Sensor data collection. With sensor data collection, the Ambient IoT device is associated with a sensor. Transfer of sensor data can be initiated by the Ambient IoT device, e.g. periodically or when the Ambient IoT device has power, or can be initiated by the network.

-	Asset tracking. With asset tracking, the main purpose is to determine the location of goods. Ambient IoT devices attached to these goods report report an identifier associated with the good in combination with a location. Asset tracking can also be initiated by an Ambient IoT capable UE, thus finding the location of Ambient IoT devices within a particular range of the UE.

-	Actuator control. With actuator control, the Ambient IoT device is associated with an actuator. Transfer of actuator commands is generally initiated by the network.

## 4.4	Communication modes

Ambient IoT devices can communicate with the network or directly with Ambient IoT capable UEs. There is also an Ambient IoT indirect mode, where the Ambient IoT device communicates with the network via a UE that is equiped to perform such Ambient IoT relay functionality.

*******************End of Change*****************