3GPP TSG-SA WG1 Meeting #103 	S1-232255

Gothenburg, Sweden, 21 - 25 Aug 2023	(revision of S1-231707, 2237)

Source:	Qualcomm, Lenovo, CableLabs, Xiaomi, Comcast Corporation, Verizon UK Ltd, Tencent, Thales, Charter Communications, SyncTechno Inc., InterDigital, KDDI, Nokia, Nokia-Shanghai Bell, Vivo, Lockheed Martin, Sennheiser, Viasat, KPN, LG Electronics, Apple, Novamint, NEC, ETRI, IRT Saint Exupery, CATT, DSIT, Ericsson

Title:	pCR for FS_DualSteer Definitions and Consolidated requirements

Agenda Item:	7.9.1

Draft Spec:	3GPP TR 22.841

Contact:	Francesco Pica
fpica <at> qti.qualcomm.com

Abstract: This document includes a pCR proposal to add new definitions to the TR, and to fill sec.6 on consolidated potential requirements.

======================================  First CHANGE ========================================

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Traffic steering, splitting and switching: In the context of this TR, considering scenarios where UE’s traffic can be transmitted over two 3GPP access networks, the following definitions apply:

- Traffic Steering: the procedure that selects an access network for a new data flow and transfers the traffic of this data flow over the selected access network.

- Traffic Splitting: the procedure that splits the traffic of a data flow across multiple access networks. When traffic splitting is applied to a data flow, some traffic of the data flow is transferred via one access and some other traffic of the same data flow is transferred via another access.

- Traffic Switching: the procedure that moves all traffic of an ongoing data flow from one access network to another access network in a way that maintains the continuity of the data flow.

======================================  Next CHANGE ========================================

# 6	Consolidated Potential Requirements

===============================  All new text ================================

## 6.1	Introduction

The following requirements represent a consolidation of the various potential requirements captured in the above use cases, describing different scenarios and functionalities for supporting enhanced traffic steering, splitting and switching of UE’s user data (of the same data session) across two 3GPP access networks, using a single PLMN subscription.

Target scenarios cover two 3GPP access networks belonging to the same PLMN, or between two different PLMNs, or between one PLMN and one NPN, using same or different RAT, e.g., NR plus NR, or NR plus E-UTRA (in case of combined EPC and 5GC), where NR can be terrestrial or satellite access (including the case of two different satellite orbits).

The requirements do not foresee RAN impacts, and can apply to different UE types (e.g. smartphones, IoT, UAV devices).

## 6.2	General

Table 6.2-1 – General Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.2-1 | CPR 6.2-1a: The 5G system shall be able to support mechanisms to enable traffic steering, splitting and switching of UE’s user data (of the same data session) across two 3GPP access networks belonging to the same PLMN (either HPLMN or VPLMN). NOTE 1: For intra-HPLMN scenarios, the user data transferred over the two 3GPP access networks is anchored in the HPLMN core network. For intra-VPLMN scenarios, the user data may be anchored in the HPLMN or may use local break out. | 5.1.6-001 5.2.6-001 5.2.6-002 5.3.6-001 5.4.6-001 5.5.6-001 5.6.6-001 5.6.6-002 5.7.6-001 5.8.6-001 5.9.6-001 5.10.6-001 5.11.6-001 5.12.6-001 5.14.6-001 5.17.6-001 5.18.6-001 5.12.6-001 |  |
|  | CPR 6.2-1b: The 5G system shall be able to support mechanisms to enable traffic steering, splitting and switching of UE’s user data (of the same data session) across two 3GPP access networks belonging to two PLMNs. CPR 6.2-1c: The 5G system shall be able to support mechanisms to enable traffic steering, splitting and switching of UE’s user data (of the same data session) across two 3GPP access networks belonging to a PLMN and an NPN (can be standalone or PLMN-integrated). NOTE 2: For the above requirements, - the user data transferred over the two 3GPP access networks is anchored in the HPLMN core network. - a proper business agreement is assumed to be in place between the two mobile network operators. |  |  |
| CPR 6.2-2 | The 5G system shall be able to support mechanisms to allow a HPLMN to provide a UE with policies and criteria to connect to an additional PLMN/NPN, or an additional RAT within the same PLMN, for traffic splitting, steering and switching of UE’s user data across two 3GPP access networks. | 5.14.6-001 |  |
| CPR 6.2-3 | The 5G system shall be able to support traffic steering, splitting and switching of UE’s user data for multiple UEs such that specific service flow(s) for those UEs use the same 3GPP access network(s). | 5.16.6-002 |  |
| NOTE 3: The above requirements assume configuration of traffic policies, under HPLMN control or negotiated between the HPLMN and other network operators, considering e.g., user subscription, application/traffic type, service preference, QoS requirements, location, time, UE capabilities, mobility, connectivity conditions. Traffic policies can also include data duplication over the two access networks. NOTE 4: The above requirements include the possibility to report UE specific user data measurements (e.g., RTT, packet loss rate) between the UE and the anchoring core network. NOTE 5: The three functionalities (traffic steering, splitting and switching) may or may not be used together (during the same UE data session). |  |  |  |

## 6.3	Mobility and connectivity changes

Table 6.3-1 – Mobility Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.3-1 | The 5G system shall be able to support mechanisms to enable service continuity when switching UE’s user data, of the same data session, between two 3GPP access networks. | 5.2.6-002 5.6.6-002 |  |
| CPR 6.3-2 | The 5G system shall be able to support traffic steering, splitting and switching of UE’s user data across two 3GPP access networks belonging to the HPLMN and a VPLMN, including mechanisms to move UE’s user data from one VPLMN to another VPLMN, while maintaining the same access connection with the HPLMN. NOTE 1: UE can be connected to a maximum of two PLMNs simultaneously, including the HPLMN. NOTE 2: the above requirement may also apply to two 3GPP access networks belonging to a single PLMN, when moving UE’s user data from one 3GPP access network to a third 3GPP access network of the same PLMN. | 5.10.6-002 |  |
| CPR 6.3-3 | The 5G system shall be able to support traffic steering, splitting and switching of UE’s user data across two 3GPP access networks including mechanisms to change one 3GPP access network to a non-3GPP access network (and vice versa). NOTE 3: The 3GPP and non-3GPP access networks are assumed to be managed by the same network operator, and data is anchored in the HPLMN 5G core network. | 5.13.6-001 |  |

## 6.4	Charging and other aspects

Table 6.4-1 – Charging and other Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.4-1 | The 5G system shall be able to collect charging information during traffic steering, splitting and switching of UE’s user data across two 3GPP access networks. NOTE 1: charging information should be collected for both 3GPP access networks; in case the two 3GPP access networks belong to different PLMNs, or a PLMN and NPN, a proper business agreement among network operators is assumed. | 5.5.6-002 5.9.6-002 5.17.6-002 |  |
| CPR 6.4-2 | Based on operators’ policy, the 5G system shall be able to support traffic steering, splitting and switching of UE’s user data between a NPN and a PLMN, for one or more UEs with a NPN subscription accessing NPN services, to meet specific QoS requirements for each UE. NOTE 2: the above assumes a NPN hosted by a PLMN or offered as a slice of a PLMN, data anchoring in the NPN, and a business agreement between the PLMN and the NPN operators (if different). | 5.15.6-001 5.16.6-001 |  |

===============================  End of Change ===============================