3GPP TSG SA WG 1 Meeting #103 	S1-232262

Gothenburg, Sweden, 21 - 25 August 2023	(revision of S1-232096)

Source:	Qualcomm

pCR Title:	Pseudo-CR on CPR proposals for Ambient IoT

Draft Spec:	3GPP TR 22.840 v.1.2.0

Agenda item:	7.2.1

Document for:	Approval

Contact:	Lola Awoniyi-Oteri oawoniyi@qti.qualcomm.com

Abstract: Provide CPR proposal for positioning

1. Introduction

<Introduction part >

2. Reason for Change

- Positioning

There are quite a number of CPR proposals with positioning aspects that are being discussed during this study. These proposals are presented  below:

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.2-1 | The 5G system shall support positioning mechanisms for Ambient IoT devices (e.g locating Ambient IoT device using absolute or relative positioning methods or locating a UE using Ambient IoT devices). | PR 5.9.6-002 PR 5.12.6-004 PR 5.2.6-004 | Qualcomm: clarification for what positioning service means. Suggestion “, e.g. locationing AIoT device” Proposal: merge CPR 7.1.2-a1/b1/2 by adding “e,g,” for clarification Ericsson: strike out “or locating a UE using Ambient IoT devices” Nokia: more clarification for “relative positioning” “The 5G system shall allow an Ambient IoT device to measure the distance to specific Ambient IoT devices or AIoT capable UEs “ KPN: similar to Daniel’s comment. The relative positioning refers to device-UE, or device-device? Huawei: not sure if “device -device” scenario is in TR |
| CPR 7.1.1-a1 | The 5G system shall be able to assist an Ambient IoT device with acquiring positioning related information (e.g. relative or absolute position of the Ambient IoT device). | PR.5.8.6-001 | Huawei: suggest to simplify the wording “5GS shall be able to provide some information so that can be used to derive the positioning information of Ambient IoT device" Ericsson: avoid solution specific Qualcomm: remove overlapping |
| CPR 7.1.2-b1 | The 5G system shall be able to support an authorized UE to perform relative positioning with specific Ambient IoT devices. | PR 5.10.6-001 PR.5.21.6-002 | Nokia: optional UE functionality? Qualcomm: discuss together: -a1,-b1, -b2 |
| CPR 7.1.2-b2 | The 5G system shall support a UE to obtain its location with the assistance of Ambient IoT devices (e.g. providing the UE with some nearby Ambient IoT devices' location information). | PR.5.14.6-002 | Ericsson: solution specific |

The proposed CPR 7.1.2-1, CPR 7.1.1-a1 and CPR 7.1.2-b1 focus on locating a Ambient IoT device using various positioning approaches such as absolute or relative positioning techniques so we think these three proposed CPRs can be combined and our proposal for this combination CPR in presented in clause 7.1.2. The proposed CPR 7.1.2-b2 in shown in the Table above discuss how to locate UEs using Ambient IoT devices, therefore, we think it is best to capture this CPR proposal separately from the combination CPR proposal.  In section, 7.1, we present a CPR proposal based on email discussions between the following companies: KPN, Ericsson, Huawei, Philips, Nokia, DT, OPPO and Qualcomm.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 2.840 v1.2.0.

* * * First Change * * * *

# 7	Consolidated potential requirements and KPIs

## 7.1	Consolidated potential requirements

Mapping table is used per each subclause for consolidated potential requirements.

### 7.1.1	Communication aspects of Ambient IoT devices

Table 7.1.1 – [Communication aspects of Ambient IoT devices] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

### 7.1.2	Positioning/location of Ambient IoT devices

Table 7.1.2 – [Positioning/location of Ambient IoT devices] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.2-1 | The 5G network and AIoT capable UEs shall support location services for Ambient IoT devices ( e.g., to locate Ambient IoT devices using absolute or relative positioning methods) | PR 5.9.6-002 PR 5.12.6-004 PR 5.2.6-004 PR.5.8.6-001 PR 5.10.6-001 PR.5.21.6-002 |  |

### 7.1.3	Management of Ambient IoT devices

Table 7.1.3  [Management of Ambient IoT devices] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

### 7.1.4	“Collected information” and network capability exposure

Table 7.1.4  [“Collected information” and network capability exposure] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

### 7.1.5	Charging

Table 7.1.5  [Charging] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

### 7.1.6	Security and privacy

Table 7.1.6  [Security and privacy] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

* * * End of Change * * * *