3GPP TSG SA WG 1 Meeting #103		 S1-232284

Gothenborg, Sweden, Aug 21, 2023 - Aug 25, 2023					(revision of S1-232185)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0719 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting UE Mobility for XR service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRMobility |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the existing spec, the KPI related to AR/VR service under high-speed is not clear. It is important to provide suitable KPIs under this condition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Propose to add KPI values related to AR/VR service under high-speed scenarios to support immersive user experience. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE in high speed mobility cannot have a good experience of AR/VR service |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S1-232185 |  |  |  |  |  |  |  |  |

********** First Changes *************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	NGMN 5G White Paper v1.0, February 2015.

[3]	3GPP TS 22.011: "Service accessibility".

[4]	Void

[5] 	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[6] 	3GPP TS 22.101: "Service aspects; Service principles".

[7]	3GPP TS 22.146: "Multimedia Broadcast/Multicast Service (MBMS)".

[8]	3GPP TS 22.246: "Multimedia Broadcast/Multicast Service (MBMS) user services".

[9] 	3GPP TS 22.186: "Enhancement of 3GPP support for V2X scenarios".

[10]	NGMN, "Recommendations for NGMN KPIs and Requirements for 5G", June 2016

[11]	3GPP TS 22.115: "Service aspects; Charging and billing".

[12] 	Void

[13]	Soriano, R., Alberto, M., Collazo, J., Gonzales, I., Kupzo, F., Moreno, L., & Lorenzo, J. OpenNode. Open Architecture for Secondary Nodes of the Electricity Smartgrid. In Proceedings CIRED 2011 21st International Conference on Electricity Distribution, CD1. June 2011.

[14] 	North American Electric Reliability Council. Frequently Asked Questions (FAQs) Cyber Security Standards CIP–002–1 through CIP–009–1. Available: http://www.nerc.com/docs/standards/sar/Revised_CIP-002-009_FAQs_06Mar06.pdf. 2006.

[15]	McTaggart, Craig, et al. "Improvements in power system integrity protection schemes". Developments in Power System Protection (DPSP 2010). Managing the Change, 10th IET International Conference on. IET, 2010.

[16]	IEEE Power Engineering Society – Power System Relaying Committee – System Protection Subcommittee Working Group C-6. Wide Area Protection and Emergency Control.

[17]	Begovic, Miroslav, et al. "Wide-area protection and emergency control". Proceedings of the IEEE 93.5, pp. 876-891, 2005.

[18]	ITU-T Recommendation G.1000 "Communications quality of service: A framework and definitions".

[19]	IEC 61907, "Communication network dependability engineering".

[20]	NIST, "Framework for Cyber-Physical Systems", 2016.

[21]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[22]	3GPP TS 22.262: "Message Service within the 5G System".

[23]	3GPP TS 22.289: "Mobile Communication System for Railways".

[24]	3GPP TS 22.071: "Location Services".

[25]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[26]	3GPP TS 22.125: "Unmanned Aerial System (UAS) support in 3GPP".

[27]	Void

[28]	3GPP TS 22.263: "Service requirements for Video, Imaging and Audio for Professional Applications (VIAPA)".

[29]	Void

[30]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT)".

[31]	IEEE 1588-2019, IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems.

[32]	IEC 61850-9-3-2016 - IEC/IEEE International Standard - Communication networks and systems for power utility automation – Part 9-3: Precision time protocol profile for power utility automation.

[33]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN"

[34]	ATIS-0900005: "Technical Report on GPS Vulnerability", https://access.atis.org/apps/group_public/download.php/36304/ATIS-0900005.pdf

[35]	European Commission, Regulatory Technical Standard 25. Level of accuracy of business clocks
https://ec.europa.eu/finance/securities/docs/isd/mifid/rts/160607-rts-25_en.pdf (annex https://ec.europa.eu/finance/securities/docs/isd/mifid/rts/160607-rts-25-annex_en.pdf)

[36]	5G-ACIA, "Exposure of 5G capabilities for Connected Industries and Automation Applications", 5G-ACIA white pater, February 2021, https://5g-acia.org/wp-content/uploads/2021/04/5G-ACIA_ExposureOf5GCapabilitiesForConnectedIndustriesAndAutomationApplications.pdf

[37]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services".

[38]	ITU-T, "Technology Watch Report: The Tactile Internet", August 2014.

[39]	D. Soldani, Y. Guo, B. Barani, P. Mogensen, I. Chih-Lin, S. Das, "5G for ultra-reliable low-latency communications". IEEE Network. 2018 Apr 2; 32(2):6-7.

[40]	O. Holland et al., "The IEEE 1918.1 "Tactile Internet" Standards Working Group and its Standards," Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019.

[41]	Altinsoy, M. E., Blauert, J., & Treier, C., "Inter-Modal Effects of Non-Simultaneous Stimulus Presentation," A. Alippi (Ed.), Proceedings of the 7th International Congress on Acoustics, Rome, Italy, 2001.

[42]	Hirsh I.J., and Sherrrick C.E, 1961. J. Exp. Psychol 62, 423-432

[43]	Altinsoy, M.E. (2012). "The Quality of Auditory-Tactile Virtual Environments," Journal of the Audio Engineering Society, Vol. 60, No. 1/2, pp. 38-46, Jan.-Feb. 2012.

[44]	M. Di Luca and A. Mahnan, "Perceptual Limits of Visual-Haptic Simultaneity in Virtual Reality Interactions," 2019 IEEE World Haptics Conference (WHC), 2019, pp. 67-72, doi: 10.1109/WHC.2019.8816173.

[45]	K. Antonakoglou et al., “Toward Haptic Communications Over the 5G Tactile Internet”, IEEE Communications Surveys & Tutorials, 20 (4), 2018.

[xx]	N. Nonaka et al., "Experimental Trial aboard Shinkansen Test Train Running at 360 km/h for 5G Evolution," 2022 IEEE 95th Vehicular Technology Conference: (VTC2022-Spring), Helsinki, Finland, 2022.

********** Second Changes *************

7.6	High data rate and low latency

7.6.1	AR/VR

Audio-visual interaction is characterised by a human being interacting with the environment or people, or controlling a UE, and relying on audio-visual feedback. In the use cases like VR and interactive conversation the latency requirements include the latencies at the application layer (e.g. codecs), which could be specified outside of 3GPP.

To support VR environments with low motion-to-photon capabilities, the 5G system shall support:

-	motion-to-photon latency in the range of 7 ms to 15ms while maintaining the required resolution of up to 8k giving user data rate of up to [1Gbit/s] and

-	motion-to-sound delay of [< 20 ms].

NOTE: 	The motion-to-photon latency is defined as the latency between the physical movement of a user's head and the updated picture in the VR headset. The motion-to-sound latency is the latency between the physical movement of a user's head and updated sound waves from a head mounted speaker reaching their ears.

To support interactive task completion during voice conversation, the 5G system shall support low-delay speech coding for interactive conversational services (100 ms, one-way mouth-to-ear).

Due to the separate handling of the audio and video component, the 5G system will have to cater for the VR audio-video synchronisation in order to avoid having a negative impact on the user experience (i.e. viewers detecting lack of synchronization). To support VR environments the 5G system shall support audio-video synchronisation thresholds:

-	in the range of [125 ms to 5 ms] for audio delayed and

-	in the range of [45 ms to 5 ms] for audio advanced.

The 5G system shall support service continuity for AR/VR to support immersive user experience under high UE mobility.

When it comes to implementation of applications containing AR/VR components, the requirements on the 5G network could depend on architectural choices implementing these services. Note 3 in table 7.1-1 above gives an example on such dependences for a VR application in a 5G system. Table 7.6.1-1 below illustrates additional use cases and provides more corresponding requirements on the 5G system.

-	Cloud/Edge/Split Rendering – Cloud/Edge/Split Rendering is characterised by the transition and exchange of the rendering data between the rendering server and device.

-	Gaming or Training Data Exchanging – This use case is characterised by the exchange of the gaming or training service data between two 5G connected AR/VR devices.

-	Consume VR content via tethered VR headset – This use case involves a tethered VR headset receiving VR content via a connected UE; this approach alleviates some of the computation complexity required at the VR headset, by allowing some or all decoding functionality to run locally at the connected UE. The requirements in the table below refer to the direct wireless link between the tethered VR headset and the corresponding connected UE.

# Table 7.6.1-1 KPI Table for additional high data rate and low latency service

| Use Cases | Characteristic parameter (KPI) |  |  | Influence quantity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | # of UEs | UE Speed | Service Area (note 2) |
| Cloud/Edge/Split Rendering (note 1) | 5 ms (i.e. UL+DL between UE and the interface to data network) (note 4) | 0,1 to [1] Gbit/s supporting visual content (e.g. VR based or high definition video) with 4K, 8K resolution and up to120 frames per second content. (note 7) | 99,99 % in uplink and 99,9 % in downlink (note 4) | - | Stationary or Pedestrian | Countrywide |
| Gaming or Interactive Data Exchanging (note 3) | 10ms (note 4) | 0,1 to [1] Gbit/s supporting visual content (e.g. VR based or high definition video) with 4K, 8K resolution and up to120 frames per second content.(note 7) | 99,99 % (note 4) | ≤ [10] | Stationary or Pedestrian | 20 m x 10 m |
| Consumption of VR content via tethered VR headset (note 6) | [5 to 10] ms (note 5) | 0,1 to [10] Gbit/s (note 5) | [99,99 %] | - | Stationary or Pedestrian | - |
| NOTE 1: Unless otherwise specified, all communication via wireless link is between UEs and network node (UE to network node and/or network node to UE) rather than direct wireless links (UE to UE). NOTE 2: Length x width (x height). NOTE 3: Communication includes direct wireless links (UE to UE). NOTE 4: Latency and reliability KPIs can vary based on specific use case/architecture, e.g. for cloud/edge/split rendering, and can be represented by a range of values. NOTE 5: The decoding capability in the VR headset and the encoding/decoding complexity/time of the stream will set the required bit rate and latency over the direct wireless link between the tethered VR headset and its connected UE, bit rate from 100 Mbit/s to [10] Gbit/s and latency from 5 ms to 10 ms. NOTE 6: The performance requirement is valid for the direct wireless link between the tethered VR headset and its connected UE. NOTE 7: The user-experienced data rate may be achievable also at higher UE speeds (Users in vehicles up to 120 km/h or users in trains up to 350 km/h). [xx] |  |  |  |  |  |  |