3GPP TSG-SA WG1 Meeting # 103 	S1-232327

# Goteborg, Sweden, Aug 21, 2023 - Aug 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.125 | CR | 0045 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional requirements for network exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Additional PCRs have been agreed in TR 22.843 at SA1 #102 meeting, including: [CPR 6.1-001] [CPR 6.1-002] [CPR 6.1-003] [CPR 6.1-006] |  |  |  |  |  |  |  |  |
| Summary of change: |  | In this CR, [CPR 6.1-001], [CPR 6.1-002], [CPR 6.1-003] and [CPR 6.1-006] are consolidated with 6.2 and 6.8 (new). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Agreed CPRs in SA1 #102 will not be properly added to TS 22.125. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2, 6.8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 6.2	Network exposure for UAV services

[R-6.2-001] The 3GPP system shall provide means to allow a 3rd party to request and obtain real-time monitoring the status information (e.g., location of UAV, communication link status) of a UAV.

[R-6.2-002] Based on operator 's policy, the 3GPP system shall provide means to provide a 3rd party with the information regarding the service status for UAVs in a certain geographical area and/or at a certain time.

NOTE: 	Service status is about the information of whether the communication service to the UAV can be provided with a certain QoS by the network.

[R-6.2-003] Based on operator’s policy, the 5G system shall be able to support a method to predict, monitor network conditions and QoS (e.g. bitrate, latency, reliability) and report to 3rd party along a continuous geographic planned flight path of a UAV at specific times of its expected flight duration.

Second CHANGE

## 6.8	UTM assistance requirements

[R-6.8-001] Based on operator’s policy, the 5G system shall be able to provide UTM with the information about geographic areas where UAV service requirements could or could not be met based on predicted network conditions and QoS (e.g. bitrate, latency, reliability).

[R-6.8-002] Based on a 3rd party request, the 5G system shall be able to assist the UTM with mechanism to provide to the 3rd party alternative UAV flight paths, e.g., based on required waypoints, QoS, and exclusion zones.

[R-6.8-003] The 5G system shall be able to support service enablement layer exposure mechanisms for the UTM or other authorized 3rd party to provide the UAV application with configuration information to route and switch traffic between one active and one standby PLMN connection, e.g. for C2 communication reliability and redundancy purpose, or to route different traffic across different PLMN connections simultaneously, e.g. C2 traffic via one PLMN and other data via the second PLMN.

NOTE 1: The above requirement can be extended to scenarios where one network is a PLMN and one is an NPN.

NOTE 2: There is no impact on legacy network selection.

NOTE 3: It is assumed that UAV traffic handling, over each PLMN, is subject to NW control mechanisms (e.g. in accordance with MNO routing priorities, available QoS/NW resources, etc.).

END OF CHANGES