3GPP TSG-SA WG1 Meeting #103 	S1-232339

Gothenburg, Sweden, 21 - 25 August 2023	(revision of S1-232227)

Source:		Huawei, HiSilicon

Title:	Pseudo-CR on CPR (Consolidation Potential Requirements) for TR 22.841

Draft Spec:	3GPP TR TR22.841 V1.1.0

Agenda item: 	7.9.1

Document for:     	Approval

Contact:	         	Shuang ZHANG < zhang.shuang2@huawei.com>

Abstract: This pCR proposes a consolidation of potential requirements for the FS-DualSteer study.

1. Introduction

Progress is made in TR 22.841 and the time to discuss use cases has almost come to an end.

2. Reason for Change

No CPRs have been introduced for TR 22.841.

3. Conclusions

Include the proposed CPRs into TR 22.841 v.1.1.0.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.841 v1.1.0.

* * * First Change * * * *

# 6	Consolidated potential requirements

## 6.1	5G Terrestrial Network + 5G Satellite Access Network

Table 6.1-1 – 5G Terrestrial Access Network and 5G Satellite Access Network Combination Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| [CPR 6.1-1] | Based on operator polices, the 5G system shall support a mechanism to switch the user traffic between different 3GPP access networks (i.e. 5G terrestrial and 5G satellite access networks) for UE with dual (3GPP RATs) radio capability, considering service preference, QoS requirements, and traffic characteristics, etc. | [PR 5.6.6-001] [PR 5.9.6-001] |  |
| [CPR 6.1-2] | Based on operator policy while satisfying the required QoS, the 5G system shall be able to support a mechanism to steer or switch UE's data transmission pertaining to the same data session onto a suitable 3GPP 5G access network (when two access networks are available with at least one of them being 5G satellite access network). NOTE 1: Available information can be considered, e.g. connectivity conditions such as access network performance, and UE's moving speed. This applies for the situation when a UE’s HPLMN has two access networks with at least one being 5G satellite access network, or the situation when two PLMNs (one of them is HPLMN) are concerned and at least one of them as 5G satellite access network. NOTE 2: Only single subscription applies, in case the two 5G access networks belong to different PLMNs. | [PR 5.4.6-001] [PR 5.5.6-001] [PR 5.6.6-001] [PR 5.10.6-001] |  |
| [CPR 6.1-3] | Based on operator policies while satisfying the required QoS, the 5G system shall be able to support mechanisms to allow steering or switching a UE’s data traffic pertaining to different applications (e.g. video streaming, FTP, various IoT applications) onto one of the two access networks, with traffic belonging to a given application being transmitted via a specific access network (i.e. 5G satellite access network, or 5G TN). | [PR 5.17.6-001] [PR 5.18.6-001] |  |
| [CPR 6.1-4] | Based on operator policy and agreement, the 5G system shall support service continuity with minimum service interruption when switching the user traffic across two 3GPP access networks (i.e. NR terrestrial and satellite access network) for UE with dual radio (3GPP RATs) capability in use, based on network availability, service preference, etc. | [PR 5.6.6-002] |  |

## 6.2	Charging and other aspects

Table 6.5-1 – Consolidated Charging and Other Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| [CPR 6.2-1] | When two 5G access networks Kallol (i.e. 5G TN, and 5G satellite access network) are used simultaneously for steering different data flows of the same data service for the same user subscription, the 5G system shall be able to collect charging information accordingly. | [PR 5.5.6-002] |  |
| [CPR 6.2-2] | The 5G system shall be able to collect charging information considering switching data transmission pertaining to the same user data session across two 3GPP access networks (i.e. 5G TN, and 5G satellite access network) of the same HPLMN. | [PR 5.9.6-002] |  |
| [CPR 6.2-3] | Based on data usage of both access networks (i.e. i.e. 5G TN, and 5G satellite access network) for sending traffic of different applications (traffic of a given application via one specific access network) of an IoT device, the 5G system shall be able to collect charging information accordingly. | [PR 5.17.6-002] |  |

* * * End of Change * * * *