3GPP TSG-SA WG1 Meeting #103 															S1-232351

(Revision of S1-232341)

Goteborg, Sweden, 21 – 25 August 2023

Source:	OPPO, CMCC, Futurewei, vivo, Huawei？

pCR Title:	Consolidation on KPI for Ambient IoT

Draft Spec:	3GPP TR 22.840

Agenda item:	7.2.2

Document for:	Approval

Contact:	xuweijie@oppo.com,xuyang@oppo.com, amanda.xiang@futurewei.com

Abstract: This document provides a Text Proposal for the consolidation on KPI for Ambient IoT.

Discussion

30 use cases and 3 traffic scenarios have been agreed and captured in the latest version of TR 22.840 v1.0.0.

KPI consolidation methodology:

In order to do KPI consolidation for ambient IoT use cases. The following methodology and steps have been proposed and performed:

- Step 1: Grouping use cases into several categorizations based on the functionality that ambient IoT can provide;

- Step 2: For each categorization, the use cases are further grouped based on their deployment scenarios, i.e. indoor, outdoor.

- Step 3: After step 1 and step 2, we get the all the sub-groups corresponding to all the combination of each categorization and each deployment scenario. For each sub-group use cases under each categorization, merge the KPI values and generate the consolidated KPI values.

Use case categorization and grouping:

Step 1 and step 2 are for use cases categorizations and grouping. The following motivation is identified for step 1 and step 2:

Motivation of use cases categorizations:

- There are different requirements (payload size, device density, transfer intveral, need of positioning etc.) for different categorizations and different deployment

- There may be similar requirements for use case under each combination of a categorization and a deployment scenario (e.g. indoor +inventory, outdoor + sensor data collection, indoor + actuator control),which is necessary to derive the consolidated KPI values.

  - Even for some combination, the KPI may be still diverse for different use cases. In this case, the use cases can be further grouped into different sub-types.

- KPI consolidation is performed for each sub-group corresponding to each combination or for each sub-type (i.e. depending on the KPI diversity). The consolidated KPI will facilitate the specification implementation for the downlink group by take the specific characteristcis of each combination or sub-types.

For step 1, based on the functionality that ambient IoT provides, the use cases and traffic scenarios are put into the following 4 categorizations as in the following:

- Use case categorization 1: Inventory

- Use case categorization 2: Sensor data collection

- Use case categorization 3: Tracking

- Use case categorization 4: Actuator control (e.g. switch)

In this step, it is noted that some use cases invole more than one function, in order to avoid KPI dupulication it is proposed one use case is only grouped within one   categorization based on the main function that ambient IoT that can provide for them. For example, use case 5.1 Automated warehousing is only put into categorization 1: Inventory, use case Ambient IoT devices for smart laundry is only put into categorization 2: Sensors. Traffic scenario 6.3 electronic shelf label is put into categorization 4: Actuator control.

For step 2, the above 4 categorizations are further grouped based on their deployment scenarios, i.e. indoor, outdoor.

Similarly, some use cases, it related to both indoor and outdoor. For this kind of use cases, it is grouped into only indoor or outdoor based on the most possible deployment scenario. For example, use case 5.7 Airport terminal / shipping port is mainly an indoor use cases althougu it also related to outdoor in some cases.

Using above methodology, all the use cases and traffic scenarios with KPI values are catorgorized and grouped into Table 1:

Table 1: Ambient IoT use case categorization and grouping

| Use cases Categories | Use cases agreed in TR 22.840 v1.2.0 |
| --- | --- |
| Inventory | 1; 2; 5; 7; 16; traffic scenario 1 |
| Sensor data collection | 3; 6; 13; 15; 18; 19; 20; 22; 23; 24; 25; traffic scenario 2; |
| Tracking | 8; 9; 10; 12; 14; 21; |
| Actuator control | 11;26;30; traffic scenario 3 |

KPI consolidation:

After use case categorization and grouping, the KPI consolidation is performed. The specific consolidation method is:

Firslt, the KPI values from use case under each combination of a categorization and a deployment scenario (e.g. indoor +inventory, outdoor + sensor data collection, indoor + actuator control) will be checked:

  - If all the values are within a similar range, the KPIs from all the use case under this combinarion will merged (consolided) into one row (as highlighted in green).

    - Example include indoor inventory etc.

  - Otherwise, the KPI may be still diverse for different use cases. In this case, the use cases within each sub-group for each combination will be further grouped into different sub-types. And the KPIs from all the use case under each sub-type will be merged (consolided) into one row (as highlighted in green).

    - Example includes indoor sensor data collection (includes 2 sub-type: Room envirionment monitoring and Indoor agriculture and husbandry), outdoor sensor data collection (include smart grid, Outdoor husbandry and logistics and smart city)  as in the following.

Table 2 KPI consolidation for inventory

| Deployment |  | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | 1 | Automated warehousing | 1s | 99% | NA | <100/128bits/s | 96/128 bits | [NA] | 30m indoors | NA | 5~10km/h | NA | NA | NA | 2~3 m | UC#5.1 |
|  | 2 | Medical instrument inventory management and positioning | Several seconds | 99% | NA | <2kbit/ss | 176bits | ≥1000/km² | 50m Indoor 200m Outdoor | NA | Stationary or walking speed <6km/h | NA | NA | NA | 3 m to 5 m indoor | UC#5.2 |
|  | 3 | Automatic Intralogistics in automobile manufacturing | >100 ms | 99% | NA | <1 kbit/s | 96 bits | <1,5 Million/km² | <30 meters Indoors | 600 000 m² | Up to 5 km/h | NA | NA | NA | [3] m | UC#5.5 |
|  | 4 | Airport terminal / shipping port | 1s to10s | 99% | NA | NA | 256 bits (UL) | 100 devices/1km² | 50m(indoor) | 1-10km² | 3 to 10km/h | NA | NA | 90% | cell-level | UC#5.7 |
|  | 5 | Ambient IoT devices for automated supply distribution | >10 s | 99% | NA | <100 bit/s | Typically, <100 bytes | <1,5 Million/km² | 30m indoor, | 600 000 m² | NA | NA | NA | NA | 3m (Indoor, 90% confidence level and in horizontal) | UC#5.16 |
|  | 6 | Container logistics in a flower auction | [<10 s] | [99,99%] | NA | [5] kbit/s | [96] bits | [< 1,3] Million/km² | [35] m Indoors | [1 700 000] m² | NA | NA | NA | NA | NA | TS#6.1 |
|  |  | Indoor inventory or asset management | Typically, second level | 99% | NA | <2 kbit/s | 96/256 bits | <1,5 Million/km² | 30~50m indoo 200m~400m outdoor | 1-10km² | 3~10km/h | NA | NA | NA | 3 m indoor, Cell-level outdoor | UC#5.1, UC#5.2, UC#5.5, UC#5.7, UC#5.16 TS#6.1 |
| Outdoor | 7 | Medical instrument inventory management and positioning | Several seconds | 99% | NA | <2kbit/s | 176bits | ≥1000/km² | 200m Outdoor | NA | Stationary or walking speed <6km/h | NA | NA | NA | 3 m to 5 m indoor | UC#5.2 |
|  | 8 | Ambient IoT devices for automated supply distribution | >10 s | 99% | NA | <100 bit/s | Typically, <100 bytes | <1,5 Million/km² | , 400m outdoor | 600 000 m² | NA | NA | NA | NA | cell-level outdoor | UC#5.16 |

Table 3 KPI consolidation for sensor data collection

| Deployment |  | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | 1 | Ambient IoT devices in smart home | 20 s | 99.9 % | NA | NA | 8~96bits | NA | [10-30]m Indoors | NA | Stationary | NA | NA | NA | NA | UC#5.6 |
|  | 2 | BSMR environmental supervision | 1s | 99% | NA | <1kbit/s | NA | NA | 30m indoors | NA | Stationary | NA | NA | NA | NA | UC#5.13 |
|  | 3 | Smart Agriculture | >1 s | 99.9% | NA | <1 kbit/s | <1000 bits | 1 per m² | 30-100m | 500-70000 m² per greenhouse | stationary | 1 hour | NA | NA | NA | UC#5.20 |
|  | 4 | Smart livestock farming (pig barns) | >10 s | NA | NA | <500 bit/s | Typically, [< 100 bytes] | 850 000 devices / km² | 250 m Indoor | 6000 m² | Quasi-stationary | 15 minutes to half an hour | NA | NA | NA | UC#5.23 |
|  | 5 | Cows in dairy stable | 1 s | [99,9%] | NA | [< 0.5] kbit/s | [500] bits | [< 1] /m² | [35] m Indoors | [1 800] m² | NA | NA | NA | NA | NA | TS#6.2 |
|  | 6 | Ambient IoT devices for food supply chain | >1 minute | NA | NA | <0.12 bit/s | Typically, < 100 bits | 1.5 Million devices/ km² | NA | 30,000m² | 1m/s | 15min | NA | NA | NA | UC#5.18 |
|  |  | Room envirionment monitoring | 1-20 s | 99 % | NA | <1 kbit/s | < 100 bits | NA | [10-30]m | NA | Stationary | NA | NA | NA | NA | UC#5.6, UC#5.13 |
|  |  | Indoor agriculture / husbandry and supply chain | >10 s | 99.9% | NA | <1 kbit/s | Typically, [< 100 bytes] | 1 per m² | 30 – 20000 m | 6000 m²~30,000m² | Quasi-stationary | 15 minutes to half an hour | NA | NA | NA | UC#5.20, UC#5.23, TS#6.2 UC#5.18 |
| Outdoor | 7 | Remote monitoring of transmission and distribution networks in smart grids | 1 s | 99% | NA | < 1kbit/s | Typically, < 100 bytes | < 10,000 /km² | Outdoor: typically 50-200 meters | [several km² up to 100 000 km²] | Stationary | 5-15 min | NA | NA | several 10 m | UC#5.3 |
|  | 8 | Forest Fire Monitor | > 10sec | 99.9% | NA | NA | NA | 100 per km² | [15-200] meters | [10000 –400,000] km² | stationary | 1hour | NA | NA | NA | UC#5.19 |
|  | 9 | Smart dairy farm | >1 s | 99% | NA | <500 bit/s | Typically, [< 100 bytes] | <5200 devices / km² | [300 m - 500 m] Outdoor | 430000 m² | Up to 3 km/h outdoor | 15 min | NA | NA | NA | UC#5.22 |
|  | 10 | Smart manhole cover remote monitoring | 10 s - 30 s | 99% | NA | <1 kbit/s | Typically, [< 100 bytes] | <1000 devices / km² | [300 m - 700 m] Outdoors | City wide including rural areas | Stationary | 15 min | NA | NA | NA | UC#5.24 |
|  | 11 | Smart bridge health monitoring | 10 s | 99% | NA | <1 kbit/s | Typically, [< 100 bytes] | <1000 devices / km² | [300 m - 500 m] Outdoors | Along the bridge | Stationary | 15 min | NA | NA | NA | UC#5.25 |
|  | 12 | Smart laundry | >10 s | NA | NA | <100 bit/s | Typically, <100 bytes | 20 / 100m² | NA | several m² up to 1000 m² | Up to 6 km/h outdoor | NA | NA | NA | NA | UC#5.15 |
|  |  | Smart grid | 1 s | 99% | NA | < 1kbit/s | Typically, < 100 bytes | < 10,000 /km² | Typically 50-200 meters | [several km² up to 100 000 km²] | Stationary | 5-15 min | NA | NA | several 10 m | UC#5.3 |
|  |  | Outdoor husbandry and logistics | Typically, > tens of seconds | 99% | NA | <500 bit/s | Typically, [< 100 bytes] | <5200 devices / km² | [300 m - 500 m] | 430000 m² | Up to 3 km/h | 15 min | NA | NA | NA | UC#5.22, UC#5.19 |
|  |  | Smart city | 10 s - 30 s | 99% | NA | <1 kbit/s | Typically, [< 100 bytes] | <1000 devices / km² | [300 m - 700 m] | City wide including rural areas | Stationary | 15 min | NA | NA | NA | UC#5.24, UC#5.25 |

Table 4 KPI consolidation for Tracking

| Deployment |  |  | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | 1 |  | Remote lost item finding (Indoor) | 5s | 99% | NA | NA | 256 bits | <250 devices/100m² | 10m | NA | NA | NA | NA | 90% | ~3m | UC#5.8 |
|  | 2 |  | Finding Items in a home | NA | NA | NA | NA | NA | 20 Ambient IoT devices/(100m²) | 10m | NA | Static/ Moving(<1m/s) | 500ms | NA | 95 % | 1-3m | UC#5.10 |
|  | 3 |  | Personal belongings finding (indoor) | 1 s | 99.9% | NA | <1 kbit/s | <1 kbits | <5 per 100 m² | 10 m | <200 m² | Static | 1 per hour | 1s | 99% | [1-3] m | UC#5.12 |
|  | 4 |  | Indoor positioning | 0.5 s | 99.9% | NA | <1 kbit/s | 96 bits | 2500/ 10000m² | 10 m | NA | NA | NA | 0.5 s | 90% | 3 m | UC#5.14 |
|  | 5 |  | Museum guide (indoor) | [2] s | 99.9% | NA | [< 1 kbit/s] UL | [96] bits | [<10,000 /km²] | [30] m | [20,000] m² | [3] km/h | NA | NA | 90% | [3] m | UC#5.21 |
|  |  |  | Indoor tracking | 1s | 99.9% | NA | <1 kbit/s | <1k bits | 25/100 m² | 10m indoors | 200 m² | Static | 1 per hour | 1s | 90% | 1-3 m, | UC#5.8 UC#5.10 UC#5.12, UC#5.14 5.21 |
| outdoor | 4 | Absolute positioning |  | NA | NA | NA | NA | NA | NA | 500m | NA | Outdoor - up to [10 km/h] | NA | [10 s] | [95 %] | Cell-level Horizontal Accuracy | UC#5.9 |
|  |  | Remote lost item finding (Outdoor) |  | 5s | 99% (Note 1) | NA | NA | 256 bits | 10 devices/100m² | 100m | NA | NA | NA | NA | 90% | ~10m | UC#5.8 |
|  | 5 | Personal belongings finding (outdoor) |  | 1 s | 99.9% | NA | <1 kbit/s | <1 kbits | <10 per 100 m² | 100 m | Up to the whole PLMN | Static | 1 per hour | 1 s | 99% | several 10m | UC#5.12 |
|  |  | Outdoor tracking for medium coverage |  | 1s | 99.9% | NA | <1 kbit/s | <1 kbits | <10 per 100 m² | 100 m | Up to the whole PLMN | Static | 1 per hour | 1 s | 99% | several 10m | UC#5.8 UC#5.12 |
|  |  | Outdoor tracking for large coverage |  | NA | NA | NA | NA | NA | NA | 500m | NA | Outdoor - up to [10 km/h] | NA | [10 s] | [95 %] | Cell-level Horizontal Accuracy | UC#5.9 |

Table 5 KPI for actuator control using ambient IoT

| Deployment |  | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | 1 | Medical instrument inventory management and positioning | Several seconds | 99% | NA | <2kbits/s | 176bits | ≥1000/km² | 50m indoor | NA | Static or walking speed <6km/h | NA | NA | NA | 3 m to 5 m indoor | UC#5.11 |
|  | 2 | Indoors elderly health care scenario | 1 s | NA | NA | <1 kbit/s | <100 bits | <20 per 100 m² | 20 m | <250 m² | Static | NA | NA | NA | NA | UC#5.26 |
|  | 3 | Electronic Shelf Label | 1s | 99% | NA | 0.8kbit/s DL | 100 Bytes | less than 1.5 million/km² (inventory), less than 10/100m² (temperature sensor) | 50m indoors | 15,800 square meters | stationary | 20 minutes to 2 hours | NA | NA | NA | TS#6.3 |
|  |  | Indoor actuator control | Several seconds | 99% | NA | <2kbit/s | <100 Bytes | less than 1.5 million/km² | 50m indoors | <250 m² for home, and 15,800 square meters for supmarket | stationary | 20 minutes to 2 hours | NA | NA | 3 m to 5 m indoor | UC#5.11 UC# 5.26,TS#6.3 |
| Outdoor | 4 | Medical instrument inventory management and positioning | Several seconds | 99% | NA | <2kbit/s | 176bits | ≥1000/km² | 200m outdoor | NA | Static or walking speed <6km/h | NA | NA | NA | 3 m to 5 m indoor | UC#5.11 |
|  | 5 | Outdoors elderly health care scenario | 1 s | NA | NA | <1 kbit/s | <100 bits | <20per 100 m² | 200 m | City wide including rural areas | Static | NA | NA | NA | NA | UC#5.26 |
|  | 6 | Ambient IoT controller in smart agriculture | hundreds ms level | 99% | N/A | NA | 128bit (DL) | NA | [500]m outdoors | NA | Static | NA | NA | NA | NA | UC#5.30 |
|  |  | Outdoor actuator control for large coverage | hundreds ms level | 99% | N/A | NA | 128bit (DL) | NA | [500]m | NA | Static | NA | NA | NA | NA | 5.30 |
|  |  | Outdoor actuator control for medium coverage | Several seconds | 99% | NA | <2 kbit/s | <200 bits | <20 per 100 m² | 200 m | City wide including rural areas | Static | NA | NA | NA | NA | UC#5.26,UC#11 |

****************Start of Change *************

7.2	Consolidated potential KPIs

7.2.1 KPIs for inventory

Table 7.2.1-1 KPIs for inventory

| Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inventory or asset management | Typically, seconds level | 99% | NA | <2 Kbit/s | 96/256 bits | <1,5 Million/km² | 30~50m indoor, 200m~400m outdoor | 1-10km² | 3~10km/h | NA | NA | NA | 3 m indoor, cell-level outdoor | UC#5.1, UC#5.2, UC#5.5, UC#5.7, UC#5.16 TS#6.1 |

7.2.2 KPI for sensor data collection

Table 7.2.2-1 KPIs for sensor data collection

| Deployment | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | Room envirionment monitoring (e.g. domicile, machinery) |  | 1-20 s |  | 99 % |  | NA |  | <1 kbit/s |  | < 100 bits |  | NA |  | [10-30]m |  | NA |  | Stationary |  | NA |  | NA |  | NA |  | NA |  |  | UC#5.6, UC#5.13 |
|  | Indoor agriculture and husbandry |  | >10 s |  | 99.9% |  | NA |  | <1 kbit/s |  | Typically, [< 100 bytes] |  | 1 per m² |  | 30 - 200 m |  | 6000 m²~30,000m² |  | Quasi-stationary |  | 15 minutes to half an hour |  | NA |  | NA |  | NA |  |  | UC#5.20, UC#5.23, TS#6.2 UC#5.18 |
| outdoor | Smart grid |  | 1 s |  | 99% |  | NA |  | < 1kbit/s |  | Typically, < 100 bytes |  | < 10,000 /km² |  | Typically 50-200 meters |  | [several km² up to 100 000 km²] |  | Stationary |  | 5-15 min |  | NA |  | NA |  | several 10 m |  |  | UC#5.3 |
|  | Outdoor husbandry and logistics |  | Typically, > tens of seconds |  | 99% |  | NA |  | <500 bit/s |  | Typically, [< 100 bytes] |  | <5200 devices / km² |  | [300 m - 500 m] |  | 430000 m² |  | Up to 3 km/h |  | 15 min |  | NA |  | NA |  | NA |  |  | UC#5.22, UC#5.19 |
|  | Smart city |  | 10 s - 30 s |  | 99% |  | NA |  | <1 kbit/s |  | Typically, [< 100 bytes] |  | <1000 devices / km² |  | [300 m - 700 m] |  | City wide including rural areas |  | Stationary |  | 15 min |  | NA |  | NA |  | NA |  |  | UC#5.24, UC#5.25 |

7.2.3 KPI for tracking

Table 7.2.3-1 KPIs for tracking

| Deployment | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | Indoor tracking | 1s | 99.9% | NA | <1 kbit/s | <1k bits | 25/100 m² | 10m indoor | 200 m² | Static | 1 per hour | 1s | 90% | 1-3 m, 90% availability | UC#5.8 UC#5.10 UC#5.12, UC#5.14 UC#5.21 |
| Outdoor | Outdoor tracking for medium coverage | 1s | 99.9% | NA | <1 kbit/s | <1 kbits | <10 per 100 m² | 100 m | Up to the whole PLMN | Static | 1 per hour | 1 s | 99% | several 10m | UC#5.8 UC5.12 |
|  |  | Outdoor tracking for large coverag |  | NA |  | NA |  | NA |  | NA |  | NA |  | NA |  | 500m |  | NA |  | Outdoor - up to [10 km/h] |  | NA |  | [10 s] |  | [95 %] |  | Cell-level Horizontal Accuracy |  | UC#5.9 |  |

7.2.4 KPI for actuator control

Table 7.2.4 KPIs for actuator control

| Deployment | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor |  | Indoor actuator control |  | Several seconds |  | 99% |  | NA |  | 2 kbit/s |  | <100 Bytes |  | less than 1.5 million/km² |  | 50m indoors |  | <250 m² for home, and 15,800 square meters for supermarket |  | stationary |  | 20 minutes to 2 hours |  | NA |  | NA |  | 3 m to 5 m indoor |  | UC#5.11 UC# 5.26,TS#6.3 |  |
| Outdoor |  | Outdoor actuator control for large coverage |  | hundreds ms level |  | 99% |  | N/A |  | NA |  | 128bit (DL) |  | NA |  | [500]m outdoors |  | NA |  | Static |  | NA |  | NA |  | NA |  | NA |  | 5.30 |  |
|  |  | Outdoor actuator control for medium coverage |  | Several seconds |  | 99% |  | NA |  | <2 kbit/s |  | <200 bits |  | <200 per 100 m² |  | 200 m |  | City wide including rural areas |  | Static |  | NA |  | NA |  | NA |  | 3 m to 5 m indoor |  | UC#5.26,UC#11 |  |

*******************End of Change*****************