3GPP TSG-SA WG1 Meeting #103 	S1-232353

Goteborg, Sweden, 21 – 25 August 2023	 (Revision of S1-232139)

Source:	OPPO

pCR Title:	pCR on ambient IoT functional requirements

Draft Spec:	3GPP TS 22XXX

Agenda item:	7.2

Document for:	Approval

Contact:	Yang Xu (xuyang@oppo.com), Weijie Xu (xuweijie@oppo.com)

1. Introduction

This pCR introduces descriptions and requirements of ambient IoT performance in section 6.

2. Reason for Change

Overall descriptions and performance requirements need to be described.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 22.XXX.

****************Start of Change *************

5	Functional requirements

5.1	General

The functional requirement for Ambient IoT service includes 6 aspects, i.e.

-	Communication;

-	Positioning/location;

-	Management;

-	Collected information and network capability exposure;

-	Charging;

-	Security and privacy

The Ambient IoT devices have some special characteristics regarding Energy harvesting, Low complexity, Low data rates, Life span, Reachability et al, which is described in clause 4.2.

The requirements below in clause 5.2 apply to 5G network, or UEs with the capability to communicate with an Ambient IoT device

5.2	Ambient power-enabled IoT performance requirements

5.2.1 Communication aspects of Ambient IoT devices

The 5G system shall be able to support mechanisms to communicate with an ambient IoT device.

The 5G system shall be able to support 5G network or an Ambient IoT capable UE to communicate with a group of Ambient IoT devices simultaneously.

The 5G network shall support a mechanism to authorize an Ambient IoT capable UE to communicate with an Ambient IoT device.

5.2.2 Positioning/location of Ambient IoT devices

5.2.3 Management of Ambient IoT devices

The 5G system shall support a mechanism to:

- disable the capability to transmit RF signals for one or more Ambient IoT device that is / are currently able to transmit RF signals

- enable the capability to transmit RF signals  for one or more Ambient IoT device that is / are currently disabled to transmit RF signals

5.2.4 Collected information and network capability exposure

Subject to user consent, operator policy and 3rd party request, the 5G system shall be able to get information from Ambient IoT devices (e.g. sensor data) and provide it to a trusted 3rd party via the 5G network.

Subject to user consent, operator’s policy and 3rd party request, the 5G system shall provide information about an Ambient IoT device or a group of Ambient IoT devices (e.g. position) to the trusted 3rd party via the 5G network.

The 5G system shall enable an authorized 3rd party to instruct the 5G network to trigger a group of Ambient IoT devices in an specific area and  which action the Ambient IoT devices need to perform when triggered (e.g. send ID, receive further information, send measurement value).

5.2.5 Charging

5.2.6 Security and privacy

The 5G system shall be able to provide a mechanism to protect the privacy of information (e.g., location and identity) exchanged during communication between an Ambient IoT device and the 5G network or an Ambient IoT capable UE.

*******************End of Change*****************