3GPP TSG-SA WG1 Meeting # 103 	S1-232363

# Gothenborg, Sweden, Aug 21, 2023 - Aug 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.882 | CR | 0002 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating existing CPR in TR 22.882 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_EnergyServ |  |  |  |  |  | Date: |  |  | 2023-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some of the existing CPRs need to be further clarified with additional notes or further consolidated into more general requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align all the description into energy consumption and efficiency; Consolidate information regarding renewable energy and carbon emission into energy consumption information; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The consolidation will not be complete and precise. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3, 6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S1-232317, S1-232180 |  |  |  |  |  |  |  |  |

FIRST CHANGE

6	Consolidated potential requirements

6.3	Monitoring and measurement related to energy consumption and efficiency

This subclause contains the requirements of monitoring and measurement related to energy consumption and efficiency .

Table 6.3-1 –Consolidated Requirements on monitoring and measurement related to energy efficiency

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.3-1 | Subject to operator's policy, the 5G network shall support energy consumption monitoring at per network slice and per subscriber granularity. NOTE x: Energy consumption monitoring as described in the preceding requirement is done by means of averaging or applying a statistical model. The requirement does not imply that some form of 'real time' monitoring is required. The granularity of the subscription policies can either apply to the subscriber (all services), or to particular services. | PR 5.1.6-4 |  |
| CPR 6.3-2 | Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to monitor energy consumption for serving this 3rd party. NOTE 1: The granularity of energy consumption measurement could vary according to different situations, for example, when several services share a same network slice, etc. NOTE 2: The energy consumption information can be related to the network resources of network slice, NPNs, etc. | PR.5.3.6-1 PR.5.4.6-1 PR.5.6.6-1 |  |

6.4	Information exposure related to energy consumption and efficiency

This subclause contains the requirements related to information exposure related to energy consumption and efficiency.

Table 6.4-1 – Consolidated Requirements on information exposure related to Energy Usage

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.4-1 | Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to expose information on energy consumption forserving this 3rd party. NOTE 1: Energy consumption information can include ratio of renewable energy and carbon emission information when available. The reporting period could be set, e.g., on monthly or yearly basis and can vary based on location. NOTE 2: The energy consumption information can be related to the network resources of network slice, NPNs, etc. | PR.5.3.6-1 PR.5.4.6-1 PR.5.9.6-2 PR 5.10.6-1 |  |
| CPR 6.4-2 | Subject to operator’s policy, the 5G system shall support a means to expose energy consumption to authorized third parties for services, including energy consumption information related to the condition of energy credit limit (e.g. when the energy consumption is reaching the energy credit limit). | P.R 5.5.6-3 |  |