3GPP TSG SA WG 1 Meeting #103 	S1-232369

Gothenburg, Sweden, 21 - 25 August 2023	(revision of S1- 232314, S1-232135)

Source:	Xiaomi, NEC, Qualcomm

pCR Title:	Pseudo-CR on Study on Upper layer traffic steer, switch and split over dual 3GPP access, Clause 5.16.5 Update - Existing functionality section update, including removal of FFS

Draft Spec:	3GPP TR 22.841 v1.1.0

Agenda item:	7.9.1

Document for:	Approval

Contact:	Chunhui Zhu (zhuchunhui@xiaomi.com), Jasmina McMenamy (jasmina.mcmenamy@neclab.eu), Go Fujimoto (g-fujimoto@nec.com)

Abstract: This document proposes to update section 5.16.5 in TR 22.841 v1.1.0 by adding further existing requirements that partly or fully cover the use case functionality, and by removing FFS.

1. Introduction

There is an FFS in section 5.16.5 ‘Existing features partly or fully covering the use case functionality’ of TR 22.841, which states:

Further existing features that partly or fully cover the use case functionality are FFS.

After a review of the PRs and section 5.3.5, the conclusion is that the clause ‘Same as in section 5.3.5 of this TR’ reflects the existing features that partly or fully cover the use case functionality.

Section 5.3.5 of TR 22.841 reads as follows:

A) Stage-2&3 include a feature called ATSSS (Access Traffic Steering, Switching, Splitting), e.g., ref to TS 25.301 sec. 5.3.2, which supports functionalities similar to those described in this use case, but limited to 3GPP access plus non-3GPP dual access.

B) Existing service requirements, e.g., from TS 22.261 sec. 6.18 and 6.41, capture some general multi-NW connectivity requirements, which do not fully cover or satisfy the specific target use case and functionalities. See some extracts (not exhaustive) listed below:

[Sec. 6.18: Multi-network connectivity and service delivery across operators]

The 5G system shall enable users to obtain services from more than one network simultaneously on an on-demand basis.

For a user with a single operator subscription, the use of multiple serving networks operated by different operators shall be under the control of the home operator.

When a service is offered by multiple operators, the 5G system shall be able to maintain service continuity with minimum service interruption when the serving network is changed to a different serving network operated by a different operator.

NOTE 1: A business agreement is required between the network operators.

In the event of the same service being offered by multiple operators, unless directed by the home operator's network, the UE shall be prioritized to receive subscribed services from the home operator's network.

NOTE 2: If the service is unavailable (e.g., due to lack of network coverage) from the home operator's network, the UE may be able to receive the service from another operator's network.

NOTE 3: QoS provided by the partner operator's network for the same service will be based on the agreement between the two operators and could be different than that provided by the home operator's network.

[Sec. 6.41: PALS]

Based on localized service agreements, the hosting network shall be able to provide required connectivity and QoS for a UE simultaneously connected to the hosting network for localized services and its home network for home network services.

2. Reason for Change

Removal of the FFS note in section 5.16.5.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to update section 5.16.5 ‘Existing features partly or fully covering the use case functionality’ of TR 22.841. v1.1.0

* * * First Change * * * *

### 5.16.5 	Existing features partly or fully covering the use case functionality

For each UE, same as in section 5.3.5 of this TR.

* * * Next Change * * * *