3GPP TSG SA WG 1 Meeting #103 	S1-232370

Gothenburg, Sweden, 21 - 25 August 2023	(revision of S1-232356)

Source:	Huawei, Hisilicon, CATT

pCR Title:	Pseudo-CR on update clause 5_6 “UE using Terrestrial and Satellite Access”

Draft Spec:	3GPP TR TR22.841 V1.1.0

Agenda item:	7.9.1

Document for:	Approval

Contact:	Shuang ZHANG <zhang.shuang2@huawei.com>

Abstract: This pCR includes a text proposal to update clause 5.6 “UE using Terrestrial and Satellite Access” in TR 22.841 v.1.1.0 , based on associated discussion paper (S1-232204).

1. Introduction

While this use case mainly focuses on switching and steering, the captured description and PRs include splitting (mixing of terms). It was done before discussing the terminology definitions.

2. Reason for Change

a. Clarify the use case and related requirement to accurately capture the needed switching and steering.

b. [PR 5.6.6-001] appears to capture “steering” but miss “switching”.

c. “seamless service continuity” is used for service requirement on 5G system, instead minimized service interruption could be a more appropriate description. Note in TS 22.261 the following definition exists:

3. Conclusions

Update potential requirements for 5GS to correctly address the problems/potential needs of the described use case.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.841 v.1.1.0.

* * * First Change * * * *

### 5.6.2	Pre-conditions

The terrestrial access network of Operator TerrA has good coverage in urban areas, suburban areas, and stopovers along the railway but limited coverage in rural areas and deserts between the stopovers. The satellite access network (e.g. LEO) of Operator SatA has covered the whole country and SatA can provide the communication service on its own.

With the agreement between TerrA and SatA, TerrA can steer or switch the traffic of the same data session of its user over the data paths of the two networks and aggregated in TerrA’s core network.

Eric’s UE e.g. cell phone, or wireless network card is capable of dual 3GPP radio access including satellite access, and has the subscription of TerrA (PLMN NetA) with the service authorization.

By default, only the terrestrial access capability is activated.

* * * Second Change * * * *

### 5.6.3	Service Flows

Eric arrives at Makkah train station and enables the satellite access capability of UE when waiting for the check-in.

The UE registers to NetA through terrestrial access and satellite access network with service preference e.g. instant chat with terrestrial access preferred, HD TV with satellite access preferred.

After Eric is on board, the online video meeting starts via the terrestrial access network first. During 1 hour meeting, UE’s traffic will be autonomously steered, or switched between the traffic paths of terrestrial network and satellite network with the assistance of TerrA’s core network, and aggregated in TerrA’s core network to ensure the video meeting is ongoing without obvious service interruption.

When the train reaches the destination, UE will update registration to NetA once Eric disables the satellite access capability.

* * * Third Change * * * *

### 5.6.4	Post-conditions

The video meeting is finished smoothly without interruption.

The user has no awareness of traffic path steering or switching during the video meeting.

* * * Fourth Change * * * *

### 5.6.6	Potential New Requirements needed to support the use case

[PR 5.6.6-001] Based on operator policy and agreement, the 5G system shall support a mechanism to steer UE’s data flow(s) of the same data session (i.e. the same service) across, or switch the user traffic of the same data session between different 3GPP access networks (i.e. 5G terrestrial and satellite access networks)  for UE with dual 3GPP access capability, considering service preference, traffic characteristics, radio characteristics, QoS, etc.

[PR 5.6.6-002] Based on operator policy and agreement, the 5G system shall support service continuity with minimum service interruption when steering UE’s data flow(s) of the same data session (i.e. the same service) across, or when switching the user traffic of the same data session between two different 3GPP access networks (i.e. 5G terrestrial and 5G satellite access networks) for UE with dual 3GPP access capability in use, based on network availability, service preference, etc.

* * * End of Changes * * * *