3GPP TSG-SA1 Meeting #103	S1-232391

Goteborg, Sweden, 21 – 25 Aug., 2023	  (Revision of 232342, 2264, 2177, 1456)

Title:	Ambient IoT functional CPR

Agenda Item:	7.2

Source:	OPPO (Rapporteur)

Contact:	Yang Xu 	xuyang@oppo.com

Weijie Xu xuweijie@oppo.com

Abstract:  This is the CPRs in S1-231456 which were converged in SA1#102, except the text in yellow highlight for further clarification. It is proposed to bring it back and get approved.

****************Start of Change***************

7	Consolidated potential requirements and KPIs

7.1	Consolidated potential requirements

Mapping table is used per each subclause for consolidated potential requirements.

7.1.1	Communication aspects of Ambient IoT devices

Table 7.1.1 – [Communication aspects of Ambient IoT devices] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.1-2 | The 5G system shall be able to support 5G network or an Ambient IoT capable UE to communicate with a group of Ambient IoT devices simultaneously. | PR 5.2.6-001 PR.5.6.6-001 PR 5.11.6-001 PR.5.12.6-002 PR.5.13.6-001 PR.5.1.6-001 PR.5.4.6-001 PR.5.2.6-002 PR.5.12.6-002 PR.5.8.6-003 PR.5.27-004 PR.5.13.6-004 PR.5.3.6-001 PR 5.22.6-1, PR5.23.6-1, PR.5.24.6-1 PR.5.26.6-001 |  |
| CPR 7.1.1-3 | The 5G network shall support a mechanism to authorize an Ambient IoT capable UE to communicate with an Ambient IoT device. | PR 5.8.6-002 PR.5.12.6-001 PR.5.14.6-001 PR.5.21.6-003 PR.5.21.6-001 |  |

7.1.2	Positioning/location of Ambient IoT devices

Table 7.1.2 – [Positioning/location of Ambient IoT devices] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

7.1.3	Management of Ambient IoT devices

Table 7.1.3  [Management of Ambient IoT devices] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.3-1 | The 5G system shall support a mechanism to: - disable the capability to transmit RF signals for one or more Ambient IoT device that is / are currently able to transmit RF signals - enable the capability to transmit RF signals for one or more Ambient IoT device that is / are currently disabled to transmit RF signals | P.R.5.1.6-005 PR 5.16.6-003 PR 5.17.6-001 |  |

7.1.4	Collected information and network capability exposure

Table 7.1.4  [Collected information and network capability exposure] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.4-1 | Subject to user consent, operator policy and 3rd party request, the 5G system shall be able to get information from Ambient IoT devices (e.g. sensor data) and provide it to a trusted 3rd party via the 5G network. | PR.5.1.6-002 PR.5.12.6-003 PR 5.13.6-005 PR.5.14.6-003 PR 5.16.6-004 PR 5.18.6-1 PR.5.21.6-004 PR 5.2.6-003 |  |
| CPR 7.1.4-2 | Subject to user consent, operator’s policy and 3rd party request, the 5G system shall provide information about an Ambient IoT device or a group of Ambient IoT devices (e.g. position) to the trusted 3rd party via the 5G network. |  |  |
| CPR 7.1.4-3 | The 5G system shall enable an authorized 3rd party to instruct the 5G network to trigger a group of Ambient IoT devices in an specific area and which action the Ambient IoT devices need to perform when triggered (e.g. send ID, receive further information, send measurement value). | PR 5.3.6-003 PR 5.19.6-001 PR.5.27-003 |  |

7.1.5	Charging

Table 7.1.5  [Charging] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

7.1.6	Security and privacy

Table 7.1.6  [Security and privacy] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.6-2 | The 5G system shall be able to provide a mechanism to protect the privacy of information (e.g., location and identity) exchanged during communication between an Ambient IoT device and the 5G network or an Ambient IoT capable UE. | P.R.5.1.6-003 PR 5.3.6-002 PR.5.6-002 P.R.5.13.6-002 PR.5.21.6-005 PR.5.8.6-004 P.R.5.1.6-004 PR 5.8.6-005 PR. 5.12.6-006 P.R.5.13.6-003 PR. 5.14.6-005 PR 5.16.6-001 PR.5.20.6-002 |  |

*******************End of Change*****************