3GPP TSG SA WG1 Meeting #103	S1-232435

# Goteborg, Sweden, 21-25 August				                                            (was S1-232221)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.837 | CR | 0013 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on consolidated KPI tables |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Sensing |  |  |  |  |  | Date: |  |  | 2023-06-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are 2 Editor’s Notes left in section KPI table to be resolved. In parallel, TS22.137 also has discussions on KPI table. It is proposed to utilized a single one for convergence firstly and mapping to the other. Editor’s note: The values in brackets are FFS Values with brackets are determined based on the most stringent value as given in the table. All the values when given are the least requirement for 3GPP system to enable such service. E.g.: - Accuracy 5m, means to satisfy such service, 3GPP system needs to provide accuracy at least 5m, smaller is better. therefore, all the symbols “≤” , and right-hand side range value with “-” are removed. Editor’s note: Different service level of the KPIs is FFS To define sensing service level that similar to positioning services as provided by the 3GPP system is not correct when only taking the accuracy of positioning and/or velocity. Sensing service is provided by taking multiple domains such as delay, AoD, AoA, Doppler, RSRP, Micro-doppler into account. Therefore, it is proposed to take “service category” to distinguish different application groups. Basically: Detection: the radio signal is processed to extract distances, angles, or Doppler shifts, to detect the presence and state (position, velocity) of objects/targets. This mainly enables intruder detection applications in static areas, such as home intruder detection, highway/railway human/animal detection, UAV as intruder flying into restriction area (typical 3D zone).; Category #1. Category #2; Finer location estimate is required. Tracking: the radio signal is processed to extract distances, angles, or Doppler shifts, to detect the presence and state (position, velocity) of objects/targets first, followed by data association (utilizing sensing assistance information) with the object/target for tracking. This mainly enables navigation applications to track the moving area with the object/target, such as car navigation, UAV navigation, AGV tracking; Category #3. Category #4; Finer location estimate is required. Public safety including ADAS: utilize detection and/or tracking applications to rescue and safeguard people.; Category #5. Environment monitoring: relies on features extracted from the received waveform, or directly on the received waveform by utilizing machine learning for classification or regression. In the context of this document, such applications can be achieved by extracting features from the receiver waveform, e.g. tourist monitoring can via measuring the RSRP attenuation to determine the presence of people; rainfall monitoring can via measuring the RSRP attenuation to determine the rainfall intensity. This mainly enables weather monitoring applications in static area such as weather monitoring, tourist monitoring.; Category #6; environment. Motion monitoring: relies on features extracted from the received waveform, or directly on the received waveform by utilizing machine learning for classification or regression. In the context of this document, such applications can be achieved by extracting features from the receiver waveform, e.g. sleep monitoring can via measuring the micro doppler shift over a period to determine people’s respiration rate. This mainly enables monitoring applications such as health monitoring, gesture recognition.; Category #7; Human motions, activities. Category #8; Hand motions. Therefore, it is better to change service level name as category, and change “sensing service area” as “sensing service description” to sync with what will be captured in TS22.137 (which will be discussed in S1-232064 that drafted by Nokia and DT.) |
| Summary of change: |  | Delete Ens Remove brackets and symbols of ≤, - to take the least requiring values Change “service level” to “service category” Change “sensing service area” to “sensing service description” to sync with S1-232064. Sync contents of “sensing service description” and “example use cases” in S1-232064. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specificaiton |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 7.2	Consolidated potential KPIs of sensing results

| Scenario | Sensing service category | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 1 | Indoor/Outdoor | 95 | 10 | 10 | N/A | N/A | 10 NOTE 2 | 10 NOTE 3 | 1000 | 1 | 5 | 2 | intruder detection in smart home, UAV intrusion detection in restricted area |
|  | 2 | Outdoor | 95 | 5 | 1 | N/A | 1 | 10 NOTE 2 | 10 NOTE 3 | 1000 | 1 | 5 | 5 | UAV flight route intrusion detection, intruder detection in surroundings of smart home, highway/railway, tourist spot detection, parking space determination |
|  | 3 | Indoor/Outdoor | 95 | 1 | N/A | 1 NOTE 5 | N/A | 1 NOTE 5 NOTE 8 | 1 x 1 NOTE 9 | 100 NOTE 6; 1000 NOTE 10 | 0.1 NOTE 11 | 2 | 2 | sensing (tracking bicycle) at crossroads with/without obstacle, UAV flight trajectory tracing |
|  | 4 | Indoor/Outdoor | 99 | 0.5 | 0.5 | 0.1 Pedestrian: ≤1.5 Vehicle: ≤15 | N/A Pedestrian: ≤1.5 | 0.5m | 5 x 5 for factories: 0.5 x 0.5 | 250 | 0.25 | 1 | 5 | UAV collision avoidance, AGV/AMR collision avoidance in factory, UAVs/vehicles/pedestrians collision avoidance toSmart Grid equipment (NOTE 7), immersive experience based on sensing |
|  | 5 | Indoor/Outdoor | 95 | 0.1; short-range radar: 0.02 | TBD | 0.03 | N/A | 0.4 | 0.1x 0.6] | 50 | 0.05 | 1 | 1 | ADAS, public safety search and rescue or apprehend |
| Environment monitoring | 6 | Outdoor | 95 | 10 | 0.2 NOTE 15 | N/A | N/A | N/A | N/A | 6000 | 60 | 10 | 3 | rainfall monitoring, flooding monitoring (NOTE 14) |
| Motion monitoring | 7 | Indoor | 95 | N/A | N/A | N/A; | N/A; | N/A | N/A | 60000 | 60 | 5 | 5 | sleep monitoring (NOTE 12), sports monitoring (NOTE 13) |
|  | 8 | Indoor | 95 | N/A | N/A | 0.1 | 0.1 | FFS | FFS | FFS | 0.1 | 5 | 5 | Hand gesture recognition for application navigation and immersive interaction (NOTE 16) |
| NOTE 1: The terms in Table 7.2-1 are found in Section 3.1. NOTE 2: To detect the UAV existance (e.g., for intrusion detection), the sensing resolution of distance is 10m [25]. NOTE 3: To detect the UAV existence, the sensing resolution of velocity is 10m/s [25]. NOTE 4: The typical size (Length x Width x Height) of UAV is 1.6m x 1.5m x 0.7m, the typical size of pedestrian is 0.5m x 0.5m x 1.75m, and the typical size of engineering vehicle is 7.5m x 2.5m x 3.5 m. NOTE 5: The KPI values for UAVs are sourced from [25] and [40] and for factories are sourced from [47]. NOTE 6: The value 100 ms is sourced from [28] and is valid for sensing at crossroads. NOTE 7: The safe distance between pedestrian/vehicle and transmission station/line is 0.7m/0.95m [46]. The size of the park of Smart Grid depends on the real environment. NOTE 8: To track the UAV flying (e.g., for collision detection and warning), the sensing resolution of distance is 1m [25]. NOTE 9: To track the UAV flying, the sensing resolution of velocity is 1m/s [25]. NOTE 10: To realize 1m granularity tracking, when the velocity resolution is 1 m/s, the maximum corresponding sensing service latency is 1s. NOTE 11: Echodyne MESA-DAATM has approximate 1Hz scan rate [40]. NOTE 12: Additional KPI on human motion rate accuracy of 2 times/min (0.033 Hz). NOTE 13: Additional KPI on human motion rate accuracy of 3 times/min (0.05Hz) and 4 times/min (0.07 Hz) NOTE 14: Rainfall estimation accuracy is1 mm/h[39] and describes the closeness of the measured rainfall estimation to its true rainfall value. NOTE 15: This value is for the water level. Description related to NOTE in clause 5.5.1 suggests 0.01 m. [≤0.2] is derived from the water level where people feel difficulty in walking. NOTE 16: This has sependency on use case 5.26 gesture recognization. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *