3GPP TSG SA WG 1 Meeting #103 	S1-232470

Gothenburg, Sweden, 21 - 25 August 2023	(revision of S1-232240)

Source:	Deutsche Telekom, Nokia

pCR Title:	Pseudo-CR on Overview – 4.1 General section

Draft Spec:	3GPP TS 22.137

Agenda item:	7.1.2

Document for:	Approval

Contact:	Vasil Aleksiev, Vasil dot Aleksiev at magenta dot at

Abstract: This document introduces general section in overview of TS 22.137

1. Introduction

This pCR introduces section 4.1 General into the new TS on Sensing.

2. Reason for Change

General text is needed to describe the context of the new TS on Sensing.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 22.137.

* * * First Change * * * *

# 4	Overview

# 4.1	General

5G wireless sensing is a technology enabler to acquire information about characteristics of the environment and/or objects within the environment, that uses radio frequency to determine the distance (range), angle, or instantaneous linear velocity of objects, etc.

Radio frequency sensing functionality provides services for device-free object localization as there is lack of need for the object to be connected via a device in the network. The estimation of parameters such as signal strength, delay, doppler and angle spectrum information is obtained from scattered and reflected radio frequency signals transmitted and received by RAN nodes or UEs by using NR radio frequency signals which, in some cases, previously defined information available in EPC and/or E-UTRA can be used, without leading to impacts on EPC and E-UTRA. By processing these radio frequency signals, features such as the location, velocity, geometric information of the objects can be extracted and further exposed together with contextual information towards different applications.

The capabilities to obtain range, velocity, and angle information from the radio frequency signals can provide a broad range of new functionality, such as various objects detection, object recognition (e.g., vehicle, human, animal, UAV) and high accuracy localization, tracking and activity recognition.

This technical specification describes the sensing technology as part of the 5G system for enabling new services and use cases. 5G wireless sensing service new possibilities for enhanced usage of the telecommunication infrastructure. It provides input to different verticals (i.e., UAVs, smart home, V2X, factories, railways, public safety, etc) enabling applications offering e.g. intruder detection, assisted automotive maneuvering and navigation, trajectory tracing, collision avoidance, traffic management, health and activity monitoring.

In some cases, 5G wireless sensing can also use non-3GPP type sensors (e.g., Radar, camera) to further support the 3GPP-based sensing.

5G wireless sensing service also brings challenges related to confidentiality and privacy. There is a need to protect the sensing data from unauthorized access, interception and eavesdropping, but also to make sure there is compliance with regulation and user awareness.

* * * End of Changes * * * *