3GPP TSG SA WG1 Meeting #103 	S1-232471

Goteborg, Sweden, 21 - 25 August 2023	(revision of S1-232446)

Source:	Xiaomi

pCR Title:	Pseudo-CR on 5.2 Requirements – 5.2.1 General subsection

Draft Spec:	3GPP TS 22.137

Agenda item:	7.1.2

Document for:	Approval

Contact:	Chendong7@xiaomi.com

Abstract: This document introduces 5.2.1 General subsection of 5.2 Requirements in TS 22.137

1. Introduction

This pCR introduced 5.2.1 General subsection of 5.2 Requirements into the new TS on Sensing.

2. Reason for Change

Based on the discussion in the study report TR 22.837, some requirements on general have been introduced. Here we capture these requirements in the 5.2.1 General subsection of 5.2 Requirements into TS 22.137.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 22.137.

* * * First Change * * * *

5.2	5G Wireless Sensing service requirements

### 5.2.1	General

The 5G system shall be able to provide sensing service to detect, identify and/or track one or more objects (e.g., UAVs, birds) and the environment around he object(s).

Editor’s note: Identify is FFS.

Based on operator’s policies, operator’s control and regulation, the 5G system shall be able to collect 3GPP sensing data from sensing receivers for processing.

### 5.2.2	Configuration and authorization

Subject to regulation and operator’s policies, the 5G network shall be able to configure and/or authorize or revoke authorization of sensing transmitter(s) and sensing receiver(s) for 5G wireless sensing service.

NOTE 1: Such configuration and authorization can be based on sensing transmitter or sensing receiver location, specific time, sensing duration, sensing accuracy, target sensing geographical area, establishing of communication to transfer sensing data, etc.

NOTE 2: Such configuration and authorization can also include the selection of multiple sensing transmitters/receivers for 5G wireless sensing services.

### 5.2.3	Network exposure

Subject to user’s consent, regulation and operator’s policy, the 5G network may provide secure means to expose to a trusted third-party the combined sensing result derived from the joint processing of the 3GPP sensing data and non-3GPP sensing data.

Subject to operator’s policy, the 5G network may provide secure means for the operator to expose information towards trusted third-party on whether a given sensing service is available and the estimated quality of the given service for a certain geographic area and time.

* * * End of Changes * * * *