3GPP TSG-SA1 Meeting #103	S1-232517

# Goteborg, Sweden, 21 - 25 August 2023     (revision of S1-232515 of S1-232511 of S1-232029 of S1-232015 )

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0696 | rev | 3 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General and charging requirements to NetShare |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, Charter Communications, ZTE Corporation and CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NetShare |  |  |  |  |  | Date: |  |  | 2023-8-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR proposes to add the requirements to newly defined network sharing scenario to TS 22.261 based on the consolidation of TR 22.851, mainly focusing on general and charging requirements of Indirect Network Sharing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | General and charging requirements are added to capture the requirements to Indirect Network Sharing, with clarification according to TS 22.101. Related description is added to Clause 6.21. Annex I is updated. Some editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements related to indirect network sharing in Rel-19 are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.21, Annex I |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

6.21	NG-RAN Sharing

6.21.1	Description

The increased density of access nodes needed to meet future performance objectives poses considerable challenges in deployment and acquiring spectrum and antenna locations. RAN sharing is seen as a technical solution to these issues.

In RAN Sharing operations, NG-RAN resources can be used by multiple network operators. Indirect Network Sharing is one of the possible sharing methods.

6.21.2	Requirements

6.21.2.1	General

Requirements related to NG-RAN sharing are described in 3GPP TS 22.101 [6], mainly in clause 28.2.

A 5G satellite access network shall support NG-RAN sharing.

6.21.2.2	Indirect network sharing

The 5G system shall be able to support Indirect Network Sharing between the Shared NG-RAN and one or more Participating NG-RAN Operators’ core networks, by means of the connection being routed through the Hosting NG-RAN Operator’s core network.

NOTE:	For more information on Indirect Network Sharing see Annex I.

Indirect Network Sharing shall be transparent to the user.

NOTE: This requirement is aligned with the existing requirement in 3GPP TS 22.101 [6] clause 4.9.

Subject to the agreement between hosting and the participating operator, the following service requirements apply to Indirect Network Sharing, for a UE of the Participating NG-RAN Operator, besides 3GPP TS 22.101 [6] clause 4.2.1:

-	The 5G system shall support a mechanism to enable a UE to access their subscribed PLMN services when accessing a Shared NG-RAN.

-	The 5G system shall support a mechanism to enable a UE to obtain its subscribed services, including Hosted Services, of participating operator via a Shared NG-RAN.

NOTE: the requirement assumes no impact to UE.

The following accounting and charging requirements apply to Indirect Network Sharing, besides 3GPP TS 22.101 [6] clause 16 and 28.2.3:

-	The 5G core network shall be able to support collection of charging information associated with a UE accessing a Shared NG-RAN using Indirect Network Sharing, which refers to the resource usage of hosting operator’s core network.

Second Change

Annex I (informative):
Indirect Network Sharing of NG-RAN Sharing

This annex clarifies scenarios applicable for Indirect Network Sharing between a Shared NG-RAN and the corresponding participating operator's core network as alternatives for operators who intend to deploy a NG Radio Access Network. Examples of such scenarios include wide-range coverage of rural areas, long-distance road coverage, compatibility with existing networks, service consistency, cooperation with diverse networks, considering different operators' strategies, commercial agreements, and specific rules/legislation in different countries.

Two or more operators have deployed or plan to deploy 5G access networks and core networks with MOCN. The challenge for the network operators is the maintenance generated by the interconnection (e.g., number of network interfaces) between the shared RAN and two or more core networks, especially for a large number of shared base stations. For these reasons, it is valuable to introduce a newly supported network sharing scenario as the operators' agreement.

In case of Indirect Network Sharing, the communication between the Shared NG-RAN and the Participating Operator’s core network happens via a number of inter-operator interfaces that are independent of the actual number of base stations at the Hosting NG-RAN Operator.

There is an agreement between all the operators to work together and build a shared network together cover the entire country, utilizing the different operator’s allocated spectrum appropriately in different parts of the coverage area (for example, Low Traffic Areas, LTA and High Traffic Areas, HTA). Multiple operators share one NG-RAN, but their 5GCs are independent. UEs access their subscribed PLMN services and/or subscribed services, including Hosted Services, provided by their participating operators respectively, when entering the Shared NG-RAN.

The following figures illustrate the example in which Indirect Network Sharing is realized via routing through the Hosting Operator’s core network.

-	The Hosting NG-RAN Operator 1, as illustrated below, can share its NG-RAN with the participating operators with or without direct connections between the shared access and the core networks of the participating operators.

-	The Participating NG-RAN Operators 2 and 3, using shared NG-RAN resources provided by the Hosting NG-RAN Operator, e.g., within a specific 5G frequency band or within a specific area, when	the Shared NG-RAN does not have direct connections between the shared access and the core networks of the Participating NG-RAN Operators 2 and 3.

-	The Participating NG-RAN Operator 4, using shared NG-RAN resources provided by the Hosting NG-RAN Operator, with direct connections between the shared NG-RAN and the core networks of the participating operator, is in a MOCN arrangement.

Figure I-1: Different options both direct and indirect connections between the Shared NG-RAN and the core networks of the participating operators.

Figure I-2: Indirect Network Sharing scenario involving core network of Hosting NG-RAN Operator between the Shared NG-RAN and the core networks of the participating operators.

The network sharing partners can set a specific sharing allocation model for the network sharing method they are using. The collection of charging information associated with the sharing method that the UE accesses with can be possible. It is also necessary to understand the charging information between the networks of both parties, e.g., the number of the users, and how long users using a certain shared network method will take. This information is also needed when users use the participating operator's hosted services they have subscribed to and their flexible charging via Shared NG-RAN.