3GPP TSG-SA1 Meeting #103	S1-232532

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.865 | CR | 0003 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on use case on Store and Forward-MO for TR 22.865 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Reliance Jio, Nokia, Nokia Shanghai Bell, Tejas Networks, CEWIT, Novamint, Sateliot, CICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-09 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR proposes the applicable potential requirements needed to support the use case on Store and Forward – MO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New potential requirements has been identified for UE registration and authorization in S&F Satellite operation mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some potential requirements in the use case will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 5	Use cases

## 5.1	Use case on store and forward - MO

### 5.1.1	Description

This use case illustrates the realization of a S&F service between a UE with satellite access and an Application Server for a delay-tolerant/non-real-time IoT NTN service in the case of a Mobile Originated message.

A description of store and forward operation is provided in Annex A.

Company TrackingInc offers a service of remote monitoring of fields and deploys and tracks many battery-powered IoT type UEs across the globe. All the IoT remote monitoring UEs deployed include a 5G communication with satellite access. Some of the UEs are deployed in a remote area where there is no mobile coverage by MNO and only satellite is possible.

For the satellite access, TrackingInc uses the service of IoTSAT for the 5G IoT connectivity by satellite and IoTSAT uses a LEO constellation which supports S&F operation mode.

All IoT remote monitoring UEs regularly send information related to the area they are monitoring to the application server of TrackingInc and sometimes receive new parameters from the application server. In most of the cases, the messages exchanged are delay-tolerant/non-real-time IoT.

### 5.1.2	Pre-conditions

In the present use case, the IoT remote monitoring UE is in a remote area with no ground stations available for feeder link connectivity and the IoT remote monitoring UE is aware that IoTSAT constellation operates in S&F mode.

### 5.1.3	Service Flows

The IoT remote monitoring UE needs to send a message to the TrackingInc application server. The UE waits for satellite network coverage and sends its message when the satellite passes by.

The IoT remote monitoring UE and the satellite providing coverage interact over the service link, allowing the UE to transfer the message to the satellite, which has no connectivity to the ground segment. And consequently, the satellite has to store locally the received message.

At this point:

- Limitations to the size/amount of data that can be sent from the UE could be enforced.

- Forwarding priority for the stored data to the ground station and data retention period for the exchanged data could be established.

- Acknowledgement of the received data by the satellite could be issued.

At a later time, the satellite with the stored message establishes connectivity with the ground network via a feeder link and relays/forwards/downloads the message to the ground network. All accumulated and stored MO messages are delivered to the ground once the feeder link is available, at the same time, all accumulated and stored relevant MT messages are also delivered to the satellite via the same feeder link, which will impact the performance of the feeder link, 5GC, and satellite significantly. The relevant performance optimization method will be taken into consideration accordingly.

The ground network, based on established connectivity configuration and routing, delivers message to the TrackingInc application server.

### 5.1.4	Post-conditions

The message generated by the IoT remote monitoring UE has been either delivered successfully to the TrackingInc application server without relying on a continuous end-to-end network connectivity path between them or, in case the data retention period has been exceeded, the message has been discarded.

### 5.1.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2], clause 6.3.2.3 on satellite access includes the following requirements:

The 5G system shall be able to provide services using satellite access.

The 5G system with satellite access shall be able to support low power MIoT type of communications.

However, it is not sufficient in regards of S&F operation especially for the delivery of delay-tolerant/non-real-time IoT NTN services with NGSO satellites.

### 5.1.6	Potential New Requirements needed to support the use case

[PR 5.1.6-001] The 5G system with satellite access shall be able to support store and forward operation.

[PR 5.1.6-002] The 5G system with satellite access shall be able to inform a UE that "store and forward" operation is applied.

[PR 5.1.6-003] Subject to operator policy, the 5G system with satellite access supporting store and forward operation shall be able to allow the operator or a trusted 3rd party to set and enforce, on a per UE basis, a S&F data retention period.

[PR 5.1.6-004] Subject to operator policy, the 5G system with satellite access supporting store and forward operation shall be able to allow the operator or a trusted 3rd party to set and enforce, on a per UE basis, a S&F data storage quota.

[PR 5.1.6-005] The 5G system with satellite access supporting store and forward operation shall be able to support a mechanism to configure and provision specific required QoS and policies for S&F operation (e.g. forwarding priority, acknowledgment policy).

[PR 5.1.6-006] The 5G system with satellite access shall be able to provide integrity protection and confidentiality for communications between an authorized UE and the network when store and forward operation is applied.

[PR 5.1.6-007] The 5G system with satellite access supporting the S&F operation shall be able to support suitable means to resume communication between the ground station and satellite once the feeder link becomes available.

[PR.5.1.6-008] Subject to operator’s policies, a 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support forwarding of the stored data from one satellite to another satellite, which has an available feeder link to the ground network, through Inter-Satellite Links.

[PR.5.1.6-009] The 5G system with satellite access and with support for S&F Satellite operation mode shall be able to support mechanisms for allowing a UE to register with the network when the network is operating in S&F Satellite operation mode.

[PR.5.1.6-010] The 5G system with satellite access and support for S&F Operation Mode, shall support mechanisms to authorize subscribers for receiving services under S&F Operation mode.

NOTE: 	It is assumed that the constellation knows which satellite has a feeder link available. However, this is outside the scope of 3GPP.