3GPP TSG SA WG 1 Meeting #103 	S1-232622

Gothenburg, Sweden, 21 - 25 August 2023	(revision of S1-232617)

Source:	Nokia, Nokia Shanghai Bell, Deutsche Telekom

pCR Title:	Pseudo-CR on 5G Wireless sensing performance requirements

Draft Spec:	3GPP TS 22.137

Agenda item:	7.1.2

Document for:	Approval

Contact:	hideaki.takahashi@nokia.com

Abstract: This document introduces descriptions and requirements of 5G wireless sensing performance.

1. Introduction

This pCR introduces descriptions and requirements of 5G wireless sensing performance in section 6.

2. Reason for Change

Overall descriptions and performance requirements need to be described.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 22.137.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication".

[3]	5GAA_White_Paper_C-V2X Use Cases Volume II: Examples and Service Level Requirements.

* * * Next Change * * * *

# 6	5G wireless sensing performance requirements

## 6.1	Description

5G wireless Sensing service is required to fulfil different performance requirement (e.g., accuracy, resolution, latency, etc.) based on the characteristics of one or multiple target object(s) and/or the environment to be sensed in a target sensing service area. The following set of key performance requirements is used and shown for each service scenario in Table 6.2-1.

-	Accuracy of positioning estimate describes the closeness of the measured sensing result (i.e., position) of the target object to its true position value. It can be further derived into a horizontal sensing accuracy – referring to the sensing result error in a 2D reference or horizontal plane, and into a vertical sensing accuracy – referring to the sensing result error on the vertical axis or altitude.

-	Accuracy of velocity estimate describes the closeness of the measured sensing result (i.e., velocity) of the target object to its true velocity.

-	Confidence level describes the percentage of all the possible measured sensing results that can be expected to include the true sensing result considering the accuracy.

-	Sensing Resolution describes the minimum difference in the measured magnitude of target objects (e.g., range, velocity) to be allowed to detect objects in different magnitude.

-	Missed detection probability describes the conditional probability of not detecting the presence of target object/environment when the target object/environment is present. This probability is denoted by the ratio of the number of events falsely identified as negative, over the total number of events with a positive state. It applies only to binary sensing results.

NOTE 1:	An event with a positive state refers to the presence of the characteristics of a target object or environment, including the event falsely identified as being negative and truly identified as being positive.

-	False alarm probability describes the conditional probability of falsely detecting the presence of target object/environment when the target object/environment is not present. This probability is denoted by the ratio of the number of events falsely identified as being positive, over the total number of events with a negative state. It applies only to binary sensing results.

NOTE 2:	An event with a negative state refers to the non-presence of the characteristics of a target object or environment, including the event falsely identified as being positive and truly identified as being negative.

-	Max sensing service latency: time elapsed between the event triggering the determination of the sensing result and the availability of the sensing result at the sensing system interface.

-	Refreshing rate: rate at which the sensing result is generated by the sensing system. It is the inverse of the time elapsed between two successive sensing results.

5G wireless sensing performance requirements are applied to 3GPP sensing data and sensing results.

## 6.2	Requirements

The 5G system shall be able to provide sensing results with the performance requirements in Table 6.2-1.

Table 6.2-1:	Performance requirements for 5G Wireless Sensing

| Scenario | Sensing service category |  |  | Confidence level [%] |  |  |  |  |  | Accuracy of positioning estimate by sensing (for a target confidence level) |  |  |  |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  |  |  |  | Sensing resolution |  |  |  | Max sensing service latency [ms] |  | Refreshing rate [s] |  | Missed detection [%] |  | False alarm [%] |  | Sensing service description in a target sensing service area |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Horizontal [m] |  |  | Vertical [m] |  | Horizontal [m/s] |  |  | Vertical [m/s] |  | Range resolution [m] |  |  | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |  |  |  |  |  |  |
| Object detection and tracking |  |  | 1 |  |  |  | 95 |  | 10 |  |  | 10 |  | N/A |  |  | N/A |  | 10 |  |  | 10 |  | 1000 |  | 1 |  | 5 |  | 2 |  | Indoor/outdoor (e.g., detection of human, UAV) |  |
|  |  |  | 2 |  |  |  | 95 |  | 5 |  |  | 1 |  | 1 |  |  | 1 |  | 1 |  |  | 1 |  | 1000 |  | 1 |  | 5 |  | 5 |  | Outdoor (e.g., detection of human, UAV) requiring higher performance than category 1 |  |
|  |  |  | 3 |  |  |  | 95 |  | 1 |  |  | N/A |  | 1 |  |  | N/A |  | 1 |  |  | 1 x 1 |  | 100 (NOTE 2), or 1000 (NOTE 3) |  | 0.1 |  | 2 |  | 2 |  | Indoor/outdoor (e.g., detection tracking of human, animal, UAV) |  |
|  |  |  | 4 |  |  |  | 95 |  | 0.5 |  |  | 0.5 |  | 15 for vehicle, otherwise, 0.1 |  |  | N/A |  | 0.5 |  |  | 5 x 5 for factories 0.5 may be needed |  | 250 |  | 0.25 |  | 1 |  | 5 |  | Indoor/outdoor (e.g., detection tracking of human, animal, UAV, AGV, vehicle) requiring higher performance than category 3 |  |
|  |  |  | 5 |  |  |  | 99 for public safety, otherwise, 95 |  | short range radar; 0.02, otherwise; 0.1 |  |  | 0.5 |  | 1.5 for pedestrian, otherwise, 0.03 |  |  | 1.5 for pedestrian, otherwise, N/A |  | 0.4 |  |  | 0.1 x 0.6 |  | 50 |  | 0.05 |  | 1 |  | 1 |  | Indoor/outdoor (e.g., detection tracking of human, animal, UAV, AGV, vehicle) aimed for mission critical and safety requiring higher performance than category 4 |  |
| Environment monitoring | 6 | 95 | 10 | 0.2 | N/A | N/A | N/A | N/A | 6000 | 60 | 10 | 3 | Nature of environments monitored by sensing. |
| Motion monitoring | 7 | 95 | N/A | N/A | N/A | N/A | N/A | N/A | 60000 | 60 | 5 | 5 | Human motions and activities obtained by sensing (NOTE 4). |
|  | 8 | 95 | 0.2 | 0.2 | 0.1 | 0.1 | 0.375 | 0.3 | 5 | 0.1 | 5 | 5 | Human hand gestures obtained by sensing |
| NOTE 1: The additional information on some of the performance requirements can be found in [2]. NOTE 2: The value 100 ms is sourced from [3] and is valid for sensing at crossroads. NOTE 3: To realize 1m granularity tracking, when the velocity resolution is 1 m/s, the maximum corresponding sensing service latency is 1 s. NOTE 4: Additional KPI on human motion rate accuracy is defined for contactless sleep/sports monitoring [2]. |  |  |  |  |