3GPP TSG SA WG 1 Meeting #103 	S1-232640

Gothenburg, Sweden, 21 - 25 August 2023	               (revision of S1-232442,S1-232216, S1232280)

Source:	Ericsson, Nokia, Deutsche Telekom AG

pCR Title:	Pseudo-CR on sensing operation chapter for overview

Draft Spec:	3GPP TS 22.137

Agenda item:	7.1.2

Document for:	Approval

Contact:	Daniel Lönnblad Daniel.lonnblad at Ericsson.com

Abstract: This is a proposal for a chapter in overview called sensing operation. High level description of sensing operation and describing some of the definitions in this document.

1. Introduction

To introduce a reader of this TS in general overview of the sensing operation, to allow for better understanding when reading the document.

2. Reason for Change

To get a better understanding of sensing operation and some of the terms used in this document

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 22.137

* * * First Change * * * *

# 4	Overview

## 4.2 Sensing operation

5G Wireless sensing is a technology enabler to acquire information about characteristics of the environment and/or objects within the environment, that uses NR radio waves to determine the distance (range), angle, or instantaneous linear velocity of objects, etc. The operation of the 5G wireless sensing service, a.k.a. sensing operation, relies on processing the transmissions, reflections, and scattering of wireless sensing signals.

5G wireless sensing, therefore, has the opportunity to enhance the 5G system from a communication network to a wireless communication and sensing network, where it uses 5G entities to sense objects and the environment in its surroundings.

Sensing operation can be implemented in a couple of different ways, from radar like sensing where the sensing transmitter and sensing receiver are co-located in the same entity (figure 4.2-1), also called Monostatic sensing, to have the sensing receiver and sensing transmitter in different entities (figure 4.2-2), also called Bistatic sensing. A more advanced scenario with multiple sensing transmitters and receivers is also possible, called Multistatic sensing. The reflections of the sensing signal sent from the sensing transmitter are received by the sensing receiver and processed to obtain characteristics of the sensed object and its environment (e.g. location). For example measuring parameters such as Doppler shift of the received signal, the velocity of the sensed object can be estimated. Below is a schematic picture showing the different parts of the sensing operation and the nomenclature used in this specification.

Non-3GPP based sensing is when information from non-3GPP sensors is used to determine characteristics of objects and their environment. These non-3GPP sensors could include radar camera or Wi-Fi sensing. While the mechanism of these types of sensing is not considered in this specification, non-3GPP sensing data from these non-3GPP sensors, if available, can be used in 5G wireless sensing to achieve improved sensing result, or in any other way to enhance the sensing service.

This TS describe requirement for services consumed by either the 3GPP system or trusted third party and not the internal 3GPP usage (e.g the use of sensing assisted communication).

It is expected that sensing service will work independently of positioning service.

There are some factors affecting what resolution/granularity the sensing service can achieve.

- Operating frequencies and used bandwidth, typically higher resolution is achieved with higher bandwidth and higher frequency.

- The propagation environment also plays an important role. Environments with many objects that can block radio signals, leading to interruption of the Line of Sight (LOS) path and reflections/scattering can increase the number of interfering signal paths, as well as clutter and thus make it harder to reach higher resolutions.

Sensing operations, such as authorization, and parameters such as sensing area, sensing operation period and sensing operation time window etc., could be configured and adjusted for efficient use of all kinds of resources, such as energy and radio spectrum, etc.

Figure 4.2-1: Example of sensing with co-located sensing receiver and sensing transmitter

Figure 4.2-2: Example of sensing with separated sensing receiver and sensing transmitter

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>