3GPP TSG- SA1 Meeting # 103	S1-232656

Gothenburg, Sweden,  21 - 25 August 2023	(revision of S1-232610, S1-232281, S1-232106)

# Source:	China Telecom

# Title:	New WID on local traffic routing for multi-access UE

# Document for:	Approval

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Local traffic routing for multi-access UE

Acronym: LTR_MA

Unique identifier:

Potential target Release: Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
| X | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | N/A |

Dependency on non-3GPP (draft) specification: N/A

3	Justification

The 5G system supports ATSSS functionality. The 5G system also supports local traffic routing when the UE is connected to 5G via one access technology (e.g. UL CL for 3GPP access and NSWO for non-3GPP access.) However, when the UE is connected to 5G system via multiple accesses, local traffic routing is not supported on any access. This may result in inefficient traffic routing and large latency. To enable data path selection/optimization, it is proposed to support local traffic routing on each access when the UE is connected to 5G via multiple accesses. In addition, the 5G system shall be able to optimize data transmissions considering these available data paths.

4	Objective

Update the stage 1 specifications to support local traffic routing for UEs with multiple accesses to 5G. This applies to the scenario with simultaneous 3GPP and non-3GPP accesses.

5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 22.261 | Update the stage 1 specifications to support local traffic routing for UEs with multiple accesses to 5G. | TSG#101 (Sept 2023) |  |

6	Work item Rapporteur(s)

Yinglin Chen, China Telecom, chenyl37@chinatelecom.cn

7	Work item leadership

SA1

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

China Telecom

CATT

Apple

vivo

ZTE

KPN