| 3GPP TS 22.cde V0.1.0 (2023-08) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group TSG SA; Service requirements for ambient power-enabled IoT; Stage 1 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Overview	8

4.1	Introduction	8

4.2	Characteristics of Ambient IoT	8

4.3	Typical Ambient IoT use cases	9

4.4	Communication modes	9

5	Functional service requirements of Ambient IoT	9

5.1	General	9

5.2	Functional service requirements of Ambient IoT	10

5.2.1 	Communication aspects	10

5.2.2 	Positioning	10

5.2.3 	Management	10

5.2.4 	Exposure	10

5.2.5 	Charging	10

5.2.6 	Security and privacy	11

6	Performance service requirements of Ambient IoT	11

6.1	General	11

6.2	Performance service requirements for inventory	11

6.3	Performance service requirements for sensor data collection	11

6.4	Performance service requirements for tracking	12

6.5	Performance service requirements for actuator control	12

Annex <A> (normative): <Normative annex for a Technical Specification>	14

Annex <B> (informative): <Informative annex for a Technical Specification>	15

B.1	Heading levels in an annex	15

Annex <B>: <Informative annex title for a Technical Report>	16

Annex <C> (informative): Bibliography	17

Annex <D> (informative): Index	18

Annex <X> (informative): Change history	19

Foreword

This Technical Specification|Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

- Scope

The present document describes service and performance requirements for ambient power-enabled Internet of Things (i.e. Ambient IoT). In the context of the present document, Ambient IoT device is an IoT device powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g. using a capacitor) and the energy is provided through the harvesting of radio waves, light, motion, heat, or any other power source that could be seen suitable. An Ambient IoT device has low complexity, small size and lower capabilities and lower power consumption than previously defined 3GPP IoT devices (e.g. NB-IoT/eMTC devices). Ambient IoT devices can be maintenance free and can have long life span (e.g. more than 10 years).

The aspects addressed in this document include:

-	Overview of Ambient IoT service and operation,

-	Functional service requirements for Ambient IoT,

-	Performance service requirements for Ambient IoT.

NOTE: How Ambient IoT device performs energy harvesting is out of scope of this technical specification.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

4	Overview

4.1	Introduction

In the 5G era, various IoT technologies such as eMTC, NB-IoT, and RedCap have been developed to fulfil the increasing demand from verticals. These IoT technologies have achieved low cost, low power and massive connections and can meet requirements of many applications. However, there are still some use cases and applications that cannot be addressed. For example, a conventional battery-powered device cannot be deployed in extreme environmental conditions (e.g. high pressure, extremely high/low temperature, humid environment). Also, conventional battery devices are less suitable where maintenance-free devices are required (e.g. where the devices are inaccessible and it is not possible to replace the device battery). Finally, ultra-low complexity, very small device size/form factor (e.g. thickness of mm), longer life cycle, etc. are required for some use cases. Ambient IoT is a promising technology to fulfil these unmet market requirements.

This technical specification describes the Ambient IoT technology service requirements as part of the 5G system to enable new services and use cases. Ambient IoT has the potential to benefit a large number of vertical industries, e.g. smart manufacturing, logistics and warehousing, smart grid, agriculture, and smart home by providing functionalities that fulfil the needs of industrial use cases. Therefore, a new kind of IoT service for the vertical industries will be enabled by combining Ambient IoT with cellular networks, vastly benefitting the 3GPP ecosystem.

4.2	Characteristics of Ambient IoT

Not all Ambient IoT devices are the same. Nevertheless, Ambient IoT devices have the following characteristics.

-	Energy harvesting. An Ambient IoT device is an IoT device powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g. using a capacitor). Energy is provided through harvesting of radio waves, light, motion, heat, or any other suitable power source. Energy harvesting can be continuous or incidental (e.g. based on the vibration that a vibration sensor has to report). It cannot be assumed that Ambient IoT devices always have enough power to initiate or receive communication.

-	Low complexity. Ambient IoT devices are expected to have lower complexity, smaller size, reduced capabilities and lower power consumption than previously defined 3GPP IoT devices (e.g. NB-IoT/eMTC devices). Low complexity of Ambient IoT devices is also expected to be reflected in efficient use of network resources. In general, Ambient IoT applications will deploy very large numbers of Ambient IoT devices.

-	Low data rates. Generally, Ambient IoT data transmissions contain only a few hundred bits of data. Typical 5G services with high data rates such as streaming are not supported.

-	Life span. Ambient IoT devices can be maintenance free and can have long life span (e.g. more than 10 years). However, the life span of an Ambient IoT device can also be relatively short, e.g. when tracking a package through a logistics chain.

-	Reachability. Ambient IoT devices can have different reachability characteristics. Some Ambient IoT devices will only be able to receive Mobile Terminated communication when they occasionally obtain power. Some Ambient IoT devices can be powered on demand when they need to be reachable. Some Ambient IoT devices will be able to receive Mobile Terminated communication on a regular basis and have reachability characteristics similar to mobile phones.

Editor’s Note:	Terminology for the Reachability text to be revisited. Specifically how to consider also Mobile Originated communication.

4.3	Typical Ambient IoT use cases

Ambient IoT can support many different use cases. Nevertheless, in general the Ambient IoT use cases can be characterised in four different use case categories:

-	Inventory taking. With inventory taking, the main purpose is to discover what goods (e.g. boxes, containers, packages, tools) are present in a specific area. Upon request sent by the network within the specific area, Ambient IoT devices attached to these goods report an identifier associated with the good, possibly supplemented with other information such as status, measurement results and/or location.

-	Sensor data collection. With sensor data collection, the Ambient IoT device is associated with a sensor. Transfer of sensor data can be initiated by the Ambient IoT device, e.g. periodically or when the Ambient IoT device has power, or can be triggered by the network.

-	Asset tracking. With asset tracking, the main purpose is to determine the location of goods. Ambient IoT devices attached to these goods report an identifier associated with the good. This can then be combined with location information. Asset tracking can also be initiated by an Ambient IoT capable UE, thus finding the location of Ambient IoT devices within a particular range of the UE.

-	Actuator control. With actuator control, the Ambient IoT device is associated with an actuator. Transfer of actuator commands is generally initiated by the network.

4.4	Communication modes

Editor’s Note:	It is FFS whether to describe communication modes here, and/or define them in the definition section, or not at all.

5	Functional service requirements of Ambient IoT

5.1	General

The functional requirement for Ambient IoT service includes 6 aspects, i.e.

-	Communication;

-	Positioning/location;

-	Management;

-	Collected information and network capability exposure;

-	Charging;

-	Security and privacy

The Ambient IoT devices have some special characteristics such as Energy harvesting, Low complexity, Low data rates, Life span, and Reachability, etc.

The requirements below in clause 5.2 only apply to 5G network, and 3GPP UEs with the capability to communicate with an Ambient IoT device.

5.2	Functional service requirements of Ambient IoT

5.2.1 	Communication aspects

The 5G system shall be able to support 5G network or an Ambient IoT capable UE to communicate with a group of Ambient IoT devices simultaneously.

The 5G network shall support a mechanism to authorize an Ambient IoT capable UE to communicate with an Ambient IoT device.

5.2.2 	Positioning

The 5G system shall support location services for Ambient IoT devices (e.g. to locate Ambient IoT devices using absolute or relative positioning methods)

NOTE 1: The intention is not to use Ambient IoT devices to locate other Ambient IoT devices.

5.2.3 	Management

The 5G network shall support suitable management mechanisms for an Ambient IoT device or a group of Ambient IoT devices.

The 5G system shall support a mechanism to:

- disable the capability to transmit RF signals for one or more Ambient IoT device that is / are currently able to transmit RF signals

- enable the capability to transmit RF signals for one or more Ambient IoT device that is / are currently disabled to transmit RF signals

Based on operator policy, the 5G system shall provide a suitable mechanism to permanently disable the capability of an Ambient IoT device or a group of Ambient IoT devices to transmit RF signals.

5.2.4 	Exposure

Subject to user consent, operator policy and 3rd party request, the 5G system shall be able to get information from Ambient IoT devices (e.g. sensor data) and provide it to a trusted 3rd party via the 5G network.

Subject to user consent, operator’s policy and 3rd party request, the 5G system shall provide information about an Ambient IoT device or a group of Ambient IoT devices (e.g. position) to the trusted 3rd party via the 5G network.

The 5G system shall enable an authorized 3rd party to instruct the 5G network to trigger a group of Ambient IoT devices in an specific area and which action the Ambient IoT devices need to perform when triggered (e.g. send ID, receive further information, send measurement value).

5.2.5 	Charging

The 5G system shall be able to collect charging information in a suitable way for using Ambient IoT services on per Ambient IoT device basis or a group of Ambient IoT devices (e.g., total number of communications per charging period).

5.2.6 	Security and privacy

The 5G system shall enable security protection suitable for Ambient IoT, without compromising overall 5G security protection.

The 5G system shall be able to provide a mechanism to protect the privacy of information (e.g., location and identity) exchanged during communication between an Ambient IoT device and the 5G network or an Ambient IoT capable UE.

Based on subscription and operator policies, the 5G system shall authorize an Ambient IoT capable UE to communicate with a specific Ambient IoT device or with a group of Ambient IoT devices.

6	Performance service requirements of Ambient IoT

6.1	General

Ambient IoT service can be categorized into 4 categories, namely inventory, sensor data collection, tracking and actuator control. The corresponding performance services requirements are listed in the following subclauses.

6.2	Performance service requirements for inventory

Table 6.2-1 KPIs for inventory

| Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inventory or asset management | Typically, seconds level | 99% | NA | <2 kbit/s | 96/256 bits | <1,5 Million/km² | 30~50 m indoor, 200 m~400 m outdoor | 1-10 km² | 3~10 km/h | NA | NA | NA | 3 m indoor, cell-level outdoor |  |
| NOTE 1: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.3	Performance service requirements for sensor data collection

Table 6.3-1 KPIs for sensor data collection

| Deployment | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | Room envirionment monitoring (e.g. domicile, machinery) | 1-20 s | 99 % | NA | <1 kbit/s | < 100 bits | NA | [10-30] m | NA | Stationary | NA | NA | NA | NA |  |
|  | Indoor agriculture and husbandry | >10 s | 99.9% | NA | <1 kbit/s | Typically, [< 100 bytes] | 1 per m² | 30 - 200 m | 6000 m²~30,000 m² | Quasi-stationary | 15 min to half an hour | NA | NA | NA |  |
| Outdoor | Smart grid | 1 s | 99% | NA | < 1 kbit/s | Typically, < 100 bytes | < 10,000 /km² | Typically, 50 m-200 m | [several km² up to 100 000 km²] | Stationary | 5-15 min | NA | NA | several 10 m |  |
|  | Outdoor husbandry and logistics | Typically, > tens of seconds | 99% | NA | <500 bit/s | Typically, [< 100 bytes] | <5200 devices / km² | [300 m - 500 m] | 430000 m² | Up to 3 km/h | 15 min | NA | NA | NA |  |
|  | Smart city | 10 s - 30 s | 99% | NA | <1 kbit/s | Typically, [< 100 bytes] | <1000 devices / km² | [300 m - 700 m] | City wide including rural areas | Stationary | 15 min | NA | NA | NA |  |
| NOTE 1: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.4	Performance service requirements for tracking

Table 6.4-1 KPIs for tracking

| Deployment | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | Indoor tracking | 1 s | 99.9% | NA | <1 kbit/s | <1 k bits | 25/100 m² | 10 m indoor | 200 m² | Static | 1 per hour | 1 s | 90% | 1-3 m, 90% availability |  |
| Outdoor | Outdoor tracking | 1 s | 99.9% | NA | <1 kbit/s | <1 kbits | <10 per 100 m² | 500 m | Up to the whole PLMN | up to [10 km/h] | 1 per hour | 1 s | 99% | several 10 m |  |
| NOTE 1: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.5	Performance service requirements for actuator control

Table 6.5-1 KPIs for actuator control

| Deployment | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | Indoor actuator control | Several seconds | 99% | NA | 2 kbit/s | <100 Bytes | less than 1.5 million/km² | 50- m indoors | <250 m² for home, and 15,800 square meters for supermarket | stationary | 20 min to 2 h | NA | NA | 3 m to 5 m indoor |  |
| Outdoor | Outdoor actuator control for large coverage | hundreds ms level | 99% | N/A | NA | 128 bit (DL) | NA | [500] m outdoors | NA | Static | NA | NA | NA | NA |  |
|  | Outdoor actuator control for medium coverage | Several seconds | 99% | NA | <2 kbit/s | <200 bits | <200 per 100 m² | 200 m | City wide including rural areas | Static | NA | NA | NA | 3 m to 5 m indoor |  |
| NOTE 1: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Annex <A> (normative):
<Normative annex for a Technical Specification>

Start each annex on a new page.

Annexes are labelled A, B, C, etc. and designated either "normative" or "informative" depending on their content.

Normative annexes only to appear in Technical Specifications. Use style "Heading 8".

Annex <B> (informative):
<Informative annex for a Technical Specification>

Informative annexes may appear in both Technical Specifications and Technical Reports. Use style "Heading 8" for use in TSs.

Informative annexes shall not contain requirements for the implementation of the Technical Specification.

B.1	Heading levels in an annex

Heading levels within an annex are used as in the main document, but for Heading level selection, the "A.", "B.", etc. are ignored. e.g. B.1.2 is formatted using Heading 2 style.

Annex <B>:
<Informative annex title for a Technical Report>

Informative annexes in Technical Reports do not use "(informative") in the title, since all annexes in TRs are informative. Use style "Heading 9" in TRs.

Annex <C> (informative):
Bibliography

Use style "Heading 8" in TSs and "Heading 9" in TRs. Do not use "informative" in the title in TRs.

The Bibliography is optional. If it exists, it shall follow the last technical annex in the document.

The following material, though not specifically referenced in the body of the present document (or not publicly available), gives supporting information.

Bibliography format

<Publication>: "<Title>".

Annex <D> (informative):
Index

Use style "Heading 8" in TSs and "Heading 9" in TRs. Do not use "informative" in the title in TRs.

The Index is optional. If it exists, it shall immediately precede the Changes history annex.

Generate the index using MS Word's index field feature.

Annex <X> (informative):
Change history

Use style "Heading 8" in TSs and "Heading 9" in TRs. Do not use "informative" in the title in TRs.

This is the last annex for TS/TSs which details the change history using the following table.
This table is to be used for recording progress during the WG drafting process till TSG approval of this TS/TR.
For TRs under change control, use one line per approved Change Request
Date: use format YYYY-MM
CR: four digits, leading zeros as necessary
Rev: blank, or number (max two digits)
Cat: use one of the letters A, B, C, D, F
Subject/Comment: for TSs under change control, include full text of the subject field of the Change Request cover
New vers: use format [n]n.[n]n.[n]n

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |