3GPP TSG SA WG 1 Meeting #104 	S1-233048

Chicago, USA, 13 - 17 November 2023	(revision of S1-23xxxx)

Source:	Nokia, Nokia Shanghai Bell

pCR Title:	Pseudo-CR on finalising KPI table for sensing

Draft Spec:	3GPP TS 22.137 v1.0.0

Agenda item:	7.1.2

Document for:	Approval

Contact:	hideaki.takahashi@nokia.com

Abstract: This pCR proposes resolving editor’s note on the KPI table and clean-up on sensing service descriptions.

1. Introduction

At SA1 #103, there was no conclusion whether the sensing service categories 5 and 8 are part of Rel-19 standards or not. Thus, editor’s note was added as a place holder to be discussed at the next meeting. The current KPI table in version 1.0.0 of TS 22.137 was based on the conclusion that the service category 5 is targeted for ADAS and category 8 is for human hand gesture recognition.

2. Reason for Change

Editor’s note is proposed to remove since it would be an open-ended argument in SA1 whether it is feasible to support or not. Instead, it is sensible to let the downsteam WGs to decide the supported functionalities. A potential compromise is to add a note to state that the target KPI can infer the required radio resources.

The sensing service description of Category 5 needs to be updated for the intneded service, i.e., ADAS. The other miscellaneous updates are proposed to finalise the contents of the sensing KPI table.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 22.137 v1.0.0.

* * * First Change * * * *

## 6.2	Requirements

The 5G system shall be able to provide sensing results with the performance requirements in Table 6.2-1.

Table 6.2-1:	Performance requirements for 5G Wireless sensing

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 1 | 95 | 10 | 10 | N/A | N/A | 10 | 10 | 1000 | 1 | 5 | 2 | Indoor/outdoor (e.g., detection of human, UAV) |
|  | 2 | 95 | 5 | 1 | 1 | 1 | 1 | 1 | 1000 | 1 | 5 | 5 | Outdoor (e.g., detection of human, UAV) requiring higher performance than category 1 |
|  | 3 | 95 | 1 | N/A | 1 | N/A | 1 | 1 x 1 | 100 (NOTE 2), or 1000 (NOTE 3) | 0.1 | 2 | 2 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV) |
|  | 4 | 99 for public safety, otherwise, 95 | 0.5 | 0.5 | 1.5 for pedestrian, 15 for vehicle, otherwise, 0.1 | 1.5 for pedestrian, otherwise, N/A | 0.5 | 5 x 5 for factories 0.5 may be needed | 250 | 0.25 | 1 | 5 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV, AGV, vehicle) requiring higher performance than category 3 |
|  | 5 | 95 | short range radar; 0.02, otherwise; 0.1 | 0.5 | 0.03 | N/A | 0.4 | 0.1 x 0.6 | 50 | 0.05 | 1 | 1 | outdoor (e.g., detection and tracking of human, animal, vehicle for ADAS) requiring higher performance than category 4 |
| Environment monitoring | 6 | 95 | 10 | 0.2 | N/A | N/A | N/A | N/A | 6000 | 60 | 10 | 3 | Nature of environments monitored by sensing. |
| Motion monitoring | 7 | 95 | N/A | N/A | N/A | N/A | N/A | N/A | 60000 | 60 | 5 | 5 | Human motions and activities obtained by sensing (NOTE 4). |
|  | 8 | 95 | 0.2 | 0.2 | 0.1 | 0.1 | 0.375 | 0.3 | 5 | 0.1 | 5 | 5 | Human hand gestures obtained by sensing |
| NOTE 1: The additional information on some of the performance requirements can be found in [2]. NOTE 2: The value 100 ms is sourced from [3] and is valid for sensing at crossroads. NOTE 3: To realize 1m granularity tracking, when the velocity resolution is 1 m/s, the maximum corresponding sensing service latency is 1 s. NOTE 4: Additional KPI on human motion rate accuracy is defined for contactless sleep/sports monitoring [2]. NOTE 5: The sensing resolution (in range and velocity) can provide the information on radio resources required to fulfil the target resolution for each sensing service category. |  |  |  |  |  |  |  |  |  |  |  |  |  |