3GPP TSG SA WG 1 Meeting #104 	S1-233056

Chicago, US, 13 - 17 November 2023	(revision of S1-23XXXX)

Source:	Xiaomi

pCR Title:	Adding a new requirement to section 5.2.1 General

Draft Spec:	3GPP TS 22.137

Agenda item:	7.1.2

Document for:	Approval

Contact:                     Chen Dong chendong7@xiaomi.com

Abstract: This document introduces a new requirement to General subsection TS 22.137

1. Introduction

This pCR introduced a new requirement to section 5.2.1 General into the new TS on Sensing.

2. Reason for Change

It was agreed in SA1#103 to introduce the CPR that subject to operator’s policy, the 5G system may be able to use sensing assistance information to derive the sensing result into the study report TR 22.837. It is proposed to introduce the requirement to TS 22.137 as well.

3. Proposal

It is proposed to agree the following change to 3GPP TS 22.137.

* * * First Change * * * *

5.2.1	General

The 5G system shall be able to provide sensing service to detect, identify and/or track one or more objects (e.g., UAVs, birds) and the environment around the object(s).

Editor’s note: “Identify” is FFS.

Based on operator’s policies, operator’s control and regulation, the 5G system shall be able to collect 3GPP sensing data from sensing receivers for processing.

The 5G system shall be able to provide 5G wireless sensing service in a target sensing service area location using sensing transmitters and sensing receivers.

Subject to regulation and operator policy, the 5G network shall be able to activate, configure, and deactivate 5G wireless sensing based on parameters such as location and network conditions (e.g., network load).

Subject to user consent, regulation, and operator’s policy, the 5G system shall be able to collect non-3GPP sensing data from authorized non-3GPP sensors and securely provide it to 5G network.

Subject to user consent, regulation, and operator’s policy, the 5G system should support the joint processing of the 3GPP sensing data and non-3GPP sensing data to derive a combined sensing result.

The 5G system shall support continuity for 5G wireless sensing service (e.g., for sensing a moving object).

Subject to operator’s policy, the 5G System shall be able to provide the 5G wireless sensing service in case of roaming.

Subject to regulation and operator’s policy, 5G network shall provide prioritization among 5G wireless sensing services as well as prioritizing between communication and sensing services.

The 5G network shall enable UEs without 5G coverage to use unlicensed spectrum to provide 5G wireless sensing service.

Subject to regulation, the 5G network shall enable UEs supporting V2X application to perform 5G Wireless sensing when not served by RAN using the allowed ITS spectrum and unlicensed spectrum.

Subject to operator’s policy, the 5G system may be able to use sensing assistance information to derive the sensing result.

* * * End of Changes * * * *