3GPP TSG SA WG 1 Meeting #104 	S1-233058

Chicago, US, 13 - 17 November 2023	(revision of S1-23XXXX)

Source:	Xiaomi

pCR Title:	Adding a new requirement to section 5.2.3 Network exposure

Draft Spec:	3GPP TS 22.137

Agenda item:	7.1.2

Document for:	Approval

Contact:                     Dong Chen chendong7@xiaomi.com

Abstract: This document introduces a new requirement to Network exposure subsection TS 22.137

1. Introduction

This pCR introduced a new requirement to subsection 5.2.3 network exposure into the new TS on Sensing.

2. Reason for Change

Based on the discussion and output in the study report TR 22.837, two new requirements for 5G network have been introduced for Network exposure subsection of TS 22.137. Here we capture these two new requirements in the network exposure subsection of TS 22.137.

3. Proposal

It is proposed to agree the following change to 3GPP TS 22.137.

* * * First Change * * * *

5.2.3 Network exposure

Subject to operator’s policy, the 5G network shall be able to provide secure means to report sensing result to a trusted third-party requesting information about a target object when specific requested conditions are met.

NOTE: These conditions could be e.g., the target object distance from the restricted area border or entering restricted area.

Subject to operator’s policy, the 5G network shall provide secure means for a trusted third-party to request 5G wireless sensing service based on specific parameters (e.g., refresh rate, period of time, sensing KPIs, geographical location) and to receive the corresponding sensing results.

Subject to operator’s policy and regulation, the 5G system shall be able to provide secure means for a trusted third-party to receive sensing results with contextual information.

Subject to user’s consent, regulation and operator’s policy, the 5G network may provide secure means to expose to a trusted third-party the combined sensing result derived from the joint processing of the 3GPP sensing data and non-3GPP sensing data.

Subject to operator’s policy, the 5G network may provide secure means for the operator to expose information towards trusted third-party on whether a given sensing service is available and the estimated quality of the given service for a certain geographic area and time.

Subject to operator’s policy, the 5G network shall be able to provide secure means to enable trusted third-party to request discovering a sensing group in the proximity of the UE that is requesting a 5G wireless sensing service from application server.

Subject to operator’s policy, the 5G network may enable secure means for a trusted third party to provide sensing assistance information.

* * * End of Changes * * * *