3GPP TSG-SA WG1 Meeting #104 	S1-23xxxx

(Revision of S1-232651)

Chicago, USA, 13 - 17 November 2023

Source:	OPPO

pCR Title:	Input to new TS - Ambient IoT performance requirements

Draft Spec:	3GPP TS 22.XXX

Agenda item:	7.2.2

Document for:	Approval

Contact:	xuweijie@oppo.com, Yang Xu (xuyang@oppo.com)

1. Introduction

This pCR introduces descriptions and requirements of ambient IoT performance in section 6. The agreed KPI value updation in SA1#103 is captured in KPI consolidation.

2. Reason for Change

Overall descriptions and performance requirements need to be described.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 22.XXX.

****************Start of Change *************

6	Performance service requirements of Ambient IoT

6.1	General

Ambient IoT service can be categorized into 4 categories, namely inventory, sensor data collection, tracking and actuator control. The corresponding performance services requirements are listed in the following subclauses.

6.2	Performance service requirements for inventory

Table 6.2-1 KPIs for inventory

| Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inventory or asset management | Typically, seconds level | 99% | NA | <2 kbit/s | 96/256 bits | <1,5 Million/km² | 30~50 m indoor, 200 m~400 m outdoor | 1-10 km² | 3~10 km/h | NA | NA | NA | 3 m indoor, cell-level outdoor |  |
| NOTE 1: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.3	Performance service requirements for sensor data collection

Table 6.3-1 KPIs for sensor data collection

| Deployment | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | Room envirionment monitoring (e.g. domicile, machinery) | 20-30 s | 99 % | NA | <1 kbit/s | < 100 bits | 1.5/m2 | 10-30 m | NA | Stationary | NA | NA | NA | NA |  |
|  | Indoor agriculture and husbandry | >10 s | 99.9% | NA | <1 kbit/s | Typically, < 1000 bits | 1 per m² | 30 - 200 m | 6000 m²~30,000 m² | Quasi-stationary | 15 min to half an hour | NA | NA | NA |  |
| Outdoor | Smart grid | 1 s | 99% | NA | < 1 kbit/s | Typically, < 100 bytes | < 10,000 /km² | Typically, 50 m-200 m | [several km² up to 100 000 km²] | Stationary | 5-15 min | NA | NA | several 10 m |  |
|  | Outdoor husbandry and logistics | Typically, > tens of seconds | 99% | NA | <500 bit/s | Typically, [< 100 bytes] | <5200 devices / km² | [300 m - 500 m] | 430000 m² | Up to 3 km/h | 15 min | NA | NA | NA |  |
|  | Smart city | 10 s - 30 s | 99% | NA | <1 kbit/s | Typically, < 100 bytes | <1000 devices / km² | 300 m - 500 m | City wide including rural areas | Stationary | 15 min | NA | NA | NA |  |
| NOTE 1: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.4	Performance service requirements for tracking

Table 6.4-1 KPIs for tracking

| Deployment | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | Indoor tracking | 1 s | 99.9% | NA | <1 kbit/s | <1 k bits | 25/100 m² | 10 m indoor | 200 m² | up to 3km/h | 1 per hour | 1 s | 90% | 1-3 m, 90% availability |  |
| Outdoor | Outdoor tracking | 1 s | 99.9% | NA | <1 kbit/s | <1 kbits | <10 per 100 m² | 500 m | Up to the whole PLMN | up to 10 km/h | 1 per hour | 1 s | 95% | several 10 m |  |
| NOTE 1: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.5	Performance service requirements for actuator control

Table 6.5-1 KPIs for actuator control

| Deployment | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | Indoor actuator control | Several seconds | 99% | NA | 2 kbit/s | <100 Bytes | less than 1.5 million/km² | 50 m indoors | <250 m² for home, and 15,800 square meters for supermarket | stationary | 20 min to 2 h | NA | NA | 3 m to 5 m indoor |  |
| Outdoor | Outdoor actuator control for large coverage | Several seconds | 99% | N/A | NA | 128 bit (DL) | NA | 500 m outdoors | 40,000~4,000,000m2 | Static | NA | NA | NA | NA |  |
|  | Outdoor actuator control for medium coverage | Several seconds | 99% | NA | <2 kbit/s | <200 bits | <20 per 100 m² | 200 m | City wide including rural areas | Static | NA | NA | NA | NA |  |
| NOTE 1: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

*******************End of Change*****************