3GPP TSG SA WG 1 Meeting #104 	S1-233086

Chicago, USA, 13 - 17 November 2023	(revision of S1-23xxxx)

Source:	Deutsche Telekom AG, vivo, Nokia

pCR Title:	Pseudo-CR on Correcting references in the KPI table

Draft Spec:	3GPP TS 22.137 v 1.0.0

Agenda item:	7.1.2

Document for:	Approval

Contact:	Vasil Aleksiev, vasil dot aleksiev at magenta dot at

Abstract: This pCR proposes changes in the existing notes of KPI table in section 6.2

1. Introduction

Existing KPI table notes refer directly to the TR 22.837. It is important to introduce the respective references from the study into normative work.

2. Reason for Change

SA1 study on Sensing is already finished and it is not expected to be further updated. Refering it in normative work is not a good practice and there is expectance the KPI references to be seen directly in the TS 22.137.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 22.137 v 1.0.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	5GAA_White_Paper_C-V2X Use Cases Volume II: Examples and Service Level Requirements.

[3]	Moore, Erik George, "Radar Detection, Tracking and Identification for UAV Sense and Avoid Applications" (2019). Electronic Theses and Dissertations. https://digitalcommons.du.edu/etd/1544/

[4]	Roberto Opromolla, etc., “Perspectives and Sensing Concepts for Small UAS Sense and Avoid”, 2018 IEEE/AIAA 37th Digital Avionics Systems Conference (DASC). https://www.mdpi.com/2072-4292/13/13/2523

* * * Next Change * * * *

## 6.2	Requirements

The 5G system shall be able to provide sensing results with the performance requirements in Table 6.2-1.

Table 6.2-1:	Performance requirements for 5G Wireless sensing

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 1 | 95 | 10 | 10 | N/A | N/A | 10 [3] | 10 [3] | 1000 | 1 | 5 | 2 | Indoor/outdoor (e.g., detection of human, UAV) |
|  | 2 | 95 | 5 | 1 | 1 | 1 | 1 | 1 | 1000 | 1 | 5 | 5 | Outdoor (e.g., detection of human, UAV) requiring higher performance than category 1 |
|  | 3 | 95 | 1 | N/A | 1 [3], [4] | N/A | 1 [3], [4] | 1 x 1 [3] | 100 [2], or 1000 (NOTE 3) | 0.1 | 2 | 2 | Indoor/outdoor (e.g., detection tracking of human, animal, UAV) |
|  | 4 | 99 for public safety, otherwise, 95 | 0.5 | 0.5 | 1.5 for pedestrian, 15 for vehicle, otherwise, 0.1 | 1.5 for pedestrian, otherwise, N/A | 0.5 | 5 x 5 for factories 0.5 may be needed | 250 | 0.25 | 1 | 5 | Indoor/outdoor (e.g., detection tracking of human, animal, UAV, AGV, vehicle) requiring higher performance than category 3 |
|  | 5 | 95 | short range radar; 0.02, otherwise; 0.1 | 0.5 | 0.03 | N/A | 0.4 | 0.1 x 0.6 | 50 | 0.05 | 1 | 1 | Indoor/outdoor (e.g., detection tracking of human, animal, UAV, AGV, vehicle) requiring higher performance than category 4 |
| Environment monitoring | 6 | 95 | 10 | 0.2 (NOTE 4) | N/A | N/A | N/A | N/A | 6000 | 60 | 10 | 3 | Nature of environments monitored by sensing (e.g. rainfall, flooding monitoring). |
| Motion monitoring | 7 | 95 | N/A | N/A | N/A | N/A | N/A | N/A | 60000 | 60 | 5 | 5 | Human motions and activities obtained by sensing (NOTE 5). |
|  | 8 | 95 | 0.2 | 0.2 | 0.1 | 0.1 | 0.375 | 0.3 | 5 | 0.1 | 5 | 5 | Human hand gestures obtained by sensing |
| NOTE 1: For sensing service categories to which UAV, human or vehicle is a sensing target, the typical size (Length x Width x Height) of UAV is 1.6m x 1.5m x 0.7m, the typical size of human is 0.5m x 0.5m x 1.75m, and the typical size of vehicle is 7.5m x 2.5m x 3.5 m. NOTE 2: The safe distance between pedestrian/vehicle and power transmission station/line is 0.7m/0.95m. NOTE 3: To realize 1m granularity tracking, when the velocity resolution is 1 m/s, the maximum corresponding sensing service latency is 1 s. NOTE 4: This value is derived from the water level where people feel difficulty in walking. NOTE 5: To achieve human motion monitoring, different accuracy KPI is needed to measure different human motions. E.g., respiration rate accuracy (2 times/min) is a KPI used to measure the accuracy of sleep monitoring, sit-up rate accuracy (3 times/min) is a KPI used to measure the accuracy of sports monitoring. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Editor’s note: TBD if sensing service categories 5 and 8 are included in Rel-19.

* * * Next Change * * * *

<Proposed change in revision marks>