3GPP TSG SA WG 1 Meeting #104 	S1-233093

Chicago, IL, USA, 13 - 17 November 2023	(revision of S1-23xxxx)

Source:	LG Electronics

pCR Title:	Proposed text for Deployment scenarios on FS_SOBOT

Draft Spec:	3GPP TR 22.916

Agenda item:	7.10

Document for:	Approval

Contact:	Ki-Dong Lee (kidong.lee <at> zenith (dot) com)

1. Introduction

This contribution provides text for Deployment scenarios in Section 6 of TR 22.916.

2. Reason for Change

The TR needs some text relevant for Deployment scenarios.

3. Conclusions

See below.

4. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.916v0.5.0.

* * * First Change * * * *

# 6	Other considerations

## 6.x	High-level deployment scenarios

Robotic Application Enablement Communication (in short Robotic Communication) is a technology that enables a group of robots (or a robot) to communicate with various entities, including other robots (or robotic applications), humans ([indoor or outdoor] x [structured environment or unstructured environment]; e.g., residents, workers, pedestrians), infrastructure, and networks. Here are several deployment scenarios for Robotic Communication.

NOTE 1: A robot can be an ordinary UE, a UE acting as a relay for other UE (e.g., UE Relay), and a gNB (e.g., IAB-Node) that is fixed or mobile depending on scenarios.

NOTE 2: In the following scenarios, the communication direction is assumed to be unidirectional (either  or ) or bidirectional (both  and ).

NOTE 3: In the following scenarios, the communication path is assumed to be a single hop or multi-hop. In a multi-hop scenario, a combination of different scenarios are possible when applicable, e.g., R2R and R2C.

NOTE 4: In some of the following scenarios, the counterpart of a robot (or a group of robots) in communication path can be an IoT device, an ordinary UE, a UE acting as a relay, and so on.

1.	Robot-to-Robot (R2R) Communication:

- Collision Avoidance: Robots can exchange information about their speed, direction, and position to avoid collisions, especially in intersections or blind spots.

- Traffic Congestion Management: R2R communication allows robots to share real-time traffic information, helping them choose less congested routes. Both road traffic (e.g., in public roads) and robot traffic are considered.

- Examples:

- A group of robots for public safety purposes.

- Indoor and outdoor delivery robots.

- Internet of aerial robots (or Internet of drones).

- A group of robots using machine-type media communications.

- Robots maneuvering in unstructured environments or in structured environments such as public roads, sidewalks.

- In the United States, currently, robot operations lack national regulation and are governed by individual states. Virginia set a precedent in 2017 by regulating robot operation, allowing them to move on sidewalks and crosswalks at speeds not exceeding 10 mph (ca. 16km/h). If these paths are unavailable, robots are permitted to operate on the roadside with a speed limit of 25 mph (ca. 40km/h). In 2020, Pennsylvania enacted regulations allowing robots weighing up to 550 pounds (ca. 250kg) without payload [X1-NGA].

2.	Robot-to-Infrastructure (R2I) Communication:

- Traffic Signal Coordination: Robots can receive data from traffic signals to optimize their speed and reduce unnecessary stops, improving traffic flow.

- Roadside Assistance: Remote drivers can receive alerts and assistance information, such as nearby charging stations or emergency services, from roadside infrastructure.

NOTE 5: For example, in some States of the United States where robots are legally considered as road vehicles, such robots are assumed to be capable of supporting communication features relevant to Cellular V2X (e.g., LTE-V2X, NR-based V2X).

- Examples:

- Indoor and outdoor robots.

- Internet of aerial robots (or Internet of drones).

- Car valet robot at parking lots

- A group of robots using machine-type media communications.

- A group of aerial roots (e.g., UAVs) performing geo-surface sensing and/or environmental monitoring.

3.	Robot-to-Pedestrian (R2P) Communication:

- Pedestrian/Human Safety: Robots can detect pedestrians/human equipped with or without communication devices, issuing warnings to both robot operators and pedestrians/human to prevent accidents or collision.

- Crosswalk and human-zone Safety: Pedestrians/humans can receive alerts on their devices (UE’s, tethered/untethered devices) about approaching robots, ensuring timely safety-related alert.

- Examples:

- Surveillance robot (e.g., in CCRC)

- Disinfection robot (e.g., in hospitals, hotels)

- Outdoor delivery robots allowed to use public roads some of which are shared by humans.

- Personalized assistive robots: personalized digital experience (e.g., VR/XR/MR assisted shopping, gaming). Both physical robots and virtual bots are considered where virtual bots are assumed to be linked to authorized multi-sensory input (e.g., acoustic signals, voice, audio, or visual input such as facial expressions, gesture, and/or gait) and display mechanisms that are security and privacy ensured.

4.	Robot-to-Network (R2N) Communication:

- Robot/road Traffic Management: Traffic authorities can collect real-time data from robots to monitor traffic patterns, analyze congestion, and adjust traffic signals for optimal flow. See NOTE 5.

- Emergency Response: Robots can transmit information about accidents or road hazards directly to emergency services, enabling faster response times.

- Timely Response: Robots that are in inter-continental real-time trading environments (e.g., trans-Atlantic financial market), realistic mixed-reality gaming and entertainment, and so on.

NOTE 6: the term `Network’ can include 3GPP NTN and TN.

5.	Robot-to-Cloud (R2C) Communication:

- Fleet Management: Fleet operators can track robots, monitor their health, and optimize routes, leading to efficient operations and reduced maintenance expenses.

- Over-the-Air (OTA) Updates: Robot manufacturers can remotely deploy software updates and patches to improve robot performance and security.

- Examples:

- A group of robots in healthcare, delivery, and/or manufacturing environments.

- Tele-operated robots with haptic feedback/control.

6.	Robot-to-Grid (R2G) Communication:

- Smart Charging: Electric robots can communicate with the grid to schedule charging during off-peak hours, optimizing energy usage and reducing costs for both consumers and utilities.

- Grid Stability: Electric robots can provide feedback to the grid about available battery capacity, enabling the grid to balance demand and supply effectively.

- Examples:

- A group of robots in smart city settings, in hazardous control environments.

7.	Robot-to-Home (R2H) Communication:

- Home Automation: Robots can communicate with smart home devices, allowing users to control home appliances, lighting, and security systems remotely from their robots.

- Energy Management: Electric robots can supply power back to homes during peak demand periods, reducing the load on the grid and lowering household electricity expenses.

- Examples:

- A group of robots in smart home that can provide timely and accurate information for the smart home server for high-quality decision-making.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains; Stage 1".

[3] 	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[4] 	3GPP TS 22.263, "Service requirements for video, imaging and audio for professional applications (VIAPA); Stage 1".

[5] 	Next G Alliance Report: 6G Applications and Use Cases, May 2022; https://www.nextgalliance.org/wp-content/uploads/dlm_uploads/2022/07/NGA-Perspective-Brochure-V6.pdf

[6] 	K.-D. Lee, "A Smart Network of Service Robots: Technical Challenges and Design Considerations," IEEE Communications Magazine, pp. 28-34, August 2021.

[7] 	K.-D. Lee and C. Gray-Preston, "Everyday Living Assisted by 6G Applications and Solutions," IEEE Wireless Communications Magazine, October 2022.

[8]	W. Zhuang, Y. Shen, L. Li, C. Gao and D. Dai. “Develop an Adaptive Real-Time Indoor Intrusion Detection System Based on Empirical Analysis of OFDM Subcarriers.” Sensors (Basel, Switzerland). 2021 Mar;21(7):2287

[9] 	1872-2015 IEEE Standard for Ontologies for Robotics and Automation

[10] 	2755-2017 IEEE Guide for Terms and Concepts in Intelligent Process Automation

[11]	http://dictionary.ieee.org

[12]	NIST https://www.nist.gov/system/files/documents/el/isd/ks/ALFUS-BG.pdf

[13] 	SAE AS4D Committee

[14] 	NIST, Autonomy Levels for Unmanned Systems (ALFUS) Framework; https://tsapps.nist.gov/publication/get_pdf.cfm?pub_id=823618

[15] 	Multimodal Fusion for Objective Assessment of Cognitive Workload: A Review, September 2019 IEEE Transactions on Cybernetics PP(99):1-14, DOI:10.1109/TCYB.2019.2939399

[16]	Data Fusion and IoT for Smart Ubiquitous Environments: A Survey, April 2017IEEE Access PP(99):1-1, DOI:10.1109/ACCESS.2017.2697839

[17]	MPEG Use cases and requirements for Video Coding for Machines; https://www.mpeg.org/wp-content/uploads/mpeg_meetings/138_OnLine/w21545.zip

[18]	3GPP TR 22.886: "Study on enhancement of 3GPP support for 5G V2X services”.

[19]	3GPP TR 26.926: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems

[20]	A.-L. Kim, et al., "Development of change detection algorithm using high resolution SAR complex image," in Proc. 2015 IEEE Asia-Pacific Conference on Synthetic Aperture Radar (APSAR), Online: https://ieeexplore.ieee.org/abstract/document/7306330

[21]	G.W. Morgenthaler, et al, "Feasibility of Using a Miniature-UAV Remote-Sensing/Precision-Agriculture System (MINI-UAV RSPA SPACE SYSTEM) to Increase Crop Yields, Lower Costs, Save Water, and Reduce Pollution of Air, Soil, and Water," Aerospace Research Central Nov. 2012. Online: https://arc.aiaa.org/doi/abs/10.2514/6.IAC-06-D3.2.09

[23]	Juliet.J. Lee, "Remote Sensing and Artificial Intelligence for Wildlife Conservation – A Survey," WPAC, 2023. Online: https://wvwpac.wixsite.com/westview/post/remote-sensing-and-artificial-intelligence-for-wildlife-preservation-a-survey

[24] 		E. Felemban, et al., "Underwater Sensor Network Applications: A Comprehensive Survey," International Journal of Distributed Sensor NetworksVolume 11, Issue 11, November 2015. Online: https://journals.sagepub.com/doi/epub/10.1155/2015/896832

[25] 	Ki-Dong Lee, “An efficient real-time method for improving intrinsic delay of capacity allocation in interactive GEO Satellite networks,” IEEE Transactions on Vehicular Technology, vol 53, no. 2, pp. 538 – 546, March 2004.  Online: https://ieeexplore.ieee.org/document/1275718

[26] 	ITU-R Future technology trends towards 2030, Available online: https://www.itu.int/en/ITU-T/Workshops-and-Seminars/2023/0724/Pages/default.aspx

[27]	Statista, https://www.statista.com/topics/1143/mining/

[28]	3GPP TS 22.847: "Study on supporting tactile and multi-modality communication services; stage 1 (Release 18)", 2022-03.

[X1-NGA]	Network-Enabled Robotic and Autonomous Systems, NextG Alliance, ATIS. May 2023. Available online: https://www.nextgalliance.org/white_papers/network-enabled-robotic-autonomous-systems/

* * * END * * * *