3GPP TSG SA WG 1 Meeting #104 	S1-233094

Chicago, IL, USA, 13 - 17 November 2023	(revision of S1-23xxxx)

Source:	LG Electronics

pCR Title:	Proposed text for Related features and aspects from existing studies/works

Draft Spec:	3GPP TR 22.916

Agenda item:	7.10

Document for:	Approval

Contact:	Ki-Dong Lee (kidong.lee <at> zenith (dot) com)

1. Introduction

This contribution provides text for Related features and aspects from existing studies/works in Section 6 of TR 22.916.

2. Reason for Change

The TR needs some text on Related features and aspects from existing studies/works as planned in the SID.

3. Conclusions

See below.

4. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.916v0.5.0.

* * * First Change * * * *

# 6	Other considerations

## 6.x	ISAC aspects related to robot applications

### 6.x.1	General description

Autonomous Mobile Robots (AMRs) are revolutionizing logistics operations in industries such as manufacturing and warehousing. Unlike traditional Automated Guided Vehicle (AGV) systems, AMRs possess the intelligence to navigate and perform tasks autonomously, offering unparalleled flexibility. However, a significant challenge for AMRs lies in acquiring accurate and continuous sensing data as they move through complex environments [X1-NGA, X2_22837]. Factors like unexpected obstacles or sudden appearances of people and machines can compromise their safety.

To address this challenge, 5G base stations are strategically deployed in factories. These stations serve a dual purpose: not only do they provide essential communication capabilities for factory equipment, but they also act as sensing nodes. By transmitting sensing signals and receiving reflected signals, these base stations capture real-time 3GPP sensing data. This data is then processed and analyzed by the core network, generating valuable insights into the AMRs' surroundings.

Crucially, these sensing results can be shared with trusted third-party platforms, empowering AMRs with a comprehensive understanding of their environment. This enhanced sensing capability allows AMRs to adapt their routes dynamically, ensuring both efficiency and safety. Moreover, in scenarios where obstacles obstruct radio signals or AMR paths traverse indoor and outdoor spaces, collaboration between multiple base stations further improves sensing accuracy and service continuity.

The TR [X2-22837] presents a few use cases addressing sensing issues related to robot operation. Examples include the following with some KPIs as shown in, e.g., Table 5.23.6-1 and Table 5.32.6-1:

[PR 5.23.6-1] The 5G system shall be able to provide the continuity of sensing service for a specific target object, across indoor and outdoor.

[PR 5.23.6-2] The 5G system shall be able to provide a secure mechanism to ensure sensing result data privacy within the sensing service area.

[PR 5.32.6-1] Based on operator’s policy, the 5G system may provide a mechanism for a trusted third party to provide sensing assistance information about a sensing target.

### 6.x.2	Challenges and potential gaps

While the PRs referenced above have addressed potential new requirements, such as on the continuity of sensing services both in indoor and outdoor settings, data privacy and network exposure, it is still worthwhile to consider the support of scalable and efficient use of communication resources needed for stable operation of multiple service robots especially when a large number of service robots are present.

The following is a high-level recommendation for future enhancement on integrated sensing and communication capability.

[CPG-6.x.2-001] 5G system is expected to be evolved so that it can support some or all of the referenced sensing-realted PRs and KPIs even when a large number of service robots are present.

NOTE: The above high-level recommendation is intended to ensure a group of robots to receive communication services with a minimum level of disruption as required by or appropriate for robot applications in order to maintain the quality of robot services on given tasks.

## 6.y	Metaverse aspects related to robot applications

### 6.y.1	General description

The Technical Report on Localized Mobile Metaverse Services [X2-22856] addressed several use cases related to robot applications. The following includes some examples.

1)  Metaverse use case on Spatial Mapping and Localization (Clause 5.5, [X2-22856])

In the context of robot operations, spatial mapping involves constructing or updating a map of an unknown location, while localization tracks an object to identify its location and orientation over time. For the localized mobile Metaverse use case (e.g., clause 5.5, [X2-22856]), communication technology support is vital. Spatial Mapping Service creates and maintains a 3D map of indoor or outdoor environments, enabling the identification of stationary and moving objects. This spatial map is utilized by Spatial Localization Service, allowing for accurate positioning and orientation of users.

Spatial mapping gathers sensing data to create a detailed 3D map, integrating information from various sources like sensors and architectural specifications. Localization, based on this spatial map, identifies users' positions and viewing angles in 3D space. Examples of spatial mapping applications include government projects mapping entire cities, navigation service providers mapping roads, and customers mapping indoor spaces. To achieve this, vehicles or robots equipped with multiple cameras and LiDAR devices capture images and depth information. This data is then processed, enhancing location accuracy and enabling various applications like visual positioning and Metaverse content management. The information is communicated through uplink sensor data, enabling precise localization and enriching the spatial internet experience.

The following includes some example of potential new requirements:

[PR 5.5.6.1-1] Subject to operator policy and relevant regional and national regulation, the 5G system shall support mechanisms for an authorized UE to provide sensing data that can be used to produce or modify a spatial map.

[PR 5.5.6.1-2]  Subject to operator policy, user consent and relevant regional and national regulation, the 5G system shall support mechanisms to receive and process sensing data to produce or modify a spatial map.

[PR 5.5.6.1-3]  Subject to operator policy and relevant regional and national regulation, the 5G system shall support mechanisms to expose a spatial map or derived localization information from that map to authorized third parties.

NOTE 1: Some KPI’s required to fulfil e.g., [PR 5.5.6.1-2] is not specified in this particular use case.

2) Metaverse use case on Immersive tele-operated driving (Clause 5.20, [X2-22856])

The use case involves operating vehicles, lifting devices, or machines/robots in hazardous industrial environments, where manual operation poses risks due to exposure to dangerous materials, extreme conditions, or radioactivity. While Automated Guided Vehicles (AGVs) exist, the proposal suggests leveraging 5G technology to create a system allowing remote users to control these devices. This control occurs through an immersive cockpit displayed on a virtual reality head-mounted display and haptic gloves. The cockpit integrates data from the digital twin of the operating environment, including information from factory sensors and the surroundings. This approach enhances user safety and operational accuracy by merging data from the digital twin, enabling remote control in hazardous settings.

This use case includes some requirements that are needed to support immersive teleoperations (or digital twins) within a service area up to 10km (radius).

[PR 5.20.6-1] The 5G system shall be able to provide a means to associate data flows related to one or multiple UEs with a single digital twin maintained by the mobile metaverse service.

[PR 5.20.6-2] The 5G system shall be able to provide a means to support data flows from one or multiple UEs to update a digital twin maintained by the mobile metaverse service.

### 6.y.2	Challenges and potential gaps

None.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains; Stage 1".

[3] 	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[4] 	3GPP TS 22.263, "Service requirements for video, imaging and audio for professional applications (VIAPA); Stage 1".

[5] 	Next G Alliance Report: 6G Applications and Use Cases, May 2022; https://www.nextgalliance.org/wp-content/uploads/dlm_uploads/2022/07/NGA-Perspective-Brochure-V6.pdf

[6] 	K.-D. Lee, "A Smart Network of Service Robots: Technical Challenges and Design Considerations," IEEE Communications Magazine, pp. 28-34, August 2021.

[7] 	K.-D. Lee and C. Gray-Preston, "Everyday Living Assisted by 6G Applications and Solutions," IEEE Wireless Communications Magazine, October 2022.

[8]	W. Zhuang, Y. Shen, L. Li, C. Gao and D. Dai. “Develop an Adaptive Real-Time Indoor Intrusion Detection System Based on Empirical Analysis of OFDM Subcarriers.” Sensors (Basel, Switzerland). 2021 Mar;21(7):2287

[9] 	1872-2015 IEEE Standard for Ontologies for Robotics and Automation

[10] 	2755-2017 IEEE Guide for Terms and Concepts in Intelligent Process Automation

[11]	http://dictionary.ieee.org

[12]	NIST https://www.nist.gov/system/files/documents/el/isd/ks/ALFUS-BG.pdf

[13] 	SAE AS4D Committee

[14] 	NIST, Autonomy Levels for Unmanned Systems (ALFUS) Framework; https://tsapps.nist.gov/publication/get_pdf.cfm?pub_id=823618

[15] 	Multimodal Fusion for Objective Assessment of Cognitive Workload: A Review, September 2019 IEEE Transactions on Cybernetics PP(99):1-14, DOI:10.1109/TCYB.2019.2939399

[16]	Data Fusion and IoT for Smart Ubiquitous Environments: A Survey, April 2017IEEE Access PP(99):1-1, DOI:10.1109/ACCESS.2017.2697839

[17]	MPEG Use cases and requirements for Video Coding for Machines; https://www.mpeg.org/wp-content/uploads/mpeg_meetings/138_OnLine/w21545.zip

[18]	3GPP TR 22.886: "Study on enhancement of 3GPP support for 5G V2X services”.

[19]	3GPP TR 26.926: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems

[20]	A.-L. Kim, et al., "Development of change detection algorithm using high resolution SAR complex image," in Proc. 2015 IEEE Asia-Pacific Conference on Synthetic Aperture Radar (APSAR), Online: https://ieeexplore.ieee.org/abstract/document/7306330

[21]	G.W. Morgenthaler, et al, "Feasibility of Using a Miniature-UAV Remote-Sensing/Precision-Agriculture System (MINI-UAV RSPA SPACE SYSTEM) to Increase Crop Yields, Lower Costs, Save Water, and Reduce Pollution of Air, Soil, and Water," Aerospace Research Central Nov. 2012. Online: https://arc.aiaa.org/doi/abs/10.2514/6.IAC-06-D3.2.09

[23]	Juliet.J. Lee, "Remote Sensing and Artificial Intelligence for Wildlife Conservation – A Survey," WPAC, 2023. Online: https://wvwpac.wixsite.com/westview/post/remote-sensing-and-artificial-intelligence-for-wildlife-preservation-a-survey

[24] 		E. Felemban, et al., "Underwater Sensor Network Applications: A Comprehensive Survey," International Journal of Distributed Sensor NetworksVolume 11, Issue 11, November 2015. Online: https://journals.sagepub.com/doi/epub/10.1155/2015/896832

[25] 	Ki-Dong Lee, “An efficient real-time method for improving intrinsic delay of capacity allocation in interactive GEO Satellite networks,” IEEE Transactions on Vehicular Technology, vol 53, no. 2, pp. 538 – 546, March 2004.  Online: https://ieeexplore.ieee.org/document/1275718

[26] 	ITU-R Future technology trends towards 2030, Available online: https://www.itu.int/en/ITU-T/Workshops-and-Seminars/2023/0724/Pages/default.aspx

[27]	Statista, https://www.statista.com/topics/1143/mining/

[28]	3GPP TS 22.847: "Study on supporting tactile and multi-modality communication services; stage 1 (Release 18)", 2022-03.

[X1-NGA]	Network-Enabled Robotic and Autonomous Systems, NextG Alliance, ATIS. May 2023. Available online: https://www.nextgalliance.org/white_papers/network-enabled-robotic-autonomous-systems/

[X2_22837]	3GPP TR 22.837, Study on Integrated Sensing and Communication – Stage 1.

[X2_22856]	3GPP TR 22.856, Study on Localized Mobile Metaverse Services – Stage 1.

* * * END * * * *