3GPP TSG SA WG 1 Meeting #104 	S1-23xxxx

Chicago, USA, 13 - 17 November 2023	(revision of S1-23xxxx)

Source:	vivo, Lenovo

pCR Title:	Pseudo-CR on Inclusion of Sensing Assistance Information Requirements

Draft Spec:	3GPP TS 22.137

Agenda item:	7.1.1

Document for:	Approval

Contact:	Amy Zhang <amy.zhang@vivo.com>

Abstract: This document is to include the missing requirements on obtaining and utilizing sensing assistance information for a sensing service that are already part of TR22.837 in TS 22.137.

1. Introduction

This pCR is to include sensing assistance information requirements.

2. Reason for Change

TR22.837 v19.1.0 agreed requirements on securely receiving and utilizing sensing assistance information provided by the third party for a sensing service, i.e., “CPR 7.1.3-7: Subject to operator’s policy, the 5G network may enable secure means for a trusted third party to provide sensing assistance information” and “ CPR 7.1.1-12	Subject to operator’s policy, the 5G system may be able to use sensing assistance information to derive the sensing result” are missing from TS22.137. Hence, it is proposed to include such requirements in TS22.137 v1.0.0.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 22.137.

* * * First Change * * * *

### 5.2.1	General

The 5G system shall be able to provide sensing service to detect, identify and/or track one or more objects (e.g., UAVs, birds) and the environment around the object(s).

Editor’s note: “Identify” is FFS.

Based on operator’s policies, operator’s control and regulation, the 5G system shall be able to collect 3GPP sensing data from sensing receivers for processing.

The 5G system shall be able to provide 5G wireless sensing service in a target sensing service area location using sensing transmitters and sensing receivers.

Subject to regulation and operator policy, the 5G network shall be able to activate, configure, and deactivate 5G wireless sensing based on parameters such as location and network conditions (e.g., network load).

Subject to user consent, regulation, and operator’s policy, the 5G system shall be able to collect non-3GPP sensing data from authorized non-3GPP sensors and securely provide it to 5G network.

Subject to user consent, regulation, and operator’s policy, the 5G system should support the joint processing of the 3GPP sensing data and non-3GPP sensing data to derive a combined sensing result.

The 5G system shall support continuity for 5G wireless sensing service (e.g., for sensing a moving object).

Subject to operator’s policy, the 5G System shall be able to provide the 5G wireless sensing service in case of roaming.

Subject to regulation and operator’s policy, 5G network shall provide prioritization among 5G wireless sensing services as well as prioritizing between communication and sensing services.

The 5G network shall enable UEs without 5G coverage to use unlicensed spectrum to provide 5G wireless sensing service.

Subject to regulation, the 5G network shall enable UEs supporting V2X application to perform 5G Wireless sensing when not served by RAN using the allowed ITS spectrum and unlicensed spectrum.

Subject to operator’s policy, the 5G system may be able to use sensing assistance information to derive the sensing result.

* * * Second Change * * * *

### 5.2.3	Network exposure

Subject to operator’s policy, the 5G network shall be able to provide secure means to report sensing result to a trusted third-party requesting information about a target object when specific requested conditions are met.

NOTE: These conditions could be e.g., the target object distance from the restricted area border or entering restricted area.

Subject to operator’s policy, the 5G network shall provide secure means for a trusted third-party to request 5G wireless sensing service based on specific parameters (e.g., refresh rate, period of time, sensing KPIs, geographical location) and to receive the corresponding sensing results.

Subject to operator’s policy and regulation, the 5G system shall be able to provide secure means for a trusted third-party to receive sensing results with contextual information.

Subject to user’s consent, regulation and operator’s policy, the 5G network may provide secure means to expose to a trusted third-party the combined sensing result derived from the joint processing of the 3GPP sensing data and non-3GPP sensing data.

Subject to operator’s policy, the 5G network may provide secure means for the operator to expose information towards trusted third-party on whether a given sensing service is available and the estimated quality of the given service for a certain geographic area and time.

Subject to operator’s policy, the 5G network may enable secure means for a trusted third party to provide sensing assistance information.

* * * End of Change * * * *