3GPP TSG-SA WG1 Meeting #104 	S1-233200

Chicago, USA, 13 - 17 November 2023	(revision of S1-23xxxx)

Source:	Novamint, b-com, EDF

pCR Title:	Pseudo-CR on consolidated requirements for TR 22.848

Draft Spec:	3GPP TR 22.848 version 0.2

Agenda item:	7.11

Document for:	Approval

Contact:	Thierry Bérisot (tberisot@novamint.com)

Abstract: This document includes a text proposal for section Consolidated Requirements of TR 22.848, capturing a set of final consolidated potential requirements

1. Introduction

The “Consolidated Requirements” of TR 22.848 are currently not included in version 0.2.0

2. Reason for Change

adding texts and tables for section Consolidated Requirements of TR 22.848, capturing a set of final consolidated potential requirements

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.848 v 0.2.0.

===============================  All new Text ================================

# 6	Consolidated Requirements

## 6.1	Introduction

The following requirements represent a consolidation of the various potential requirements captured in the above use cases related to the interconnect of SNPN.

## 6.2	Scalable SNPN Interconnect with dynamic connections

The potential requirements corresponding to the support of Scalable SNPN Interconnect with dynamic connections are listed in the table below

Table 6.2-1 – Consolidated Requirements for Scalable SNPN Interconnect with dynamic connections

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

## 6.3	Interconnect between SNPNs

The potential requirements corresponding to the support of interconnect between SNPNs are listed in the table below

Table 6.3-1 – Consolidated Requirements for interconnect between SNPNs

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.3-1 | Subject to SNPNs operators’ agreement, the 5G system shall be able to support mechanisms to enable interconnect between SNPNs | [PR 5.2.6-001] |  |
| CPR 6.3-2 | Subject to SNPNs operators’ agreement, the 5G system shall be able to support mechanisms for a selection of 5GS endpoints for SNPNs’ interconnect. | [PR 5.2.6-002] |  |
| CPR 6.3-3 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support interconnect of standalone non-public networks through links established on a temporary basis forming a group of interconnected SNPNs with a variable topology. | [PR 5.3.6-001] |  |
| CPR 6.3-4 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support routing of user data through interconnected SNPNs. | [PR 5.3.6-002] |  |
| CPR 6.3-5 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support discovery, addition and authentication, or detachment of SNPN in a group of interconnected SNPNs. | [PR 5.3.6-003] |  |
| CPR 6.3-6 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support per UE communication, the establishment, reconfiguration and release of user data transmission path between two SNPNs in a group of interconnected SNPNs. | [PR 5.3.6-004] |  |
| CPR 6.3-7 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support a user subscribed to one Standalone Non-Public Network to consume a service offered by another SNPN in a group of interconnected SNPNs. | [PR 5.3.6-005] |  |
| CPR 6.3-8 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support a mechanism to exploit interconnection links metrics (e.g. jitter, latency, packet loss, capacity) as a criterion to select a suitable route between two distant SNPN in a group of interconnected SNPNs. | [PR 5.3.6-006] |  |
| CPR 6.3-9 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support discovery, addition and authentication, or detachment of SNPN in a group of interconnected SNPNs without interrupting ongoing communications. | [PR 5.4.6-001] |  |
| CPR 6.3-10 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support service continuity of UE communications between interconnected SNPNs with overlapping radio coverage. | [PR 5.4.6-002] |  |
| CPR 6.3-11 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall enable packet loss to be minimized during mobility of a UE between interconnected SNPNs with overlapping radio coverage for some or all connections associated with this UE. | [PR 5.4.6-003] |  |
| CPR 6.3-12 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall minimize communication service interruption time during mobility of a UE between interconnected SNPNs with overlapping radio coverage for some or all connections associated with this UE. | [PR 5.4.6-004] |  |

| CPR 6.3-13 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support prioritization of resources for a service offered by a SNPN that is consumed by users attached to other interconnected SNPNs | [PR 5.5.6-001] |  |
| --- | --- | --- | --- |
| CPR 6.3-14 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall enable a SNPN to monitor the currently delivered end to end QoS for a service hosted by this SNPN and consumed by one or several users attached to other SNPNs inside a group of interconnected SNPNs; | [PR 5.5.6-002] |  |
| CPR 6.3-15 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to provide event notification upon detecting that the requested end to end QoS level cannot be met for a service hosted by this SNPN and consumed by one or several users attached to other SNPNs inside a group of interconnected SNPNs. | [PR 5.5.6-003] |  |
| CPR 6.3-16 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall support a mechanism for a SNPN Interconnect Credential Provider to identify, authenticate and authorise a UE to use a visited interconnect SNPN (VISNPN) through an interconnect from a SNPN the UE is registered to. | [PR 5.6.6-001] |  |
| CPR 6.3-17 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support a user subscribed to one SNPN to consume a service offered by the visited interconnect SNPN (VISNPN) after interconnect between them. NOTE: All types of services (e.g., voice, VoIP, IoT, mission critical communications, IMS, Video Streaming) shall be supported by the interconnect between SNPNs | [PR 5.6.6-002] |  |
| CPR 6.3-18 | Subject to regulatory requirements and based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall support service continuity (with minimum service interruption) of a UE communication when the UE communication path moves between SNPNs during interconnect between 2 SNPNs. | [PR 5.6.6-003] |  |
| CPR 6.3-19 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall support a mechanism for a VISNPN to inform an authorised UE when the interconnect between the SNPNs is about to cease. | [PR 5.6.6-004] |  |
| CPR 6.3-20 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall support a mechanism for a VISNPN to become a temporary HSNPN for an authorised UE even when the interconnect between SNPNs has ceased. | [PR 5.6.6-005] |  |
| CPR 6.3-21 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support mechanisms for a UE to select the visited interconnect SNPN(VISNPN) based on factors such as coverage, capacity, relation with the network and quality of service in case of interconnect with several SNPNs. NOTE: The network selection process during interconnect between SNPNs is expected to be automatic and transparent to the user, without requiring manual intervention. | [PR 5.7.6-001] |  |
| CPR 6.3-22 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support mechanisms to collect information per UE or per application related to the use of the VISNPN during interconnect between SNPNs (e.g., number of UEs, data volume, which SNPN the UE is primary subscriber (HSNPN), which services, duration of the use of the visited network). | [PR 5.7.6-002] |  |
| CPR 6.3-23 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall support a mechanism for a SNPN Interconnect Credential Provider to inform authorised UE about the type of communication services supported by the visited interconnect SNPN (VISNPN) through an interconnect from a SNPN the UE is registered to. | [PR 5.8.6-001] |  |
| CPR 6.3-24 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall support for an authorised UE to perform an automatic discovery to determine the capabilities of the potential visited interconnect SNPNs available and the type of services supported by the SNPNs. | [PR 5.8.6-002] |  |
| CPR 6.3-25 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall support a mechanism for a SNPN Interconnect Credential Provider to authorise a UE to use a visited interconnect SNPN (VISNPN) through an interconnect from a SNPN the UE is registered to, only for a defined duration. | [PR 5.8.6-003] |  |
| CPR 6.3-26 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall support a mechanism for a SNPN Interconnect Credential Provider to allow or restrict the type of communication supported by the visited interconnect SNPN (VISNPN) through an interconnect from a SNPN the UE is registered to. NOTE: The conditions to allow or restrict the type of communication can be on a temporary base (e.g. due to the capacity problem of the SNPN). | [PR 5.8.6-004] |  |
| CPR 6.3-27 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall support a mechanism for a SNPN Interconnect Credential Provider to remotely provision credentials of the visited interconnect SNPN (VISNPN) to an authorised UE through an interconnect from a SNPN the UE is registered to. | [PR 5.8.6-005] |  |