3GPP TSG-SA WG1 Meeting #104	S1-233204

# Chicago, USA, 13 – 17 November 2023									(revision of S1-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.280 | CR | 0166 | rev |  | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of recording and audit requirements for railways |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2023-11- 03 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of metadata requirements in the UIC FRMCS FRS FU-7120, Functional Requirements Specifications (chapter 11.10.2) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Typo: Move the requirement [R-6.15.6.2-004a] in the relevant chapter for MCX Ad hoc Group Emergency Alert - Additional metadata for recording of communications |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Downstream groups will not be able to implement the recording feature, as expected by the railways, mainly for Ad hoc Group Communications due to missing stage 1 requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3, 6.15.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

### 6.7.3	Administrative

[R-6.7.3-001] The MCX Service should provide a mechanism for an MCX Service Administrator to configure whether the presence on the network of a particular MCX User is available.

[R-6.7.3-002] The MCX Service should provide a mechanism for an MCX Service Administrator to configure which MCX Users may determine whether a particular MCX User is present on the network.

[R-6.7.3-003] The MCX Service should provide a mechanism for an MCX Service Administrator to configure whether the ability to participate in Private Communications of a particular MCX User is available.

[R-6.7.3-004] The MCX Service should provide a mechanism for an MCX Service Administrator to configure which MCX Users may determine whether a particular MCX User is capable of participating in a Private Communication.

[R-6.7.3-005] The MCX Service shall provide a mechanism for an MCX Service Administrator to configure which MCX Users, within their authority, are authorized to place a Manual Commencement Private Communication (without Floor control).

[R-6.7.3-006] The MCX Service shall provide a mechanism for an MCX Service Administrator to configure which MCX Users, within their authority, are authorized to place an Automatic Commencement Private Communication (without Floor control).

[R-6.7.3-007] The MCX Service shall provide a mechanism for an MCX Service Administrator to configure for a particular authorized MCX User, a set of MCX Users under the same authority to which a Private Communication (without Floor control) can be made by this particular MCX User.

[R-6.7.3-007a] The MCX Service shall provide a mechanism for an MCX Service Administrator to configure for a particular authorized MCX User, a set of MCX Users under the same authority from which Private Communication (without Floor control) is allowed to this particular MCX User.

[R-6.7.3-008] The MCX Service shall provide a mechanism for an MCX Service Administrator to configure the maximum duration for Private Communication for MCX Users within their authority.

NOTE: 	The maximum duration can be set to infinite.

Next change

### 6.15.4	Recording and audit requirements

[R-6.15.4-001] The MCX Service shall provide a mechanism for a Mission Critical Organization to log the metadata of the MCX Service Group Communications and MCX Service Private Communications under the organization's authority.

[R-6.15.4-002] Metadata shall be logged for both the transmitting Participant and the receiving Participant(s).

[R-6.15.4-003] The MCX Service shall provide a mechanism for a Mission Critical Organization to record the transmissions of the Group Communications and Private Communications under the organization's authority.

[R-6.15.4-004] The MCX Service shall provide a mechanism for a Mission Critical Organization to log at least the following metadata per communication: depending on service this may include; start time, date, MCX User ID, functional alias(es), MCX Group ID, Location information of the transmitting and receiving Participants during the communication, end time or duration, end reason, type of communication (e.g., MCX Service Emergency, Ad hoc group, regroup, private) and success/failure indication, time for a Participant connecting or disconnecting from a communication, time of request to talk, time of revoke to talk, identification of initiating and addressed area of the communication.

[R-6.15.4-005] If an MCX Service Group Communication or MCX Service Private Communication uses end-to-end confidentiality, the MCX Service shall provide a mechanism for a Mission Critical Organization to maintain the end-to-end confidentiality when the MCX Service Group Communication or MCX Service Private Communication is logged.

[R-6.15.4-006] The MCX Service shall provide a mechanism for a Mission Critical Organization to log the metadata of non-communication related user activities under the agency's authority.

[R-6.15.4-007] The MCX Service shall provide a mechanism for a Mission Critical Organization to log at least the following non-communication activity types: MCX Service Emergency Alert, MCX Service Emergency Alert cancellation, In-progress Emergency cancellation, also applicable for MCX Service Ad hoc Group Emergency Alert, registration state change, overridden event, user remote logout, changing another user's affiliations, affiliation change, change of Selected MCX Service Group, (de)activation of functional alias(es), User and Group Regroup and Ad hoc Group operations.

[R-6.15.4-008] The MCX Service shall provide a mechanism for a Mission Critical Organization to log at least the following metadata per non-communication activity: time, date, MCX Service User identity, activity type and pertinent information about the activity (e.g., MCX User/Group ID(s) included in the temporary group created by the User/Group Regroup or Ad hoc Group operation). The following metadata should be logged if applicable to the activity type: MCX Service Group ID, Location information of the MCX User, affiliation list, target MCX Service User ID and success/failure indication.

[R-6.15.4-009] The MCX Service shall provide a mechanism for a Mission Critical Organization to log metadata for all failed authorization attempts (e.g., invalid login password) by an MCX User.

[R-6.15.4-010] The MCX Service shall provide a mechanism to collect metadata for network access events (e.g., pre-emption of the 3GPP system bearer services, loss of signal, failed registration attempts).

[R-6.15.4-011] The MCX Service shall provide recording and audit functions for all types of MCX Service group communication (e.g., normal group, regroup group, Ad hoc group).

End of change