3GPP TSG SA WG 1 Meeting #104	C1-233218

# Chicago, USA, 13– 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0757 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of restriction on access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google Inc., Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18,SMARTER |  |  |  |  |  | Date: |  |  | 2023-10-26 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As discussed in a discussion paper S1-233216, which is related to an incoming LS from CT1 in C1-236567, it is needed to specify a service requirement that a network operator should be able to restrict the usage of satellite access in a network (in the PLMN) when the user is not allowed to use the satellite access by subscription. Also since the 5G system already has a feature called "RAT restriction", which restricts a specific RAT based on a subscription, such a service requirement also needs to be specified. There might be two options, introducing a requirement for generic multiple access technologies, or introducing a requirement limited to satellite access. Since current RAT restriction has existed for generic 5G RATs, we would like to go forward with the former option. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is added that the 5G system shall be able to restrict a specific access technology among multiple access technologies, based on the operator’s and subscription policy. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether a system or an operator is able to restrict a specific RAT among multiple accesses. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

#### 6.3.2.1	General

Based on operator policy, the 5G system shall enable the UE to select, manage, and efficiently provision services over the 3GPP or non-3GPP access.

Based on operator policy, the 5G system shall support steering a UE to select certain 3GPP access network(s).

Based on operator policy, the 5G system shall be able to dynamically offload part of the traffic (e.g. from 3GPP RAT to non-3GPP access technology), taking into account traffic load and traffic type.

Based on operator policy, the 5G system shall be able to provide simultaneous data transmission via different access technologies (e.g. NR, E-UTRA, non-3GPP), to access one or more 3GPP services.

When a UE is using two or more access technologies simultaneously, the 5G system shall be able to optimally distribute user traffic over select between access technologies in use, taking into account e.g. service, traffic characteristics, radio characteristics, and UE's moving speed.

The 5G system shall be able to support data transmissions optimized for different access technologies (e.g. 3GPP, non-3GPP) and accesses to local data networks (e.g. local traffic routing) for UEs that are simultaneously connected to the network via different accesses.

NOTE:	This applies to the scenario with simultaneous 3GPP and non-3GPP accesses.

Based on operator policy, the 5G system shall be able to add or drop the various access connections for a UE during a session.

The 5G system shall be able to support mobility between the supported access networks (e.g. NG-RAN, WLAN, fixed broadband access network, 5G satellite access network).

The 5G system shall support UEs with multiple radio and single radio capabilities.

The 5G system shall support dynamic and static network address allocation of a common network address to the UE over all supported access types.

The 5G system shall support a set of identities for a single user in order to provide a consistent set of policies and a single set of services across 3GPP and non-3GPP access types.

The 5G system shall support the capability to operate in licensed and/or unlicensed bands.

Based on operator policy, the 5G system shall be able to restrict subscribers from using one or more specific access technologies amongst multiple access technologies supported by the 5G network.

***** End change *****