3GPP TSG-SA WG1 Meeting #104 	S1-233281

Chicago, USA, 13 - 17 November 2023	(revision of S1-233198)

Title:	Use case on Interconnect between SNPNs in a port with multiple terminals

Agenda Item:	7.11

Source:	Novamint, b-com, EDF

Contact:	Thierry Bérisot (tberisot@novamint.com)

Abstract: This contribution proposes a new use case as well as potential new requirements to be captured in TR 22.848 v.0.2.0.

---------- Use Case template ----------

## 5.7	Use case on Interconnect between SNPNs in a port with multiple terminals

### 5.7.1	Description

This use case illustrates the interconnect between SNPNs in the context of a port with several SNPNs deployed in the different terminals managed by different shipping and liner operation companies and a port authority umbrella SNPN. Similar situation can happen in other contexts such as airport with different terminals managed by different airline companies and an airport operator.

There are 3 terminals in a port that have deployed each Standalone Non-Public Networks “SNPN_T1” to “SNPN_T3” to provide private network services to support business digital operations inherent to the activities of each terminal. On top of this, the port authority has deployed as well an Standalone Non-Public Network “SNPN_Umbrella” to be used as umbrella of the terminal’s SNPNs to support them as well as to provide coverage in the generic container 
exchange route (CER) between the terminals due to spectrum scarcity.

In the example depicted in the Figure 5.7.1-2 below, a User1 is on board a container ship and is using the terminal’s SNPN “SNPN_T1” when the container ship is at the quayside or the port authority umbrella “SNPN_Umbrella” when in between terminals and in the CER or the terminal’s SNPN “SNPN_T2” when also possible. The UE will need to seamlessly transition from the the terminal’s SNPN to the umbrella SNPN, or vice-versa, without any disruption to the private network service.

Figure 5.7.1-1: Interconnect between SNPNs in a port with multiple terminals

### 5.7.2	Pre-conditions

- The Terminal’ SNPNs (“SNPN_T1”, “SNPN_T2”, “SNPN_T3”) and the port authority umbrella SNPN “SNPN_Umbrella” are Standalone, i.e., not connected to any public networks or other cellular networks.

- The Terminal’ SNPNs (“SNPN_T1”, “SNPN_T2”, “SNPN_T3”) have each interconnect agreement with the port authority umbrella SNPN “SNPN_Umbrella” in place to allow the users of the Terminal’ SNPNs to access services on the umbrella SNPN. The agreements may specify the terms and conditions of the interconnect, including the charges and quality of service.

- The Terminal’ SNPNs “SNPN_T1” & “SNPN_T2” have each interconnect agreement with the others in place to allow UE from one SNPN to access some of the services of the other and vice-versa as they have the same business owner.

- There are permanent interconnects between the SNPNs as part of the interconnect agreements

- The User1 is registered to SNPN_T1.

### 5.7.3	Service Flows

- The User1 is connected to the terminal’s SNPN “SNPN_T1” when the container ship is at the quayside and is accessing private network services.

- The UE has an active private network service connection.

- As the container is leaving the terminal, the user 1’s UE has lost SNPN_T1 coverage and is starting to look for SNPN available which can allow to use the service intended and the User1 is within coverage of the port authority “SNPN_Umbrella”

- The UE, among different SNPNs that are available at this location, detects that SNPN_Umbrella offers services matching their connectivity needs.

- The UE start an initial registration procedure on SNPN_Umbrella using their credential / identity from SNPN_T1

- As part of the registration procedure, SNPN_Umbrella is able to forward the credential and identities of the devices to the SNPN Interconnect Credential Provider Umbrella

- The SNPN Interconnect Credential Provider Umbrella is able to identify User1 as authorised UEs to access SNPN_Umbrella due to the interconnect;

- User1 is now using SNPN_Umbrella

- Later on, the User1 is within coverage of the the terminal’s SNPN “SNPN_T2” which is preferred to be used

- The SNPN Interconnect Credential Provider T2 is able to identify User1 as authorised UEs to access SNPN_T2 due to the interconnect;

- User1 is now using SNPN_T2

### 5.7.4	Post-conditions

User A is attached to SNPN T2.

### 5.7.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2], clause 6.25 on Non Public Network includes the following requirements:

The 5G system shall support standalone operation of a non-public network, i.e. a non-public network may be able to operate without dependency on a PLMN.

The 5G system shall support a mechanism for a UE to identify and select a non-public network.

NOTE:	Different network selection mechanisms may be used for physical vs virtual non-public networks.

The 5G system shall support a mechanism to prevent a UE with a subscription to a non-public network from automatically selecting and attaching to a PLMN or non-public network it is not authorized to select.

### 5.7.6	Potential New Requirements needed to support the use case

[PR 5.7.6-001] Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support mechanisms for a UE to select the secondary interconnect SNPN(SISNPN) based on factors such as coverage, capacity, relation with the network and quality of service in case of interconnect with several SNPNs.

NOTE:	The network selection process during interconnect between SNPNs is expected to be automatic and transparent to the user, without requiring manual intervention.

[PR 5.7.6-002] Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support mechanisms to collect information per UE or per application related to the use of the SISNPN during interconnect between SNPNs (e.g., number of UEs, data volume, which SNPN the UE is primary subscriber which services, duration of the use of the visited network).