3GPP TSG SA WG 1 Meeting #104 	S1-233317

Chicago, USA, 13 - 17 November 2023	(revision of S1-233090, 3311)

Source:	Deutsche Telekom AG, Nokia, Samsung

pCR Title:	Pseudo-CR on Update of subclause network exposure

Draft Spec:	3GPP TS 22.137 v.1.0.0

Agenda item:	7.1.2

Document for:	Approval

Contact:	Vasil Aleksiev, Vasil dot Aleksiev at magenta dot at

Abstract: This pCR proposes update of network exposure functional service requirements

1. Introduction

At 3GPP SA1#103 additional consolidated potential requirements were agreed and these need to be introduced into normative work.

2. Reason for Change

Adding of the agreed network exposure CPRs from the FS_Sensing study is needed to finalize normative work.

The configuration of the use of UEs providing non-3GPP sensing data expresses the original intent of Sensing Group discovery. This term is now defined so that it only applies to 3GPP sensing data.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 22.137 v.1.0.0.

* * * First Change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3GPP sensing data: data derived from 3GPP radio signals impacted (e.g., reflected, refracted, diffracted) by an object or environment of interest for sensing purposes, and optionally processed within the 5G system.

5G Wireless sensing: 5GS feature providing capabilities to get information about characteristics of the environment and/or objects within the environment (e.g., shape, size, orientation, speed, location, distances or relative motion between objects, etc) using NR radio frequency signals, which, in some cases, can be extended by information created via previously specified functionalities in EPC and/or E-UTRAN.

non-3GPP sensing data: data provided by non-3GPP sensors (e.g., video, LiDAR, sonar) about an object or environment of interest for sensing purposes.

Sensing assistance information: information that is provided to the 5G system from a trusted third-party and can be used to support the derivation of a sensing result. This information does not contain 3GPP sensing data.

NOTE 1:	Examples of sensing assistance information are map information, area information, a UE ID attached to or in the proximity of the sensing target, UE position information, UE velocity information etc.

Sensing contextual information: information that is exposed with the sensing results by 5G system to a trusted third-party which provides context to the conditions under which the sensing results were derived. This information does not contain 3GPP sensing data.

NOTE 2:	Examples includes map information, area information, time of capture, UE location and ID. This contextual information can be required in scenarios where the sensing result is to be combined with data from other sources outside the 5GS.

Sensing measurement process: process of collecting sensing data.

Sensing receiver: a sensing receiver is an entity that receives the sensing signal which the sensing service will use in its operation. A sensing receiver is part of a RAN node or a UE. A Sensing receiver can be located in the same or different entity as the Sensing transmitter.

Sensing result: processed 3GPP sensing data requested by a service consumer.

Sensing signals: Transmissions on the 3GPP radio interface that can be used for sensing purposes.

NOTE 3:	This definition refers to NR radio frequency signals which, in some cases, can be extended by information created via previously specified functionalities in EPC and/or E-UTRAN.

Sensing transmitter: a sensing transmitter is the entity that sends out the sensing signal which the sensing service will use in its operation. A Sensing transmitter is part of a RAN node or a UE. A Sensing transmitter can be located in the same or different entity as the Sensing receiver.

Target sensing service area: a cartesian location area that needs to be sensed by deriving characteristics of the environment and/or objects within the environment with certain sensing service quality from the impacted (e.g., reflected, refracted, diffracted) wireless signals. This includes both indoor and outdoor environments.

Moving target sensing service area: the case where a target sensing service area is moving according to the mobility of a target from sensing transmitter’s perspective.

* * * Second Change * * * *

### 5.2.2	Configuration and authorization

Subject to regulation and operator’s policies, the 5G network shall be able to configure and/or authorize or revoke authorization of sensing transmitter(s) and sensing receiver(s) for 5G wireless sensing service.

NOTE 1: Such configuration and authorization can be based on sensing transmitter or sensing receiver location, specific time, sensing duration, sensing accuracy, target sensing geographical area, establishing of communication to transfer sensing data, etc.

NOTE 2: Such configuration and authorization can also include the selection of multiple sensing transmitters/receivers for 5G wireless sensing services.

Subject to regulation and operator’s policies, the 5G network may be able to configure and/or authorize or revoke authorization of UEs to provide non-3GPP sensing data.

NOTE 1: Such configuration and authorization can be based on UE location, specific time, sensing duration, sensing accuracy, target sensing geographical area, establishing of communication to transfer non-3GPP sensing data, etc.

NOTE 2: Such configuration and authorization can also include the selection of multiple UEs providing non-3GPP sensing data.

The 5G network shall be able to provide a mechanism for an MNO to configure UEs supporting V2X applications to support 5G Wireless sensing service when not served by RAN.

* * * End of Changes * * * *