3GPP TSG-SA1 Meeting #104	S1-233338

# Chicago, USA, 13 – 17 November 2023				   									(rev of S1-233105)

Source:	Qualcomm

Title:	pCR for FS_DualSteer Consolidated requirements

Agenda Item:	7.8.1

Draft Spec:	3GPP TR 22.841

Contact:	Francesco Pica
fpica <at> qti.qualcomm.com

Abstract: This document includes a pCR proposal to add consolidated potential requirements.

======================================  First CHANGE ========================================

# 6	Consolidated Potential Requirements

## 6.1	Introduction

The following requirements cover scenarios and functionalities for supporting enhanced traffic steering and switching of UE’s user data (for different services) across two 3GPP access networks, assuming the ability to differentiate the two connections for the same UE and minimize impacts to CN, O&M or IT systems.

Target scenarios cover two 3GPP access networks belonging to the same PLMN, or between two different PLMNs, or between one PLMN and one NPN, over same or different RAT,  which can use terrestrial and/or satellite access (including the case of two different satellite orbits).

Traffic steering and switching may or may not be used together (during the same UE data session), depending e.g., PLMN-RAT combination, coverage conditions (overlapping / disjoint), type of service, target improvement (e.g., load, throughput, mobility). Traffic policies are intended to be in full control of the network operator(s).

A UE can use traffic steering and switching (over two 3GPP access networks) for certain services, while other applications or services use single 3GPP access.

## 6.2	General

Table 6.2-1 – General Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.2-1 | Subject to operator(s) policy and network control, the 5G system shall be able to support mechanisms to enable traffic steering and/or switching of UE’s user data (for different services) across two 3GPP access networks belonging to the same PLMN (either HPLMN or VPLMN), assuming data anchoring in the HPLMN and non-simultaneous transmission over the two networks. Subject to operator(s) policy and network control, the 5G system may be able to support mechanisms to enable traffic steering and/or switching with simultaneous transmission of UE’s user data (for different services) across two 3GPP access networks belonging to the same PLMN (either HPLMN or VPLMN), assuming data anchoring in the HPLMN. Subject to operator(s) policy and network control, the 5G system shall be able to support mechanisms to enable traffic steering and/or switching of UE’s user data (for different services) across two 3GPP access networks belonging to two PLMNs, assuming a business agreement between PLMN operators (if different), data anchoring in the HPLMN and non-simultaneous transmission over the two networks. Subject to operator(s) policy and network control, the 5G system may be able to support mechanisms to enable traffic steering and/or switching with simultaneous transmission of UE’s user data (for different services) across two 3GPP access networks belonging to two PLMNs, assuming a business agreement between PLMN operators (if different) and HPLMN data anchoring. NOTE 1: Inter-PLMN requirements can apply also to PLMN-NPN scenarios assuming a PLMN-integrated NPN (NPN hosted by a PLMN or offered as a slice of a PLMN). | 5.1.6-001 5.2.6-001 5.2.6-002 5.3.6-001 5.4.6-001 5.5.6-001 5.6.6-001 5.6.6-002 5.7.6-001 5.8.6-001 5.9.6-001 5.10.6-001 5.11.6-001 5.12.6-001 5.14.6-001 5.17.6-001 5.18.6-001 5.12.6-001 |  |
| CPR 6.2-2 | For traffic steering and/or switching of UE’s user data across two 3GPP access networks (if/when supported), the 5G system shall be able to allow a HPLMN to provide a UE with policies and criteria to connect to an additional PLMN/NPN, or an additional RAT within the same PLMN. | 5.14.6-001 |  |
| NOTE 2: The above requirements assume configuration of traffic policies, under HPLMN control or negotiated between the HPLMN and other network operators, considering e.g., user subscription, application/traffic type, service preference, QoS requirements, location, time, UE capabilities, mobility, connectivity conditions. NOTE 3: The above requirements include the possibility to report UE specific user data measurements (e.g., RTT, packet loss rate) between the UE and the anchoring core network. |  |  |  |

## 6.3	Mobility and connectivity changes

Table 6.3-1 – Mobility Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.3-1 | Subject to operator(s) policy and network control, the 5G system shall be able to support mechanisms to enable service continuity when switching UE’s user data, for one or multiple services, between two 3GPP access networks. | 5.2.6-002 5.6.6-002 |  |
| CPR 6.3-2 | Subject to operator(s) policy and network control, for traffic steering and/or switching of UE’s user data across two 3GPP access networks belonging to the HPLMN and a VPLMN (if/when supported), the 5G system shall be able to support mechanisms to move UE’s user data from one VPLMN to another VPLMN, while maintaining the same access connection with the HPLMN. NOTE 1: UE can be connected to a maximum of two PLMNs simultaneously, including the HPLMN. NOTE 2: The above requirement can also apply to two 3GPP access networks belonging to a single PLMN, when moving UE’s user data from one 3GPP access network to a third 3GPP access network of the same PLMN. | 5.10.6-002 |  |
| CPR 6.3-3 | Subject to operator(s) policy and network control, for traffic steering and/or switching of UE’s user data across two 3GPP access networks (if/when supported), the 5G system shall be able to support mechanisms to change one 3GPP access network to a non-3GPP access network (and vice versa). | 5.13.6-001 |  |

## 6.4	Charging and other aspects

Table 6.4-1 – Charging and other Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.4-1 | Subject to operator(s) policy and network control, the 5G system shall be able to collect charging information during traffic steering and/or switching of UE’s user data across two 3GPP access networks (if/when supported). NOTE 1: Charging information should be collected for both 3GPP access networks; in case the two 3GPP access networks belong to different PLMNs, or a PLMN and NPN, a proper business agreement among network operators is assumed. | 5.5.6-002 5.9.6-002 5.17.6-002 |  |
| CPR 6.4-2 | Subject to operator(s) policy and network control, the 5G system shall be able to support traffic steering and/or switching of UE’s user data between a NPN and a PLMN, for one or more UEs with a NPN subscription accessing NPN services, to meet specific QoS requirements for each UE, assuming non-simultaneous transmission over the two 3GPP access networks. NOTE 2: The above assumes a NPN hosted by a PLMN or offered as a slice of a PLMN, data anchoring in the NPN, and a business agreement between the PLMN and the NPN operator (if different). | 5.15.6-001 5.16.6-001 |  |

===============================  End of Change ===============================