3GPP TSG-SA WG1 Meeting #104 	S1-233403

Chicago, USA,  13 - 17 November 2023	(revision of S1-233173)

Source:	Huawei, OPPO

pCR Title:	Pseudo-CR Ambient IoT consolidated KPI Table Sensor data Communication Range alignment

Draft Spec:	3GPP TR 22.840

Agenda item:	7.2.1

Document for:	Approval

Contact:	Shuang Zhang (zhang.shuang2@huawei.com)

Abstract: This document provides a TP to TR 22.840 v.2.0.0 to align the captured (not updated with the agreed pCRs at SA1 103) for “Communication Range” in the Table 7.2.2-1 “KPIs for sensor data collection”.

1. Introduction

The consolidated KPI Table 7.2.2-1 in TR 22.840 v2.0.0 currently captures for Communication Range:

- square-bracketed “[10-30]m” in the row “Indoor Room environment  monitoring (e.g. domicile, machinery)”, with UC#5.6 and UCC#5.13 being referred to;

- square-bracketed “[300 m – 700 m]” in the row “Outdoor Smart city”, with UC#5.24 and UCC#5.25 being referred to;

2. Reason for Change

For consolidated KPI Table 7.2.2-1 row “Indoor Room environment  monitoring (e.g. domicile, machinery)”, in TR 22.840 v2.0.0 the corresponding values are captured in respective use cases:

- “10-30m (Indoors)” in Table 5.6.6-1;

- “30m indoors” in Table 5.13.6-1;

For consolidated KPI Table 7.2.2-1 row “Indoor Room envirionment  monitoring (e.g. domicile, machinery)”, at SA1 #103, S1-232380 and S1-232172 were agreed (updating Communication Range to 300-500m for UC#5.24, UC#5.26). Due to reasons such as the time restriction, it was acceptable at SA1 103 that TR v.2.0.0 would focus on implementing changes to KPI tables of individual use cases, leaving the action point of updating/aligning the values in consolidated KPI tables for SA1 104.

3. Conclusions

Update values of Communication Range in consolidated KPI Table 7.2.2-1:

- Adopt “300-500m” as the value in the row “Outdoor Smart city”;

- Remove “[]” from “[10-30]m” in the row “Indoor Room environment  monitoring (e.g. domicile, machinery)”;

Add Note2 to Table 7.2.1-1 just to synchronize the agreed changed in S1-233402.

4. Proposal

It is proposed to agree on the following changes to 3GPP TR 22.840 v2.0.0.

* * * First Change * * * *

## 7.2	Consolidated potential KPIs

### 7.2.1	KPIs for inventory

Table 7.2.1-1 KPIs for inventory

| Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inventory or asset management | Typically, seconds level | 99% | NA | <2 Kbit/s | 96/256 bits | <1,5 Million/km² (Note 2) | 30~50m indoor, 200m~400m outdoor | 1-10km² | 3~10km/h | NA | NA | NA | 3 m indoor, cell-level outdoor | UC#5.1, UC#5.2, UC#5.5, UC#5.7, UC#5.16 TS#6.1 |
| NOTE 1: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. NOTE 2: The device density is much lower outdoors as only a subset of assets (e.g. stored indoors) will be in transit, and a much larger area for transit applies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.2.2	KPI for sensor data collection

Table 7.2.2-1 KPIs for sensor data collection

| Deployment | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | Room envirionment monitoring (e.g. domicile, machinery) | 1-20 s | 99 % | NA | <1 kbit/s | < 100 bits | NA | 10-30m | NA | Stationary | NA | NA | NA | NA | UC#5.6, UC#5.13 |
|  | Indoor agriculture and husbandry | >10 s | 99.9% | NA | <1 kbit/s | Typically, [< 100 bytes] | 1 per m² | 30 - 200 m | 6000 m²~30,000m² | Quasi-stationary | 15 minutes to half an hour | NA | NA | NA | UC#5.20 UC#5.23 TS#6.2 UC#5.18 |
| Outdoor | Smart grid | 1 s | 99% | NA | < 1kbit/s | Typically, < 100 bytes | < 10,000 /km² | Typically 50-200 meters | [several km² up to 100 000 km²] | Stationary | 5-15 min | NA | NA | several 10 m | UC#5.3 |
|  | Outdoor husbandry and logistics | Typically, > tens of seconds | 99% | NA | <500 bit/s | Typically, [< 100 bytes] | <5200 devices / km² | [300 m - 500 m] | 430000 m² | Up to 3 km/h | 15 min | NA | NA | NA | UC#5.22, UC#5.19 |
|  | Smart city | 10 s - 30 s | 99% | NA | <1 kbit/s | Typically, [< 100 bytes] | <1000 devices / km² | 300 m - 500 m | City wide including rural areas | Stationary | 15 min | NA | NA | NA | UC#5.24, UC#5.25 |
| NOTE 1: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.2.3	KPI for tracking

Table 7.2.3-1 KPIs for tracking

| Deployment | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | Indoor tracking | 1s | 99.9% | NA | <1 kbit/s | <1k bits | 25/100 m² | 10m indoor | 200 m² | Static | 1 per hour | 1s | 90% | 1-3 m, 90% availability | UC#5.8 UC#5.10 UC#5.12, UC#5.14 UC#5.21 |
| Outdoor | Outdoor tracking | 1s | 99.9% | NA | <1 kbit/s | <1 kbits | <10 per 100 m² | 500m | Up to the whole PLMN | up to [10 km/h] | 1 per hour | 1 s | 99% | several 10m | UC#5.8 UC#5.9 UC5.12 |
| NOTE 1: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.2.4	KPI for actuator control

Table 7.2.4-1 KPIs for actuator control

| Deployment | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | Indoor actuator control | Several seconds | 99% | NA | 2 kbit/s | <100 Bytes | less than 1.5 million/km² | 50m indoors | <250 m² for home, and 15,800 square meters for supermarket | stationary | 20 minutes to 2 hours | NA | NA | 3 m to 5 m indoor | UC#5.11UC#5.26TS#6.3 |
| Outdoor | Outdoor actuator control for large coverage | hundreds ms level | 99% | N/A | NA | 128bit (DL) | NA | [500]m outdoors | NA | Static | NA | NA | NA | NA | UC#5.30 |
|  | Outdoor actuator control for medium coverage | Several seconds | 99% | NA | <2 kbit/s | <200 bits | <200 per 100 m² | 200 m | City wide including rural areas | Static | NA | NA | NA | 3 m to 5 m indoor | UC#5.26UC#11 |
| NOTE 1: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of the First Change * * * *