3GPP TSG-SA WG1 Meeting #104 	S1-233421

Chicago, US, 13 - 17 November 2023	(revision of S1-233407, 3400, 3167 and 2649)

Source:	KPN, Vodafone, Huawei

pCR Title:	Overview for Ambient IoT TS

Draft Spec:	3GPP TS 22.xxx

Agenda item:	7.2.2

Document for:	Approval

Contact:	Toon . Norp @tno .nl

Abstract: This document provides a revision of the S1-232649 tdoc that was agreedat SA1 #103. The update addresses the Editor’s Note related to the Reachability characteristic.

Note that this pCR includes the complete text as was proposed in S1-232649. This because the Ambient IoT work item was not yet approved and it is unclear whether S1-232649 had formally been implemented in the new Ambient IoT TS.

****************Start of Change *************

# 4	Overview

# 4.1	Introduction

In the 5G era, various IoT technologies [2][3][4][5] such as eMTC, NB-IoT, and RedCap have been developed to fulfil the increasing demand from verticals. These IoT technologies have achieved low cost, low power and massive connections and can meet requirements of many applications. However, there are still some use cases and applications that can benefit from an IoT technology that requires less power and has lower cost than previous IoT technologies. Improvementss can be made where maintenance-free devices are required (e.g. where the devices are inaccessible and it is not possible to replace the device battery) or for devices in extreme environmental conditions. Finally, ultra-low complexity, very small device size/form factor (e.g. thickness of mm), longer life cycle, etc. are required for some use cases. Ambient IoT is a technology to fulfil these market requirements.

This technical specification describes the Ambient IoT technology service requirements as part of the 5G system to enable new services and use cases. Ambient IoT has the potential to benefit a large number of vertical industries, e.g. smart manufacturing, logistics and warehousing, smart grid, agriculture, and smart home by providing functionalities that fulfil the needs of industrial use cases. Therefore, a new kind of IoT service for the vertical industries will be enabled by combining Ambient IoT with cellular networks, vastly benefitting the 3GPP ecosystem.

## 4.2	Characteristics of Ambient IoT

Not all Ambient IoT devices are the same. Nevertheless, Ambient IoT devices have the following characteristics.

-	Energy harvesting. An Ambient IoT device is an IoT device powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g. using a capacitor). Energy is provided through harvesting of radio waves, light, motion, heat, or any other suitable power source. Energy harvesting can be continuous or incidental (e.g. based on the vibration that a vibration sensor has to report). It cannot be assumed that Ambient IoT devices always have enough power to initiate or receive communication.

-	Low complexity. Ambient IoT devices are expected to have lower complexity, smaller size, reduced capabilities and lower power consumption than previously defined 3GPP IoT devices (e.g. NB-IoT/eMTC devices). Low complexity of Ambient IoT devices is also expected to be reflected in efficient use of network resources. In general, Ambient IoT applications will deploy very large numbers of Ambient IoT devices.

-	Low data rates. Generally, Ambient IoT data transmissions contain only a low amount of data.

-	Life span. Ambient IoT devices can be maintenance free and can have long life span (e.g. more than 10 years). However, the life span of an Ambient IoT device can also be relatively short, e.g. when tracking a package through a logistics chain.

-	Communication characteristics. Ambient IoT devices can have a variety of communication characteristics, different from other IoT devices, based on how the Ambient IoT devices are powered by energy harvesting and whether / how the harvested energy can be stored. Ambient IoT devices will only be able to communicate when they have enough power. This can be an issue, especially when communication is initiated towards the Ambient IoT device, while it is not known whether the Ambient IoT device has enough power to receive this communication. For communications initiated by the Ambient IoT device, the Ambient IoT device cannot transmit data until it has harvested / stored enough energy. Some Ambient IoT devices can be powered on demand when they need to communicate. Additionally, some Ambient IoT devices will be able to communicate on a regular basis and have communication characteristics similar to regular IoT devices.

## 4.3	Typical Ambient IoT use cases

Ambient IoT can support many different use cases. Nevertheless, in general the Ambient IoT use cases can be characterised in four different use case categories:

-	Inventory taking. With inventory taking, the main purpose is to discover what goods (e.g. boxes, containers, packages, tools) are present in a specific area. Upon request sent by the network within the specific area, Ambient IoT devices attached to these goods report an identifier associated with the good, possibly supplemented with other information such as status, measurement results and/or location.

-	Sensor data collection. With sensor data collection, the Ambient IoT device is associated with a sensor. Transfer of sensor data can be initiated by the Ambient IoT device, e.g. periodically or when the Ambient IoT device has power, or can be triggered by the network.

-	Asset tracking. With asset tracking, the main purpose is to determine the location of goods. Ambient IoT devices attached to these goods report an identifier associated with the good. This can then be combined with location information. Asset tracking can also be initiated by an Ambient IoT capable UE (i.e. a UE that can communicate with an Ambient IoT device), thus finding the location of Ambient IoT devices within a particular range of the UE.

-	Actuator control. With actuator control, the Ambient IoT device is associated with an actuator. Transfer of actuator commands is generally initiated by the network.

*******************Second Change*****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.368: "Service requirements for Machine-Type Communications (MTC)".

[3]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[4]	3GPP TS 22.261: "Service requirements for the 5G system".

[5]	3GPP TS 22.011: “Service accessibility”.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.368: "Service requirements for Machine-Type Communications (MTC)".

[3]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[4]	3GPP TS 22.261: "Service requirements for the 5G system".

[5]	3GPP TS 22.011: “Service accessibility”.