3GPP TSG SA WG 1 Meeting #104 	S1-233425

Chicago, USA, 13 - 17 November 2023	(Revision of S1-233413, S1-233071)

Source:	OPPO

pCR Title:	Input to new TS - Ambient IoT performance requirements

Draft Spec:	3GPP TS 22.XXX

Agenda item:	7.2.2

Document for:	Approval

Contact:	xuweijie@oppo.com, Yang Xu (xuyang@oppo.com)

Abstract: This pCR proposes updates to the TS

1. Introduction

This pCR introduces descriptions and requirements of ambient IoT performance in section 6. The agreed KPI value to TR22.840 updation in SA1#103 and #104 so far is captured in KPI consolidation.  Some format improvement for TS is also introduced.

2. Reason for Change

Overall descriptions and performance requirements need to be described.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 22.XXX

****************Start of Change *************

6.2	Performance service requirements for Inventory

Table 6.2-1 KPIs for inventory

| Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inventory or asset management | Typically, seconds level | 99% | NA | <2 kbit/s | 96/256 bits | <1.5 million devices/km² indoor only (Note 2) | 30 m – 50 m indoor, 200 m - 400 m outdoor | 1 km² – 10 km² | 3 km/h – 10 km/h | NA | NA | NA | 3 m indoor, cell-level outdoor |  |
| NOTE 1: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. NOTE 2: The device density is much lower outdoors as only a subset of assets (e.g. stored indoors) will be in transit, and a much larger area for transit applies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.3	Performance service requirements for sensor data collection

Table 6.3-1 KPIs for sensor data collection

| Deployment | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | Room environment monitoring (e.g. domicile, machinery) | 1 s - 20 s | 99 % | NA | <1 kbit/s | <100 bits | NA | 10 m - 30 m | NA | Stationary | NA | NA | NA | NA |  |
|  | Indoor agriculture and husbandry | >10 s | 99.9% | NA | <1 kbit/s | Typically, [<100 bytes] | 1 device /m² | 30 m - 200 m | 6,000 m² - 30,000 m² | Quasi-stationary | 15 min - half an hour | NA | NA | NA |  |
| Outdoor | Smart grid | 1 s | 99% | NA | <1 kbit/s | Typically, <100 bytes | < 10,000 devices /km² | Typically, 50 m - 200 m | [several km² up to 100,000 km²] | Stationary | 5 min - 15 min | NA | NA | several 10 m |  |
|  | Outdoor husbandry and logistics | Typically, > tens of seconds | 99% | NA | <500 bit/s | Typically, [<100 bytes] | <5,200 devices/ km² | [300 m - 500 m] | 430,000 m² | ≤ 3 km/h | 15 min | NA | NA | NA |  |
|  | Smart city | 10 s - 30 s | 99% | NA | <1 kbit/s | Typically, [<100 bytes] | <1,000 devices/ km² | 300 m - 500 m | City wide including rural areas | Stationary | 15 min | NA | NA | NA |  |
| NOTE: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.4	Performance service requirements for tracking

Table 6.4-1 KPIs for tracking

| Deployment | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | Indoor tracking | 1 s | 99.9% | NA | <1 kbit/s | <1 kbits | 25/100 m² | 10 m | 200 m² | Static | 1 hour | 1 s | 90% | 1 m - 3 m, 90% availability |  |
| Outdoor | Outdoor tracking | 1 s | 99.9% | NA | <1 kbit/s | <1 kbits | <10 per 100 m² | 500 m | Up to the whole PLMN | up to [10 km/h] | 1 hour | 1 s | 99% | several 10 m |  |
| NOTE: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.5	Performance service requirements for actuator control

Table 6.5-1 KPIs for actuator control

| Deployment | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | Indoor actuator control | Several seconds | 99% | NA | 2 kbit/s | <100 Bytes | <1.5 million/km² | 50 m | <250 m² for home, and 15,800 square meters for supermarket | stationary | 20 min - 2 hours | NA | NA | 3 m to 5 m indoor |  |
| Outdoor | Outdoor actuator control for large coverage | hundreds ms level | 99% | N/A | NA | 128 bit (DL) | NA | [500] m outdoors | NA | Static | NA | NA | NA | NA |  |
|  | Outdoor actuator control for medium coverage | Several seconds | 99% | NA | <2 kbit/s | <200 bits | <20 devices per 100 m² | 200 m | City wide including rural areas | Static | NA | NA | NA | 3 m to 5 m indoor |  |
| NOTE: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

*******************End of Change*****************