3GPP TSG-SA WG1 Meeting #104 	S1-233462

Chicago, USA, 13 - 17 November 2023	(revision of S1-233200)

Source:	Novamint, b-com, EDF

pCR Title:	Pseudo-CR on consolidated requirements for TR 22.848

Draft Spec:	3GPP TR 22.848 version 0.2

Agenda item:	7.11

Document for:	Approval

Contact:	Thierry Bérisot (tberisot@novamint.com)

Abstract: This document includes a text proposal for section Consolidated Requirements of TR 22.848, capturing a set of final consolidated potential requirements

1. Introduction

The “Consolidated Requirements” of TR 22.848 are currently not included in version 0.2.0

2. Reason for Change

adding texts and tables for section Consolidated Requirements of TR 22.848, capturing a set of final consolidated potential requirements

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.848 v 0.2.0.

===============================  All new Text ================================

# 6	Consolidated Requirements

## 6.1	Introduction

The following requirements represent a consolidation of the various potential requirements captured in the above use cases related to the interconnect of SNPN.

## 6.2	General

The potential requirements corresponding to general aspects of interconnect of SNPN are listed in the table below

Table 6.2-1 – Consolidated Requirements for eneral aspects of interconnect of SNPN

| CPR # |  | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- | --- |
| CPR 6.2-1 |  | Subject to SNPNs operators’ agreement, the 5G system shall be able to support mechanisms to enable interconnect between SNPNs | [PR 5.2.6-001] |  |
| CPR 6.2-2 |  | Subject to SNPNs operators’ agreement, the 5G system shall be able to support mechanisms for a selection of 5GS endpoints for SNPNs’ interconnect. | [PR 5.2.6-002] |  |
| CPR 6.2-3 | Subject to SNPNs operator’s policy, the 5G system shall be able to support routing of user data through interconnected SNPNs. |  | [PR 5.3.6-002] |  |
| CPR 6.2-4 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support prioritization of resources for a service offered by a SNPN that is consumed by users attached to other interconnected SNPNs |  | [PR 5.5.6-001] |  |

## 6.3	Service continuity

The potential requirements corresponding to the support of service continuity aspects are listed in the table below

Table 6.3-1 – Consolidated Requirements for Self Configuring group of interconnected SNPNs

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.3-1 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support service continuity of UE communications between interconnected SNPNs with overlapping radio coverage. | [PR 5.4.6-002] |  |
| CPR 6.3-2 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall enable packet loss to be minimized during mobility of a UE between interconnected SNPNs with overlapping radio coverage for some or all connections associated with this UE. | [PR 5.4.6-003] |  |
| CPR 6.3-3 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall minimize communication service interruption time during mobility of a UE between interconnected SNPNs with overlapping radio coverage for some or all connections associated with this UE. | [PR 5.4.6-004] |  |

## 6.4	Self Configuring group of interconnected SNPNs

The potential requirements corresponding to the support of Self Configuring group of interconnected SNPNs are listed in the table below

Table 6.4-1 – Consolidated Requirements for Self Configuring group of interconnected SNPNs

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.4-1 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support interconnect of standalone non-public networks through links established on a temporary basis forming a group of interconnected SNPNs with a variable topology. | [PR 5.3.6-001] |  |
| CPR 6.3-4 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support routing of user data through interconnected SNPNs. | [PR 5.3.6-002] |  |
| CPR 6.4-2 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support discovery, addition and authentication, or detachment of SNPN in a group of interconnected SNPNs. | [PR 5.3.6-003] |  |
| CPR 6.4-3 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support per UE communication, the establishment, reconfiguration and release of user data transmission path between two SNPNs in a group of interconnected SNPNs. | [PR 5.3.6-004] |  |
| CPR 6.4-4 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support a user subscribed to one Standalone Non-Public Network to consume a service offered by another SNPN in a group of interconnected SNPNs. | [PR 5.3.6-005] |  |
| CPR 6.4-5 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support a mechanism to exploit interconnection links metrics (e.g. jitter, latency, packet loss, capacity) as a criterion to select a suitable route between two distant SNPN in a group of interconnected SNPNs. | [PR 5.3.6-006] |  |
| CPR 6.4-6 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support discovery, addition and authentication, or detachment of SNPN in a group of interconnected SNPNs without interrupting ongoing communications. | [PR 5.4.6-001] |  |
| CPR 6.4-7 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall enable a SNPN to monitor the currently delivered end to end QoS for a service hosted by this SNPN and consumed by one or several users attached to other SNPNs inside a group of interconnected SNPNs; | [PR 5.5.6-002] |  |
| CPR 6.4-8 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to provide event notification upon detecting that the requested end to end QoS level cannot be met for a service hosted by this SNPN and consumed by one or several users attached to other SNPNs inside a group of interconnected SNPNs. | [PR 5.5.6-003] |  |