3GPP TSG- SA1 Meeting # 104	S1-233586

Chicago, USA,  13 - 17 November 2023                                              (revision of S1-233150)

# Source:	China Unicom

# Title:	New SID: Study on Multi-network Interoperability Enhancement

# Document for:	Approval

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on Multi-network Interoperability Enhancement

Acronym: FS_NWIOP

Unique identifier:

Potential target Release:	Rel-20

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No |  |  | X |  |  |
| Don't know | X | X |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

3	Justification

5G can be deployed in different deployment options, which are SA options and NSA options. The early deployments are adopting NSA as a means of fast launching 5G and may gradually evoluting to SA. Based on operators’ different network development progress and deployment strategies, SA networks and NSA networks will coexist and interact in same PLMN or different PLMNs for a long period of time.

3GPP TS 22.261 5.1.2 specified requirements of supporting interworking between 5G systems and interoperability with legacy 3GPP systems. TS 23.501 defined architectures for interworking between 5GS and EPC via dedicated entities (i.e. SMF+PGW-C, UPF+PGW-U, UDM+HSS), and interworking procedures are described in TS 23.502. However, a situation which subscribers of a EPC based 5G NSA network roaming and/or moving to a 5G SA only network is not included in and applicable for those requirements due to lack of interaction capability (e.g. authentication, subscription data retrievement, etc.) between 5GC and EPC without the dedicated entities.

In some scenarios, the interaction capability is desired, including:

-	While the NSA(EPC) subscriber roams into a SA only network, the roamer could not access the 5G network and use 5G data service provided by the SA network since 5GC does not support interacting with NSA EPC (i.e. the interaction between them is not specified).

-	Besides the roaming scenario, this also may cause problem for domestic scenario that subscribers of EPC based NSA networks move from NSA area to SA only area.

There are ongoing initiatives for support of interworking between 5GC and EPC (SA only network and EPC based NSA network). It is necessary to consider the interact issue between them and determine the potential requirements in order to enable 5G data service to be provided for all subscribers of a 5G network and for inbound roamers.

4	Objective

The objectives of this study are:

- Investigate use cases which enable 5G system (e.g. 5G SA network) to provide data service for R15 network (e.g. 5G NSA option 3) users.

- Derive potential requirements from those use cases regarding the following aspects:

-	Network access control

-	Subscription data retrieval & generation

-	Service continuity

-	Identify user’s subscription type (i.e., subscription 5G SA network or NSA network if those networks are coexisting).

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 22.XXX | Study on Multi-network Interoperability Enhancement | SA#105 | SA#106 |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

6	Work item Rapporteur(s)

7	Work item leadership

SA1

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

China Unicom

Huawei

ZTE