3GPP TSG- SA1 Meeting # 105	S1-240034

Athens, Greece, 26 Feb - 1 March 2024

# Source:	NEC

# Title:	New SID: Study on Advanced Non-Public Networks and their interworking

# Document for:	Agreement

# Agenda Item:	7

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Study on Advanced Non-Public Networks and their interworking

Acronym: FS_aNPNet

Unique identifier:	TBD

Potential target Release:	Rel-20

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No |  |  |  |  |  |
| Don't know | X |  | X |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a study item.

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 1020031 | Upper layer traffic steering and switching over dual 3GPP access | Antecedent Study and Work Item |
| 1000027 | Non-Public Network (NPN) security considerations | Antecedent Building Block |
| 920038 | 5G Networks Providing Access to Localized Services (PALS) | Antecedent Study and Work Item |
| 910038 | Enhanced Access to and Support of Network Slice (EASNS) | Antecedent Study and Work Item |
| 840050 | Enhancements for cyber-physical control applications in vertical domains | Antecedent Study and Work Item |
| 1020070 | Study on Multi-Access (DualSteer and ATSSS_Ph4) | Related ongoing Stage 2 work |
| 980128 | Non-Public Networks Phase 2 | Related Rel-18 Stage 2 and Stage 3 |
| 910065 | Enhanced support of Non-Public Networks | Related Rel-18 Stage 2 and Stage 3 |

# 3	Justification

The standardization up to Rel-19 on non-public networks (NPNs) enables verticals to replace existing proprietary radio or fixed Ethernet networks and supports standalone and public-network-integrated non-public networks (SNPNs and PNI-NPNs). Nevertheless, it is necessary to consider further enhancements of these networks to cater for more diverse deployment models and greater interworking between different types of private networks and, consequently, between private and public networks.

Present Stage 1 specifications define a number of 5G system requirements to support NPN subscribers (TS 22.261, Section 6.25). However, in the main, there is no differentiation of different types of NPNs and, therefore, not all the requirements for future PNI-NPNs are addressed. PNI-NPN networks are referred to in Stage 1 specifications as '…can be hosted by a PLMN, or they can be offered as a slice of a PLMN', aligned with Stage 2 specifications. PNI-NPNs hosted by PLMN cater to only one type of PNI-NPNs. While slicing enables the creation of customised networks, the focus is on the separation of customers, security aspects, and meeting specific service requirements. There is limited functionality to manage a slice in a multi-domain scenario, and a flexible way of selecting a part of or the whole serving control/data plane or network functions in a multi-domain scenario is lacking. Hence, greater flexibility in selecting network (control) functions based on a specific service, reliability requirement,  user's security profile, load, signalling latency/geographical location and other aspects of service is needed. That will enable more diverse deployment models to meet industry-tailored PNI-NPNs with services distributed between on-site, regional edge and cloud locations and where PNI-NPN UEs move between the sites. Opening up more deployment PNI-NPN models would also benefit PLMNs. Furthermore, it can be expected that the support for such service and network flexibility would prompt the work towards more dynamic and flexible Service Level Agreements in 6G.

There is also an absence of requirements on better interworking between SNPNs and PNI-NPNs, for example, in a defined locality.  Firstly, roaming and handover between SNPNs and generally PLMNs are currently explicitly excluded from the (Stage 2) specifications. Addressing the technical impediments, particularly security-related, to handover between SNPN and PNI-NPN in a defined locality is a first step. Also, concepts such as distributed device and user trust (S1-233028) can be taken into account. Secondly, the normative requirements of Rel-19 Traffic steering and switching over two 3GPP access networks (TS 22.261, Section 6.50) do not apply to SNPN networks, which also excludes any SNPN-PNI-NPN interworking. Thirdly, current Stage 1 specifications on interworking with non-3GPP access mainly targets offload and distribution of traffic.

Hence, there is a need to improve the system's reliability, provide service continuity or at least minimize service interruption, and enhance overall QoS by addressing single and dual 3GPP access and greater integration of 3GPP and non-3GPP access.

# 4	Objective

The study is proposed to address the following objectives:

- Study PNI-NPNs and requirements for a flexible selection of a serving (control/data plane) network function(s) across multi-operator domains

- Study requirements on interworking between different types of NPNs using 3GPP only, as well as 3GPP and non-3GPP accesses to increase reliability, provide service continuity and improve other aspects of QoS

  - NOTE 1: The use cases from TR 22.841 should also be considered.

  - Identify the technical impediments to handover between SNPN and PNI-NPN in a defined locality

- Identify the resulting Stage 1 impact on NPNs, network slicing, security, network exposure and charging

NOTE 2: UEs with single and multiple subscriptions, and appropriate radio capabilities should be considered.

NOTE 3: The proposed concept of distributed device and user trust (S1-233028) can be taken into account.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR | 22.xxx | Study on Advanced Non-Public Networks and their interworking | TBD | TBD | NEC |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Jasmina McMenamy, NEC

# 7	Work item leadership

SA1

# 8	Aspects that involve other WGs

None.

# 9	Supporting Individual Members

Supporting IM name

NEC