3GPP TSG- SA1 Meeting # 105	S1-240052

Athens, Greece, 26 Feb - 1 March 2024	(revision of S1-24xxxx)

# Source:	Novamint, Thales, Airbus, Eutelsat Group, Fraunhofer IIS, TNO, ESA, SES, ETRI, vivo, SKY Perfect JSAT, Sateliot, Lockheed Martin, Hughes Network systems, CATT, Nokia, Nokia Shanghai Bell, OQ Technology, China Telecom

# Title:	New SID: Study on satellite access - Phase 4

# Document for:	Approval

Agenda Item:	7

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on satellite access - Phase 4

Acronym: FS_5GSAT_Ph4

Unique identifier:

Potential target Release:	Rel-20

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 770002 | Study on using Satellite Access in 5G | Previous Rel-16 work covering satellite access in 5GS related use cases and requirements (ref. TR22.822: FS_5GSAT) |
| 790001 | New Services and Markets Technology Enablers– Phase 2 | Stage 1 work item of 5G system. |
| 720005 | New Services and Markets Technology Enablers | Stage 1 work item of 5G system |
| 800010 | Integration of Satellite Access in 5G | Rel-18 Stage-1 Normative Work on the support of satellite |
| 860005 | Integration of satellite systems in the 5G architecture (5GSAT_ARCH) | Rel-18 Stage-2 Normative Work on the support of satellite |
| 960016 | Study on satellite access - Phase 3 | Rel-19 stage-1 study on the support of satellite access phase 3 (ref. TR22.865 FS_5GSAT_Ph3) |
| 1000024 | Satellite access - Phase 3 | Rel-19 stage-1 Normative Work on the support of satellite |
| 1010033 | Study on Integration of satellite components in the 5G architecture Phase 3 | Rel-19 stage-2 study on the support of satellite access phase 3 (FS_5GSAT_Ph3_ARCH) |

# 3	Justification

New capabilities for satellite access such as Store and Forward Satellite operation and UE-Satellite-UE communication have been addressed in SA1 5GSat phase 3 for Release 19. First commercial deployments of 5G based satellite are happening and many new use cases/demands for satellite access emerging are to be addressed still in the context of 5G advanced (i.e. in Release 20).

These use cases and capabilities to be addressed in Rel-20 are the following:

1/ Support of Multicast and Broadcast Services (MBS) over Satellite

Broadcast services constitute more than 50% of the revenue of satellite operators today based on classical DVB systems. A migration to 3GPP based technology shall enable MBS services to be available to allow a smooth evolution of the satellite operator’s broadcast services. MBS has the potential to complement the terrestrial and satellite networks with an efficient means to multicast and broadcast content to the end users. which can provide substantial improvements, especially regarding efficient resource usage to distribute content, such as software updates, media content for example. It can complement as well unidirectional link services.

2/ Enhanced support for emergency calls/messaging and mission critical services via satellite

With the first commercial deployments of 5G based satellite, there is a strong demand for supporting 5G emergency communication and mission critical services via satellite (e.g., emergency call for automotive, emergency messaging for handsets or IoT devices). Given that the regulation is evolving at least with higher location accuracy requirements; there is therefore a need for enhanced support of emergency calls/messaging services and mission critical by satellite access.

3/ Enhanced support for multi-orbits satellite networks

Several satellite operators are expected to deploy multi-orbits satellite networks combining LEO, MEO and GEO leveraging as well the notion of regenerative payload on board the satellite. The impacts of such 3GPP based megaconstellation combining multi-orbits on the service requirements for satellite acces needs to be studied (e.g. enablers for QoS/SLA management, energy consumption/traffic/resource management, roaming and handover aspects) in the context of 5G Advanced.

4/ Enhanced Network selection between satellites or between satellites and terrestrial

With the deployments of different 3GPP based satellites accesses using different orbits LEO, MEO or GEO (for NR or IoT) including the deployment of megaconstellation combining multi-orbits, the network selection needs to be enhanced with the introduction of new parameters to support the different possibilities including roaming between multi orbits satellites or between satellites and terrestrial (e.g., policies depending on the location, selection based on metrics, part of mega constellation, availability of the coverage if discontinuous).

5/ Enhanced resilient UE-Satellite- UE communication with intermittent ground connectivity

There is a need to support more robust system resilience to enhance UE-Satellite- UE communication with intermittent ground connectivity for example in case of ground disaster or ground connection failure (ISL outage or feeder link failure).

In addition to the service requirements, KPIs for a 5G system with satellite access have been introduced in TS 22.261 (section 7.4) in the context of Rel-16. Since the KPIs for satellite access have not been updated and do not reflect KPIs for 5G Advanced for the satellite access. This is especially valid for service to vehicular/drone mounted devices for verticals as well as for IoT devices. It is important to review the KPIs of 5G satellite networks and potentially update them. Furthermore, there is a need to clarify KPIs associated to service continuity.

# 4	Objective

The objectives of this study are the following:

-	to study new use cases and related regulatory requirements and identify new potential service requirements and enhancement for 5G system over satellite, including:

- Support of Multicast and Broadcast Services (MBS) over satellite

- Enhanced support for emergency communications and mission critical services via satellite

- Enhanced support for multi-orbits satellite networks

- Enhanced Network selection between satellites including roaming between multi orbits satellites or between satellites and terrestrial

- Enhanced resilient UE-Satellite- UE communication with intermittent ground connectivity

-	Review and update KPIs of 5G Advanced satellite networks (especially for service to vehicular/drone mounted devices for verticals) including clarifying KPIs associated to service continuity

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 22.xxx | Study on satellite access - Phase 4 | TSG#106 | TSG#107 | Thierry Bérisot, Novamint, (tberisot@novamint.com) |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Rapporteur: Thierry Bérisot, Novamint, tberisot@novamint.com

# 7	Work item leadership

SA1

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

Novamint

Thales

Airbus

Eutelsat Group

Fraunhofer IIS

TNO

SES

ESA

ETRI

Vivo

SKY Perfect JSAT Corporation

Sateliot

Lockheed Martin

Hughes Network systems

CATT

Nokia

Nokia Shanghai Bell

OQ Technology

China Telecom