3GPP TSG-WG SA1 Meeting #105	S1-240202

# 26 February – 1 March, 2024, Athens		(revision of S1-240073)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.137 | CR | 0001 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on KPI table for sensing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | Sensing |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the SA1#104 meeting, the KPI table for sensing in section 6.2 of TS 22.137 is updated based on the agreed document S1-233339. It is agreed that the following sentences in the last column are deleted in order to simplify KPI table: - “requiring higher performance than category 1” - “requiring higher performance than category 3” However, based on the current TS 22.137, there is still one sentence in the last column for Sensing service category#4. - “requiring higher performance than category 3” It is proposed to delete it to align with the conclusion proposed by the agreed document S1-233339. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction on KPI table for sensing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The KPI table is not aligned with conclusion of the SA1#104 meeting. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2	Requirements

The 5G system shall be able to provide sensing results with the performance requirements in Table 6.2-1.

Table 6.2-1:	Performance requirements for 5G Wireless sensing

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 1 | 95 | 10 | 10 | N/A | N/A | 10 [3] | 5 [3] | 1000 | 1 | 5 | 2 | Indoor/outdoor (e.g., detection of human, UAV) |
|  | 2 | 95 | 2 | 5 | 1 | N/A | 1 | 1 | 1000 | 0.2 | 0.1 to 5 | 5 | Outdoor (e.g., detection of human, UAV) |
|  | 3 | 95 | 1 | 1 | 1 [3], [4] | 1 | 1 [3], [4] | 1 x 1 [3] | 100 [2], or 1000 (NOTE 3); 5000 for detection in highway | 0.05 to 1 | 2 | 2 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV) |
|  | 4 | 99 for public safety, otherwise, 95 | 0.5 | 0.5 | 1.5 for pedestrian, 15 for vehicle, otherwise, 0.1 | 1.5 for pedestrian | 0.5 | 0.5 x 0.5 for factories | 100 to 5000 | 0.1 | 1 | 3 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV, AGV, vehicle) |
| Environment monitoring | 5 | 95 | 10 | 0.2 (NOTE 4) | N/A | N/A | N/A | N/A | 60000 | 60 to 600 | 0.1 to 5 | 3 | Nature of environments monitored by sensing (e.g. rainfall, flooding monitoring) |
| Motion monitoring | 6 | 95 | N/A | N/A | N/A | N/A | N/A | N/A | 60000 | 60 | 5 | 5 | Human motions and activities obtained by sensing (NOTE 5) |
|  | 7 | 95 | 0.2 | 0.2 | 0.1 | 0.1 | 0.375 | 0.3 | 5 to 50 | 0.1 | 5 | 5 | Human hand gestures obtained by sensing (NOTE 6) |
| NOTE 1: For sensing service categories to which UAV, human or vehicle is a sensing target, the typical size (Length x Width x Height) of UAV is 1.6m x 1.5m x 0.7m, the typical size of human is 0.5m x 0.5m x 1.75m, and the typical size of vehicle is 7.5m x 2.5m x 3.5 m. NOTE 2: The safe distance between pedestrian/vehicle and power transmission station/line is 0.7m/0.95m. NOTE 3: To realize 1m granularity tracking, when the velocity resolution is 1 m/s, the maximum corresponding sensing service latency is 1 s. NOTE 4: This value is derived from the water level where people feel difficulty in walking. NOTE 5: To achieve human motion monitoring, different accuracy KPI is needed to measure different human motions. E.g., respiration rate accuracy (2 times/min) is a KPI used to measure the accuracy of sleep monitoring, sit-up rate accuracy (3 times/min) is a KPI used to measure the accuracy of sports monitoring. NOTE 6: Category 7 has more stringent requirements (e.g., for KPIs such as positioning accuracy and sensing resolution) compared to other categories and typically requires more radio resources. |  |  |  |  |  |  |  |  |  |  |  |  |  |

# * * * * End of changes * * * *