3GPP TSG- SA1 Meeting # 105	S1-240220

Athens, Greece, 26 Feb - 1 March 2024	(Revision of S1-240032)

# Source:	NEC

# Title:	New Study on non-3GPP fallback to gain 3GPP access

# Document for:	Approval

# Agenda Item:	7

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Study on non-3GPP fallback to gain 3GPP access

Acronym: FS_fallBACK

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-20

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a study item.

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as a Work Item in an earlier Release if further enhancing the feature from the previous Release)}

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification:

{This section is to be typically used to identify the IETF dependencies. Delete the header "Dependency on non-3GPP (draft) specification:" if no such dependency}

# 3	Justification

Current standardization considers the use of non-3GPP access by means of non-3GPP Inter-working Functions (IWF), such the N3IWF, the TWIF and the TNGF. A key feature of recent releases, that a UE’s capability for dual access and dual connectivity through 3GPP and non-3GPP access enables, is ATSSS. However, non-3GPP access can be leveraged in further beneficial ways, e.g. to boost the performance of UEs in gaining 3GPP access in crowded environments where many UEs initiate the attachment procedure concurrently.

5GNR utilizes a contention-based access procedure where a UE sends initial request messages on the Random Access Channel (RACH) during the access procedure. Especially in crowded areas, the request of two or more UEs may collide and the base station will not be able to decode the requests. In such case an exponential backoff procedure is triggered and defers the UEs’ request for a next access attempt.

While this procedure guarantees fairness among UEs accessing the radio spectrum mainly due to probabilistic and randomized procedures, it also brings unpredictable performances in terms of access latency. This is especially true when in presence of a large number of UEs that concurrently try to register with the network, the number of mobile network base stations is low, like e.g., in crowded events hosted in stadium premises, during emergency/disaster situations or due to the expected future increase in the number of UEs. In these scenarios, the probability of RACH preamble collision and deferred access increases significantly.

SA1 should study how future systems can benefit from non-3GPP access beyond current features such as ATSSS, but rather in the view of scale and performance boost during a UE’s 3GPP access procedure. New use cases with a clearly defined problem statement should build the fundament for the new study that investigates the fallback and use of non-3GPP access for UEs to proceed with the 3GPP access procedure while the procedure cannot be completed or performed through the 3GPP AN (denoted as fallBACK).

Note: Investigated use cases and enablers for non-3GPP fallBACK should assume the availability of sufficient resources at the relevant AN nodes and CN nodes, otherwise the UE should be notified accordingly through the fallBACK procedure.

# 4	Objective

This study aims to identify further use cases and service requirements for the use of non-3GPP access to accelerate a UE’s procedure in gaining 3GPP access to the Core network and services.

The objectives include:

- The analysis of Stage-1 operational aspects and impact to the current 5GS functionality.

- Study of policy aspects, incl.

  - MNO policies for the use of non-3GPP access for fallBACK,

  - Subscriber profile association,

  - Permission considerations,

  - Control of non-3GPP access incl. trust and fallBACK capability discovery.

- Other aspects of the study include security and privacy.

# 5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR | 22.XXX | Study on non-3GPP fallback to gain 3GPP access | TBD | TBD | NEC |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Marco Liebsch, NEC

# 7	Work item leadership

SA1

# 8	Aspects that involve other WGs

None.

# 9	Supporting Individual Members

Supporting IM name

NEC