3GPP TSG-SA1 Meeting #105	S1-240302

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024 							   (revision of S1-240274)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.125 | CR | 0053 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of TS 22.125 “Uncrewed Aerial System (UAS) support in 3GPP” with progress in other 3GPP WGs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | EAV, ID_UAS |  |  |  |  |  | Date: |  |  | 2024-02-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As part of the ongoing maintenance of 3GPP stage 1 specifications, this document proposes alignment between the Release 17 UAS requirements in TS 22.125 with what has been achieved in downstream groups. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Requirements which have not been met in other 3GPP WGs in Release 17 have been removed. Requirements which have been partially met in other 3GPP WGs have had notes added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between 3GPP stage 1 and stage 2/3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.2.2, 5.4, 6.3, 6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** FIRST CHANGE ***

## 5.1	General

[R-5.1-001] The 3GPP system should enable UTM to associate the UAV and UAV controller, and the UTM to identify them as a UAS.

[R-5.1-002] The 3GPP system shall be able to provide UTM with the identity/identities of a UAS.

[R-5.1-003] The 3GPP system shall enable a UAS to send UTM the UAV data which can contain: unique identity (this may be a 3GPP identity), UE capability of the UAV, make & model, serial number, take-off weight, position, owner identity, owner address, owner contact details, owner certification, take-off location, mission type, route data, operating status.

[R-5.1-004] The 3GPP system shall enable a UAS to send UTM the UAV controller data which can contain: unique identity (this may be a 3GPP identity), UE capability of the UAV controller, position, owner identity, owner address, owner contact details, owner certification, UAV operator identity, UAV operator license, UAV operator certification, UAV pilot identity, UAV pilot license, UAV pilot certification and flight plan.

[R-5.1-005] The 3GPP system shall enable a UAS to send different UAS data to UTM based on the different authentication and authorizations level which are applied to the UAS.

NOTE 0: 	Subject to the regional regulation, the different authentication and authorization levels can be: the initial network access authentication and authorization, UAS identity authentication, UAV flight plan authorization, additional UTM service authentications, such as flight monitoring, collision avoidance services, so on.

[R-5.1-006] The 3GPP system shall support capability to extend UAS data being sent to UTM with the evolution of UTM and its support applications in future.

[R-5.1-007] Based on regulations and security protection, the 3GPP system shall enable a UAS to send UTM the identifiers which can be: IMEI, MSISDN, or IMSI, or IP address.

[R-5.1-008] Void

[R-5.1-009] The 3GPP system should enable an MNO to augment the data sent to a UTM with the following: network-based positioning information of UAV and UAV controller.

NOTE 1: 	This augmentation may be trust-based (i.e. the MNO informs the UTM that the UAV position information is trusted) or it may be additional location information based on network information, such as OTDOA, cell coordinates, synchronization source, etc.

NOTE 2: 	This requirement will not be applied to the case which the UAS and UTM has direct control communication connection without going through MNO, such as OTDOA, cell coordinates, synchronization source, etc.

NOTE 2a:	This requirement is partially fulfilled by mechanisms in this release.

[R-5.1-010] The 3GPP system shall enable UTM to inform an MNO of the outcome of an authorisation to operate.

[R-5.1-011] The 3GPP system shall enable an MNO to allow a UAS authorisation request only if appropriate subscription information is present.

[R-5.1-012] The 3GPP system shall enable a UAS to update a UTM with the live location information of a UAV and its UAV controller.

[R-5.1-013] The 3GPP network should be able to provide supplement location information of UAV and its controller to a UTM.

NOTE 3: 	This supplement may be trust-based (i.e. the MNO informs the UTM that the UAV position information is trusted) or it may be additional location information based on network information.

[R-5.1-014] The 3GPP network shall support UAVs and the corresponding UAV controller are connecting to different PLMNs at the same time.

[R-5.1-014a] The 3GPP system shall support UAVs and the corresponding UAV controller are connecting to different PLMNs at the same time.

[R-5.1-015] The 3GPP system shall provide the capability for network to obtain the UAS information regarding its support of 3GPP communication capabilities designed for UAS operation.

[R-5.1-016] The 3GPP system shall support the UAS identification and subscription data which can differentiate the UAS with UAS-capable UE and the UAS with non-UAS-capable UE.

NOTE 4: 	UAS-capable UE refers to the UE which support interaction capability with UTM and certain 3GPP communication features which 3GPP provides for UAS.

[R-5.1-017] The 3GPP system shall support the UTM in detection of UAV operating without authorization.

NOTE 5: the scenarios covered by the requirement above are FFS.

NOTE 5a:	This requirement is only partially covered by 3GPP. The scenarios which have not been covered are not in scope of 3GPP.

## *** NEXT CHANGE ***

### 5.2.2	Decentralized UAS traffic management

[R-5.2.2-001] The 3GPP system shall enable a UAV to broadcast the following data for identifying UAV(s) in a short-range area for collision avoidance: e.g. UAV identities if needed based on different regulation requirements, UAV type, current location and time, flight route information, current speed, operating status.

[R-5.2.2-002] The 3GPP system shall be able to support a UAV to transmit a message via network connection for identifying itself as an UAV to the other UAV(s).

[R-5.2.2-003] The 3GPP system shall enable UAV to preserve the privacy of the owner of the UAV, UAV pilot, and the UAV operator in its broadcast of identity information.

[R-5.2.2-004] The 3GPP system shall enable a UAV to receive local broadcast communication transport service from other UAV in short range.

[R-5.2.2-005] A UAV shall be able to use a direct UAV to UAV local broadcast communication transport service in the coverage or out of coverage of a 3GPP network.

[R-5.2.2-006] A UAV shall be able to use a direct UAV to UAV local broadcast communication transport service when the sending and receiving UAVs are served by the same or different PLMNs.

NOTE 5b:	Per TS 23.526, this requirement is only met in the case where the sending and receiving UAVs are using the same sidelink carrier.

[R-5.2.2-007] The 3GPP system shall support a direct UAV to UAV local broadcast communication transport service at relative speeds of up to 320kmph.

[R-5.2.2-008] The 3GPP system shall support a direct UAV to UAV local broadcast communication transport service with variable message payloads of 50-1500 bytes, not including security-related message component(s).

[R-5.2.2-009] The 3GPP system shall support a direct UAV to UAV local broadcast communication transport service which supports a range of up to 600m.

[R-5.2.2-010] The 3GPP system shall support a direct UAV to UAV local broadcast communication transport service which can transmit messages at a frequency of at least 10 messages per second.

[R-5.2.2-011] The 3GPP system shall support a direct UAV to UAV local broadcast communication transport service which can transmit messages with an end-to-end latency of at most 100ms.

## *** NEXT CHANGE ***

## 5.4	Security

[R-5.4-001] The 3GPP system shall protect the transport of data between the UAS and UTM.

[R-5.4-002] The 3GPP system shall protect against spoofing attacks of the UAS identities.

[R-5.4-003] Void.

[R-5.4-004] Void.

[R-5.4-005] The 3GPP system shall support confidentiality protection of identities related to the UAS and personally identifiable information.

NOTE: Generic 5G security between UE and UPF provides confidentiality protection.

[R-5.4-006] The 3GPP system shall support regulatory requirements (e.g. Lawful Intercept) for UAS traffic.

## *** NEXT CHANGE ***

## 6.3	Service restriction for UEs onboard of UAV

[R-6.3-001] The 3GPP network shall be able to support network-based 3D space positioning (e.g., with altitude 30~300m) of a UE onboard UAV.

[R-6.3-002] Void.

## *** NEXT CHANGE ***

## 6.4	Void

## *** END OF CHANGES ***