3GPP TSG-SA WG1 Meeting #106 	    S1-241260

# Jeju, South Korea, 27 - 31 May 2024                                       (revision of S1-241106)

# Source:	ZTE, CEPRI, China Unicom, China Telecom, CMCC, vivo, AsianInfo

# Title:	New SID on Study on Enhanced Group Communication Service

# Document for:	Agreement

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on Enhanced Group Communication Service

Acronym: FS_EGCS

Unique identifier:

Potential target Release:	Rel-20

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No |  |  | X |  |  |
| Don't know | X |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as a Work Item in an earlier Release if further enhancing the feature from the previous Release)}

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification:

3	Justification

With the diversity application growing, multiple sensors, cameras etc., can be grouped to support specific application e.g., inspection application. The application needs one kind of group communication which supports information exchange either among multiple UEs in the group or with network. It includes multiple communication connections with different QoS demands and these communication connections each other have relationship according to the application demand.

3GPP has specified group communication and management related requirements in different specifications(e.g. TS 22.468, TR 22.874, TR 22.876, TS 22.261 ), which mainly focus on forming a collection of devices by the application layer to operate, e.g., a collection of smart home equipment or wearable equipment. However, considering the variable network conditions and status inside/outside the group during the operation, following challenge need be considered:

A.In general, there may exist multiple PDU sessions among one or more UEs in a same group, which requires the multiple QoS monitoring for the multiple PDU sessions among one or multiple UEs for a same service.

- There may exist multiple data flows among multiple UEs in a same group, considering that these UEs are grouped for a same service, there may exist specific and different QoS policies for multiple data flows among multiple UEs.

Considering that the 3GPP network naturally has the information of the group communication and group member status, following aspects need to be studied to enhance the group communication for this kind of service:

(1) The service level QoS is split by application server. The split QoS requirements and the relationship among them are shared to the network to guide the network to provide communication service in a whole for the application without the application server awaring detail communication process.

Figure 1. Example of inspection task in smart factory.

As example shown in Figure 1, the inspection task and its corresponding communication requirements are different according to its complexity, inspected product line and manufacturing environments (e.g., lighting, noise, and temperature) of workshops in a factory, thus the single device is not sufficient to guarantee efficient operations and provides real-time warning. Thus, the application server firstly splits the inspection task into three sub-tasks, where the sub-task1 is to inspect the corner area, the sub-task 2 is to inspect the high-altitude area, while the sub-task 3 is to inspect the area with large coverage. In this way, according to the different capabilities of devices (e.g., the robots can move to inspect large-scale area, the UAV can fly to inspect the high-altitude area, the camera is suitable for monitoring corner area), the application server provides following QoS information to the network:

--three communication QoS policies for the respect three sub-tasks and related UEs;

--three candidate UE lists for the respect three sub-tasks;

--information of data transmission order among the different data flows of a same service;

--the assistance information, e.g., the area information, the time information for the inspection task, etc.

Based on the provided information listed above,  the network provides the service for the application as a whole without application awareness communication detail, e.g. to combination and synchronization of different service flows among different UEs of a same service, to monitor the service quality.

Based on the analysis above, it is proposed to study the enhanced group communication service, as per detailed objectives listed in the next section.

4	Objective

This study is aiming at identifying use cases and potential new requirements related to enhanced group communication support for multiple UEs with respective different data flows for the same service.

- The Objective includes:

- Support multiple groups and different QoS policy(ies) on a per-group basis  for data flows from UE to server to UE, within the same service to facilitate communication in a group, for group communication.

- Support QoS performance monitoring, where UEs in a given group uses the same service.

- Gap analysis between the identified requirements and what has been already defined by existing 3GPP requirements.

5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 22.XXX | Study on Enhanced Group Communication Service | TSG#105 (Sep, 2024) | TSG#106 (Dec, 2024) | Yansheng Liu, ZTE , liu.yansheng@zte.com.cn |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

6	Work item Rapporteur(s)

Yansheng Liu, ZTE , liu.yansheng@zte.com.cn

7	Work item leadership

SA1

8	Aspects that involve other WGs

None identified yet

9	Supporting Individual Members

Supporting IM name

ZTE

CEPRI

China Unicom

China Telecom

CMCC

vivo

AsianInfo