3GPP TSG-SA WG1 Meeting #106 	S1-241279

Jeju, Korea, May 27 - 31, 2024	revision of S1-241131

Title:	Use case on remote sensing in satellite

Agenda Item:	7.3

Source:	China Mobile

Contact:	Xiaonan Shi(shixiaonan@chinamobile.com)

Tangqing Liu(liutangqing@chinamobile.com)

Abstract: This document proposes a use case on remote sensing in satellite

* * * First Change * * * *

## 5.x	Use case on remote sensing in satellite

### 5.x.1	Description

The control center of the Meteorological Bureau monitors Earth's atmosphere, clouds, oceans and other aspects of weather through the data and images returned by remote sensing satellites. Ministry of Natural Resources monitors earthquake, fire disaster, typhoon, hurricane, sandstorm, cold wave and other natural hazard through remote sensing satellite.

Remote sensing satellites take a set of high-resolution meteorological images at regular intervals, which clearly show the distribution, thickness and shape of cloud cover, topography. These images are rapidly transmitted back to the ground, where experts from the Meteorological Bureau analyze them to accurately predict the trend of weather systems. Experts of Ministry of Natural Resources could analyze these pictures to predict and monitor natural hazard accurately before it happens. There are a group of remote sensing satellite at LEO. Usually, they transmit these pictures back to ground. Application server on the ground will proceed data processing and analysis based on requirements of experts. Since these images are high-resolution and are generated at about 1.5G/s, which need very high bandwidth to transmit back to ground, it is more efficiency to process and analysis on the satellite and send the results to ground. However, as the load on remote sensing satellite is limited as it equipped with a lot of high-resolution cameras, it’s not very practical for remote sensing satellite to equip large amount of computing resource. Communication satellites are distributed in space on LEO, MEO, and GEO, they are more convenient to facilitate computing resource. And with the bandwidth at about 15-25Gb/s on ISL, remote sensing satellites can send the data to communication satellites and 5G system is able to provide Service Hosting Environment on satellite for the data processing.

### 5.x.2	Pre-conditions

Operator A has multiple LEO satellites and also GEO satellites, servers of Ministry of Natural Resources are deployed in the edge application server on two LEO satellites of operator, and servers of Meteorological Bureau/Ministry are deployed in the edge application server on three GEO satellites. There are a group of LEO remote sensing satellites to monitor same area.

Weather analysis service requires large number of data for analysis such as high-resolution pictures which take very large bandwidth of satellite, it’s difficult to transmit all the data at one time for LEO remote sensing satellites. Natural hazard service requires high reliability.

### 5.x.3	Service Flows

Figure 5.x.3-1 office on the ground monitor weather and natural hazard through remote sensing satellite

- 1.Remote sensing satellites take a set of pictures of earth’s atmosphere, clouds and topography with high definition.

- 2.As the pictures are high-resolution, and remote sensing satellites are LEO, the data cannot be transmitted to ground at one time. So remote sensing satellites send the data to the edge application server which is deployed on operator’s LEO to do on-board processing, so that only the processing result will be send to the ground which takes lower bandwidth.

- 3a.5G system select the most closely EAS on the operator LEO satellites. High-resolution pictures of all these remote sensing satellites will be send to the EAS of Meteorological Bureau on the operator LEO via inter-satellite link.

- 3b.5G system select most closely EAS on the GEO. High-resolution pictures of all these remote sensing satellites will be send to the EAS of Ministry of Natural Resources on the operator GEO via inter-satellite link.

- 4.EAS sends result to Meteorological Bureau/Ministry of Natural Resources on the ground.

- 5a.Alice, an expert of Meteorological Bureau, receives analysis results and part of pictures.

- 5b.Bob, an expert of Ministry of Natural Resources, receives analysis results and part of pictures.

### 5.x.4	Post-conditions

Alice receives analysis results and publish to citizens. It will be rainy tomorrow.

Bob receives analysis results and notify to related department of fire disaster.

### 5.x.5	Existing features partly or fully covering the use case functionality

None.

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] Based on regulatory and operators’ policy, the 5G system shall support Service Hosting Environment (e.g. edge application server) on multi-orbit satellites.

[PR 5.x.6-2] Based on regulatory, operators’ policy and agreement with 3rd party, the 5G system shall support a mechanism to provide different communication services via multi-orbit satellites.

* * * End of Change * * * *