3GPP TSG-SA WG1 Meeting #106	S1-241300

Jeju , Korea, 27-31 May 2024													(revision of S1-241190)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.989 | CR | 0031 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update and gap analysis of diversion of an incoming voice communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_FRMCS_Ph6 |  |  |  |  |  | Date: |  |  | 2024-05-28 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce call diversion for Ad hoc Group communications to fulfill functional requirements of FRMCS FRS (FU-7120) |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Define conditions of automatic call diversion 2.Update of potential requirements and gap analysis for Ad hoc Group communications |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing requirements in the stage 1 specifications, to allow downstream groups to implement call diversion with Ad hoc Group communications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.1, 6.22.2, 6.22.2.1, 6.22.2.2, 6.22.2.3, 6.22.2.4, 6.22.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

## 6.22	Voice communication transfer related use cases

### 6.22.1	Introduction

In this chapter the use cases related to the  diversion of incoming or transfer of ongoing voice communication are defined. The following use cases are defined:

-	Diversion of an incoming voice communication

-	Transfer of an ongoing voice communication

-	Service interworking and service continuation with GSM-R

The use cases are applicable for user-to-user and multi-user voice communications.

### 6.22.2	Use case: Diversion of an incoming voice communication

#### 6.22.2.1	Description

The use case is based on railway operational rules, either a railway station could temporarily be closed for any operational reasons or operating during the night with reduced staff. In that case, railway staff (e.g., controller) could not be joined anymore. A mechanism of diverting calls to railway staff located in another railway stations shall be provided.

The use case is applicable for user-to-user communication (e.g., driver to controller) and multi-user communication (e.g. multi-train voice communications involving controllers).

#### 6.22.2.2	Pre-conditions

All involved FRMCS users have selected their functional roles

An FRMCS User has already configured the new FRMCS User, in case of diverting call, out of a list (e.g. telephone book) or entered manually based on either FRMCS User Identity or FRMCS Functional Identity.

FRMCS User is registered to a single or multiple FRMCS Equipment.

FRMCS User is or is not part of an ongoing communication.

#### 6.22.2.3	Service flows

The FRMCS System provides and activates a mechanism to automatically divert communications.

FRMCS User is authorised to configure the mechanism to divertcommunications.

Assuming a train driver initiates a multi-train voice communication, during the night time, involving other drivers but also controller A who is responsible for the movement of the train. Due to the reduced staff operating mode during the night, activities of Controller A are handled by controller B, who is normally operating in another area. Controller A has preconfigured diversion of all the calls addressed to him to controller B, out of a list (e.g. telephone book) or entered manually based on FRMCS Functional Identity.

During the regular service time, in case controller A is not reachable (e.g., due to lack of coverage), has also preconfigured diversion of any incoming call to another controller.

Based on arbitration rules, controller A may also have preconfigured diversion of calls addressed to him, to another controller.

Due to above described functional behavior, the FRMCS system shall enable automatic diversion of communication to another FRMCS user of the same FRMCS domain, even before presentation of the communication to the destinated FRMCS user, if one of the following conditions of diverting communications is fulfilled:

- All incoming calls are diverted

- If the user is already connected to another communication

- If the user is not reachable.

The FRMCS system shall also enable the diversion of a presented incoming call in case a controller:

- Doesn’t reply during a configurable time or number of rings

- Or rejects an incoming call.

#### 6.22.2.4	Post-conditions

The incoming voice communication is diverted to the new FRMCS User by the FRMCS System.

For example, in case of multi-train voice communication involving controller A, responsible for the movement of the train, controller B will finally be addressed.

6.22.2.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.22.2-001] | The FRMCS System shall enable authorised FRMCS User to configure divertion of incoming voice communications to another FRMCS user of the same FRMCS domain, based on different criteria | A/T | 22.280 22.179 | Limited to private calls because group communication already provides other meachnisms [R-5.6.3-015], [R-6.7.4-016] a) Call Diversion is missing for Ad hoc Group calls, based on criteria. b) Authorisation of Call Diversion for Ad hoc Group calls to another FRMCS User of the same FRMCS domain based on FRMCS user identity, functional identity, or subparts of functional identities is missing. Editor’s note 1: Resolution of functional alias activated by multiple users will be further elaborated. Editor’s note 2: CR is needed |
| [R-6.22.2-002] | The FRMCS System shall support the divertionof incoming voice communications based on one of the following criteria: - All incoming communications are diverted; - When FRMCS user is not reachable (e.g. out of coverage); - Busy (i.e., the FRMCS user is connected in another call); - When the FRMCS user is not answering the incoming call within the set (time) interval; - When the FRMCS user rejects the presented communication | A/T | 22.179 22.280 | Partly covered (Busy excluded). Limited to private calls because group communication already provides other meachnisms [R-5.6.3-015], [R-6.7.4-016]. Busy concept no more applicable to MCX: an MCX UE/client can be simultaneously engaged in multiple private calls, or even mixed in private and group calls (see 22.179 5.5 & 22.280 5.4 & 6.14). a) All the diversion conditions mentionned, are missing for Ad hoc Group calls based on criteria. Automatic and manual commencement modes have to be considered. b) Busy for Ad hoc Group call can be handled at application level, so out of scope of 3GPP. (e.g., based on arbitration rules) b) Diversion of presented and rejected call by the user is missing for Private calls and Ad hoc Group calls. Editor’s note: CR is needed |
| [R-6.22.2-003] | The FRMCS System shall be able to support configurable time or number of rings when the FRMCS User is not answering | A/T | 22.179 22.280 | Partly covered (Busy excluded). Limited to private calls because group communication already provides other meachnisms [R-5.6.3-015], [R-6.7.4-016]. Busy concept no more applicable to MCX: an MCX UE/client can be simultaneously engaged in multiple private calls, or even mixed in private and group calls (see 22.179 5.5 & 22.280 5.4 & 6.14). Call diversion of Ad hoc Group call based on configurable time or number of rings for not answering, is missing. Editor’s note: CR is needed |
| [R-6.22.2-004] | The FRMCS System shall support the diversionof incoming voice communications for user-to-user and multi-user. | A/T | 22.280 22.179 | Indicated by sub-clause 5.6.3 Private Call (with Floor control) commencement requirements 6.7.4 Private Call (without Floor control) commencement requirements. Call diversion is missing for Ad hoc Group call. Same comment as for [R-6.22.2-001]. |
| [R-6.22.2-005] | The FRMCS System shall be aware of the busy condition of the targeted FRMCS user to forward the incoming voice communication to another FRMCS User. | A/T | 22.179 | Partly covered (Busy excluded). Limited to private calls because group communication already provides other meachnisms [R-5.6.3-015], [R-6.7.4-016]. Busy concept no more applicable to MCX: an MCX UE/client can be simultaneously engaged in multiple private calls, or even mixed in private and group calls (see 22.179 5.5 & 22.280 5.4 & 6.14). Busy for Ad hoc Group call can be handled at application level, as per comment of [R-6.22.2-002]. |
| [R-6.22.2-006] | The FRMCS system shall provide information to the initial destinator of the call that the incoming call has been successfully diverted sending back the new FRMCS User’s identity or functional identity. | A/T | 22.280 22.179 | Notification to the initial destinator about the diversion call status and new FRMCS User’s Identity or functional identity is missing in Ad hoc Group calls. |

End of changes