3GPP TSG-SA WG1 Meeting #106	S1-241363

Jeju, Korea, 27-31 May 2024								(revision of S1-241300, revision of S1-241190)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.989 | CR | 0031 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New use case: Call forwarding for Ad hoc Group Calls |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_FRMCS_Ph6 |  |  |  |  |  | Date: |  |  | 2024-05-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce call forwarding for Ad hoc Group communications to fulfil functional requirements of FRMCS FRS (FU-7120) |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Define conditions of automatic call forwarding 2. Update of potential requirements and gap analysis for Ad hoc Group communications |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing requirements in the stage 1 specifications, to allow downstream groups to implement call forwarding with Ad hoc Group communications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.1, 6.22.5(new), 6.22.5.1(new), 6.22.5.2(new), 6.22.5.3(new), 6.22.5.4(new), 6.22.5.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

## 6.22	Voice communication transfer related use cases

### 6.22.1	Introduction

In this chapter the use cases related to the transfer, of incoming or ongoing voice communication, as well as the forwarding of incoming voice communication, using Ad hoc Group Calls are described. The following use cases are defined:

-	Transfer of an incoming voice communication

-	Transfer of an ongoing voice communication

-	Service interworking and service continuation with GSM-R

-	Call forwarding for Ad hoc Group Call

The use cases are applicable for user-to-user and multi-user voice communications.

NOTE 1:	The assumption for all use cases is that Ad hoc Group Calls will be used for all operational railway communications.

Next change

### 6.22.5	Use case: Call forwarding for Ad hoc Group Call

#### 6.22.5.1	Description

The use case is based on railway operational rules, either a railway station could temporarily be closed for any operational reasons or operating during the night with reduced staff. In that case, railway staff (e.g., controller) could not be joined anymore. A mechanism of forwarding calls to railway staff located in another railway stations shall be provided.

The use case is applicable for user-to-user communication (e.g., driver to controller) and multi-user communication (e.g. multi-train voice communications involving controllers).

#### 6.22.5.2	Pre-conditions

All involved FRMCS users have selected their functional roles (Controller A, Controller B, Driver 1, Driver 2, Driver 3) to enable driver-to-controller communication or multi-train communications with Drivers 1,2,3 and controller A or B, responsible for the movement of the train.

Due to the reduced staff operating mode during the night, activities of controller A should be handled by controller B

The FRMCS System provides a mechanism to automatically forward communications.

Controller A is authorised to enable call forwarding.

Controller A has already configured Controller B, as destination of his calls, in case of call forwarding activation, out of a list (e.g. telephone book) or entered manually based on either FRMCS User Identity or FRMCS Functional Identity. Controller B is normally operating in another area.

Controller A is not part of an incoming communication.

#### 6.22.5.3	Service flows

- Example of night service of controller A

1) During the night operation Controller A activates call forwarding to controller B, as destination of all communications addressed to him.

2) Train driver 1 initiates a multi-train voice communication, using Ad hoc Group Call to train driver 2 and train driver 3, during the night time, also involving controller A who is responsible for the movement of the train. .

3) The multi-train voice communication is established between the involved drivers and controller B.

4) Controller A receives a notification that the call has been forwarded to controller B, with the FRMCS User Identity or Functional Identity of controller B.

- Example of controller A not reachable

1) Controller A has configured controller B, as destination of communications addressed to him, in case of not reachable (e.g., due to lack of coverage).

2) Train Driver 1 is calling controller A, in a user-to-user voice communication, using Ad hoc Group calls based on criteria.

3) An Ad hoc Group call is established between driver 1 and controller B

4) Controller A receives a notification that the call has been forwarded to controller B, with the FRMCS User Identity or Functional Identity of controller B.

#### 6.22.5.4	Post-conditions

#### The post conditions for each scenario presented in the service flow are:

- Example of night service of controller A

The multi-train voice communication will be established, using Ad hoc group call based on criteria, between driver 1, driver 2, driver 3 and controller B, who receives the forwarded calls of controller A.

- Example of controller A not reachable

The user -to-user voice communication using Ad hoc group call based on criteria, will be established between driver 1 and controller B, who receives the forwarded calls of controller A.

6.22.5.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.22.5-001] | The FRMCS System shall enable authorised FRMCS User to configure forwarding of incoming voice communications to another FRMCS user of the same FRMCS domain, based on different criteria | A/T | 22.280 22.179 | a) Call Forwarding is missing for Ad hoc Group calls, based on criteria. (CR is needed) b) Authorisation of Call Forwarding for Ad hoc Group calls to another FRMCS User of the same FRMCS domain based on FRMCS user identity, functional identity, or subparts of functional identities is missing. (CR is needed) NOTE: Resolution of functional alias activated by multiple users will be further elaborated. |
| [R-6.22.5-002] | The FRMCS System shall support the forwarding of incoming voice communications based on one of the following criteria: - All incoming communications are forwarded; - When FRMCS user is not reachable (e.g. out of coverage); - Busy (i.e., the FRMCS user is connected in another call); - When the FRMCS user is not answering the incoming call within the set (time) interval; - When the FRMCS user rejects the presented communication | A/T | 22.179 22.280 | a) All the forwarding conditions mentioned, are missing for Ad hoc Group calls based on criteria. Automatic and manual commencement modes have to be considered. (CR is needed) b) Busy for Ad hoc Group call can be handled at application level, so out of scope of 3GPP. (e.g., based on arbitration rules) c) Forwarding of presented and rejected call by the user is missing for Ad hoc Group calls with manual commencement mode. (CR is needed) |
| [R-6.22.5-003] | The FRMCS System shall be able to support configurable time or number of rings when the FRMCS User is not answering | A/T | 22.179 22.280 | Call Forwarding of Ad hoc Group call with manual commencement mode based on configurable time or number of rings for not answering, is missing. (CR is needed) |
| [R-6.22.5-004] | The FRMCS System shall support forwarding of incoming voice communications for user-to-user and multi-user. | A/T | 22.280 22.179 | Call forwarding is missing for Ad hoc Group call. Same comment as for [R-6.22.5-001]. |
| [R-6.22.5-005] | The FRMCS System shall be aware of the busy condition of the targeted FRMCS user to forward the incoming voice communication to another FRMCS User. | A/T | 22.179 | Busy for Ad hoc Group call can be handled at application level, as per comment of [R-6.22.5-002]. |
| [R-6.22.5-006] | The FRMCS system shall provide information to the initial destination of the call that the incoming call has been successfully forwarded sending back the new FRMCS User’s identity or functional identity. | A/T | 22.280 22.179 | Notification to the initial destination about the call status forwarding and new FRMCS User’s Identity or functional identity is missing in Ad hoc Group calls. (CR is needed) |

End of changes