3GPP TSG-SA WG1 Meeting #103 	S1-241366

Jeju, Korea, 20 - 24 May 2024	(Revision of S1-241269)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0791 | rev | 2 | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Monitoring of signalling traffic in 5G |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone, T-Mobile USA, KDDI, Telecom Italia, BT, KPN, Deutsche Telekom, Telefónica, ISSDU, III, U.S. National Security Agency, SK Telecom, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: |  |  |  |  |  |  | Date: |  |  | 2024-05-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current 5G system prevents continuation of the network surveillance troubleshooting capabilities carried out by MNOs. This is due to the introduction of encryption of the signalling exchanged between network functions and the different implementations of VNFs/CNFs from vendors in the 5G network. For network operators, standard monitoring capabilities are thus essential to continue performing health check and troubleshooting of networks without additional integration costs and without the need to standardise, in 3GPP, the internals of VNFs/CNFs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New section 6.51 that adds the requirements on 5G network to enable the relevant functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Monitoring network functions interactions in a multivendor environment will not enable cost effective deployment and operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause (6.51) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *****************   First Change ***********************

## 6.51	Monitoring of network elements interactions in 5G

### 6.51.1	Overview

External monitoring systems are often used by MNOs to track network activity for network surveillance and troubleshooting to perform diagnosis and fault analysis of their system. Such monitoring system is fully under the control of the MNOs, and the monitoring can be performed at signalling level. Due to the introduction of encryption of the signalling exchanged between network functions, there is no standardized, secure interface to share signalling traffic between the 5G network and the monitoring system.

A number of capabilities are required for the 5G network to continue supporting this feature, with regards to performance to minimise the impact on the real-time traffic and to consider the security needed to protect the copies sent towards the external monitoring system.

### 6.51.2	Requirements

NOTE 1: The monitoring system is outside of the 5G network. Both the monitoring system and the monitored network elements in the requirements below are fully under the control of the MNO.

The monitored network elements in the 5G network shall support the transmission of a secured copy of the outgoing and incoming signalling traffic to a monitoring system.

The 5G network shall enable the MNO to configure network monitoring, e.g., switching on/off per network element, selecting what type of elements and what type of signalling from these elements is the target for monitoring.

The 5G network shall allow the monitoring (i.e., transmit secured copies of outgoing and incoming signalling traffic) of a transmitting network element and, separately, the monitoring of the receiving network element while facilitating correlation of the information received from both network elements by the external system.

NOTE 2: These requirements do not imply/assume any design of the network elements. How the copies are created within the element, e.g., physical, virtual or container based, is expected to be implementation specific.

The signalling traffic shall be securely transmitted from the monitored network elements of the 5G network to the monitoring system while minimizing the degradation of network performance.

NOTE 3: The monitoring system is not integrated with the key management scheme of the 5G core.

The transmission of signalling traffic from the monitored network elements of the 5G network to the monitoring system shall be compliant with privacy legislation, data protection regulations and protection of confidential system internal data.

The transmission  of signalling traffic from the monitored network elements of the 5G network to the monitoring system shall be limited regarding the number of file formats (e.g., JSON, PCAP, etc.) to assist with the ingestion of traffic feeds.

## *****************   End of Changes ***********************